3GPP TSG-SA5 Meeting #150 	S5-236267

# Xiamen, China,  09-13 October 2023

# Source:	Ericsson Hungary

# Title:	Rel-18 pCR 28.111 FM updates

# Document for:	Approval

Agenda Item:	6.5.1.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposal below

# 2	References

[1]		3GPP TS 28.111: Fault management

# 3	Rationale

The Fault supervision documentation was problematic as it is distributed to a number of specifications and needs general improvement.

Changes proposed in this document:

- Change notificationId to isInvariant=False.

- Add references to 32.158 and VES

- Remove requirements 14 and 15 as there is no implementation for them and their meaning is not clear.

- Shrink the textual description of the OpenApi solution set. Only information that is not documented in stage to or in the YAML files is kept. Duplicating information in YAML is bad practice, risks inconsistency and just increases the amount of text to be read.

- Move normative documentation of code to Forge. The proposed changes to YAML are visible in Forge at https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/753. Note that the proposed changes will be mapped onto any additional updates SA5 agrees on before the real TS in submitted. Updated files include:

  - yang-models/_3gpp-common-fm.yang – change TS reference

  - OpenAPI/TS28623_ComDefs.yaml – change reference to FaultMnS.yaml

  - OpenAPI/TS28623_GenericNrm.yaml – change reference to FaultMnS.yaml;

TODO: should AlarmList-Single be moved to TS28111_FaultMnS.yaml ?

  - OpenAPI/TS28532_FileData‎ReportingMnS.yaml ‎– change reference to FaultMnS.yaml.

Editor’s note: FileData‎ReportingMnS should later remove the subscribe/unsubscribe operation and then FaultMns shall also remove the Subscription YAML item

  - OpenAPI/TS28532_FaultMnS.yaml – remove file (rename to OpenAPI/TS28111_FaultMnS.yaml)

  - OpenAPI/TS28111_FaultMnS.yaml – new file based on previous TS28532_FaultMnS.yaml, remove all operations and transform all notifications into operations

TODO: Additional connected CRs for TS 28.623 and 28.532 will be needed for a,b,c,d,e

- Added Annex where the plantuml source code for figures will be stored (later).

# 4	Detailed proposal

The changes below are proposed for TS 28.111[1].

Begin of modifications

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Management and orchestration; Management services".

[3]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	3GPP TS 28.516: "Fault Management (FM) for mobile networks that include virtualized network functions; Procedure".

[5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[7]	3GPP TS 32.111-2: " Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[8]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[9]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://github.com/onap/vnfrqts-requirements/blob/05f26fac2b941513a7d0e856b99fd8c61d688299/docs/Chapter8/ves7_1spec.rst#resource-structure.

[10]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

Next modification

# 5		Requirements for fault management service

The following requirements are valid for any MonitoredEntity.

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-FM-1 | The fault management service shall have the capability to provide alarm notifications to authorized consumers. | - Report Alarm - Notify New Alarm - Notify Changed Alarm - Notify Alarm List Rebuilt Motivation: the consumer should receive information about alarms immediately when an alarm is raised or changed. |
| REQ-FM-2 | The fault management service shall have the capability to allow authorized consumers to subscribe to alarm notifications. | - Subscription to Alarm Notifications Motivation: Needed for REQ-FM-1. Providers will not send notification without an explicit subscription. |
| REQ-FM-3 | The fault management service shall have the capability to allow authorized consumers to unsubscribe from alarm notifications. | - Unsubscription of Alarm notifications Motivation: The consumer needs to be able to indicate that it is no longer interested in receiving immediate alarm information |
| REQ-FM-4 | The fault management service shall have the capability to allow authorized consumers to provide a filter for alarm notifications. | - Subscription to Alarm Notifications Motivation: The consumer shall be able to indicate that it is interested only in a subset of alarms. |
| REQ-FM-5 | The fault management service shall have the capability to allow authorized consumers to retrieve the alarm list. | - Get Alarm List Motivation: The consumer shall be able to read all current alarms. It needs this if the sequence of received alarm notifications does not provide a reliable and complete view of the alarm situation. This may happen after the start-up of the consumer fault management service, if the connection or some alarm notifications are lost, or if the alarm producer was not able to provide on-time indication of all alarm changes. |
| REQ-FM-6 | The fault management service shall have the capability to allow authorized consumers to retrieve a filtered subset of the alarm list. | - Get Alarm List Motivation: If the consumer is interested only in a subset of alarms, it shall be retrieve only that subset. |
| REQ-FM-7 | The fault management service shall have the capability to provide changed alarm notifications to its authorized consumer. | - Notify Changed Alarm Motivation: the consumer should receive information about changed alarms immediately. |
| REQ-FM-8 | The fault management service shall have the capability to provide cleared alarm notifications to its authorized consumer. | - Notify Changed Alarm Motivation: the consumer should receive information about cleared alarms immediately. |
| REQ-FM-9 | The fault management service shall have the capability to provide new generated alarm notifications to its authorized consumer. | - Notify New Alarm Motivation: the consumer should receive information about alarms immediately when an alarm is raised. |
| REQ-FM-10 | The fault management service shall have the capability to indicate that the alarm list is potentially faulty and also that the alarm list rebuilt notifications to authorized consumers whenever the alarm list is rebuilt. | - Notify alarm list potentially faulty - Notify alarm list rebuilt Motivation: the consumer should receive information when the alarm list is corrupt or out-of-date. The consumer should also be notified when the correct alarm information is available again. |
| REQ-FM-11 | The fault management shall have the capability to satisfy the request to acknowledge one or multiple alarms. If this capability is not supported, then the producer shall be able to automatically acknowledge alarms. | - Acknowledge Alarms Motivation: the consumer should be able to register in the producer that it has received the alarm and has done some vendor specific level of processing of the alarm information. |
| REQ-FM-12 | The fault management shall have the capability to satisfy the request to clear one or multiple alarms. This capability is only required if one or more of the alarms supported by the producer is of type ADMC. | - Clear Alarms |
| REQ-FM-13 | The fault management shall have the capability to provide acknowledgement state change notifications to its authorized consumer. | - Notify Changed Alarms Motivation: If the producer supports ADMC alarms, the consumer shall be able to clear those. |

Next modification

#### 8.3.1.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| alarmId | M | T | F | T | F |
| objectInstance | M | T | F | T | F |
| notificationId | M | T | F | F | F |
| alarmRaisedTime | M | T | F | F | F (note 5) |
| alarmChangedTime | O | T | F | F | F (note 6) |
| alarmClearedTime | M | T | F | F | F (note 7) |
| alarmType | M | T | F | T | F |
| probableCause | M | T | F | T | F |
| specificProblem | O | T | F | T | F |
| perceivedSeverity | M | T | T (note 4) | F | F(note 6) |
| backedUpStatus | O | T | F | F | F |
| backUpObject | O | T | F | F | F |
| trendIndication | O | T | F | F | F |
| thresholdInfo | O | T | F | F | F |
| stateChangeDefinition | O | T | F | F | F |
| monitoredAttributes | O | T | F | F | F |
| proposedRepairActions | O | T | F | F | F |
| additionalText | O | T | F | F | F |
| additionalInformation | O (see note 3) | T | F | F | F |
| rootCauseIndicator | O | T | F | F | F |
| ackTime | M | T | F | F | F |
| ackUserId | M | T | T (see note 8) | F | F |
| ackSystemId | O | T | T (see note 8) | F | F |
| ackState | M | T | T (see note 8) | F | F |
| clearUserId | O (see note 1) | T | T | F | F |
| clearSystemId | O (see note 1) | T | T | F | F |
| serviceUser | O (see note 2) | T | F | F | F |
| serviceProvider | O (see note 2) | T | F | F | F |
| securityAlarmDetector | O (see note 2) | T | F | F | F |
| comments | O | T | T | F | F |
| correlatedNotifications | O | T | F | F | F |
| NOTE 1: These attributes and qualifiers are applicable only if producer supports consumer to set perceivedSeverity to CLEARED. NOTE 2: These attributes are supported if the producer emits notifyNewAlarm that carries security alarm information. NOTE 3: This attribute is supported to carry vendor specific information. NOTE 4: This isWritable property is True only if producer supports consumer to set perceivedSeverity to CLEARED NOTE 5: Emit notifyNewAlarm. NOTE 6: Emit notifyChangedAlarm NOTE 7: Emit notifyClearedAlarm NOTE 8: This isWritable property is True only if producer supports the consumer to acknowledge alarms. |  |  |  |  |  |

Next modification

# 10	Stage 3 – Solution Sets

## 10.1	Forge location for solution sets

Forge definitions are specified in Forge at https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA102/

Editor’s note: update to the Forge link valid when converted to a full TS.

## 10.2	YANG Definitions

YANG definitions are specified in Forge.

Directory: yang-models

Files:

_3gpp-common-fm.yang

## 10.3	OpenAPI Definitions

OpenAPI definitions are specified in Forge.

Directory: OpenAPI

Files:

TS28111_FaultMnS.yaml

### 10.4	RESTful HTTP-based solution set

#### 10.4.1	General

Mapping of Classes and attributes follows the general rules in TS 32.160. See clause 10.3.

### 10.4.2	Mapping of notifications

Principles:

- Only information not documented in the OpenAPI files is included in this clause.

- The following items are documented in the OpenAPI files: HTTP-Method, parameter name and type.

- The name of the parameter is the same in the stage 2 information model (clause 8 and 9) and in the stage 3 OpenAPI definition. Exceptions, if any, are listed below.

- The URI of the notification target on the MnS consumer is defined by [10] clause 4.4 URI structure and the notificationRecipientAddress attribute in the NtfSubscriptionControl IOC (See 4.3.22.2 in TS 28.622 [5].

Example: http://myoss.com/FM-MnS-1/18.8.9/oss-root/

### 10.5	RESTful HTTP-based solution set for integration with ONAP VES API

#### 10.5.1	General

Mapping of Classes, attributes and notifications is identical to those described in clause  10.4 with the differences listed in the following sub-clauses of 10.5.

#### 10.5.2	Mapping of notifications

##### 10.5.2.1	General

The URI of the notification target on the MnS consumer is defined by [10] clause 4.4 URI structure and the notificationRecipientAddress in the NtfSubscriptionControl IOC (See 4.3.22.2 in TS 28.622 [5]. The resource URI is extended with  /eventListener.

##### 10.5.2.2	Resources

Figure 10.2.2.2.2-1 shows the resource structure of the fault supervision data report MnS in the context of its integration with VES Event Listener 7.1.1 [9].

Figure 10.5.2.2-1: Resource URI structure of the fault supervision data report MnS for integration with ONAP VES Event Listener 7.1.1 (Resource structure section) [9]

See also Resource structure section in [9].

### 10.5.3	Integration with ONAP VES

Detailed guidelines for integration of performance assurance MnS notifications with ONAP VES are provided in Annex B of TS 28.532[2].

Next modification

# Annex <X> (informative): PlantUML souce text of Figures

# Editor’s note: Later the plantUML surce of all figures shall be placed here.

End of modifications