3GPP TSG-SA5 Meeting #152 	S5-237767

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0519 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.291 Correction of tenant identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The subscriber identifier cannot hold anything other than a SUPI and tenant identifier have been introduced in TS 32.291 but only for some NFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moving to a clause making it clear that the tenant identifier to better describe that it may any NF that require another format than SUPI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of tenant identifier is unclear which may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.1.1, 6.1.6.2.6.1, 6.1.6.2.7.1, 7.1, 7.6, and 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.290 CR 0206 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

###### 6.1.6.2.1.1	Type ChargingDataRequest

Table 6.1.6.2.1.1-1: Definition of type ChargingDataRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriberIdentifier | Supi | OM | 0..1 | Identifier of the subscriber that uses the requested service. |  |
| tenantIdentifier | string | OC | 0..1 | Identifier of the tenant e.g., a subscriber of a network slice |  |
| nfConsumerIdentification | NFIdentification | M | 1 | This is a grouped field which contains a set of information identifying the NF consumer of the charging service. |  |
| chargingId | ChargingId | OM | 0..1 | Charging identifier for correlation between different records. Only applicable if not available in the service specific information. |  |
| invocationTimeStamp | DateTime | M | 1 | The time at which the request is send |  |
| invocationSequenceNumber | Uint32 | M | 1 | This field contains the sequence number of the charging service invocation by the NF consumer, i.e. the order of charging data requests. The sequence number in charging data request [initial] starts from 1, and increased by 1 for subsequent charging data request. It is allowed to start from 0 for backwards compatibility. |  |
| retransmissionIndicator | boolean | OC | 0..1 | This field indicates, if included, this is a retransmitted request message. |  |
| oneTimeEvent | boolean | OC | 0..1 | Indicates, if included, that this is event based charging and whether this is a one-time event. If true, this is a one-time event that there will be no update or release. |  |
| oneTimeEventType | EventType | OC | 0..1 | Indicates the type of the one time event, i.e. Immediate or Post event charging. |  |
| notifyUri | Uri | OC | 0..1 | Identifies the recipient of Notifications sent by the CHF. In case of session based charging it shall be present in create request message, and may be present in update. |  |
| supportedFeatures | SupportedFeatures | OC | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| serviceSpecificationInfo | String | OC | 0..1 | Identifies service specific document that applies to the request, e.g. the service specific document ('middle tier' TS) and 3GPP release the service specific document is based upon. |  |
| multipleUnitUsage | array(MultipleUnitUsage) | OC | 0..N | This field contains the parameters for the quota management request and/or usage reporting. |  |
| triggers | array(Trigger) | OC | 0..N | This field identifies the event(s) triggering the request. |  |

Second change

###### 6.1.6.2.6.1	Type ChargingDataRequest

This clause specifies additional attributes of the type ChargingDataRequest defined in clause 6.1.6.2.1.1 for Network Slice Management (NSM) charging described in 3GPP TS 28.202 [71].

Table 6.1.6.2.6.1-1: Network Slice Management (NSM) charging specified attribute of type ChargingDataRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mnSConsumerIdentifier | string | OM | 0..1 | MnS consumer Identifier |  |
| nSMChargingInformation | NSMChargingInformation | OM | 0..1 | This field holds the Network Slice Management (NSM) specific information. |  |

Third change

###### 6.1.6.2.7.1	Type ChargingDataRequest

This clause is additional attributes of the type ChargingDataRequest defined in clause 6.1.6.2.7.1 for NS performance and analytics charging described in 3GPP TS 28.201[201].

Table 6.1.6.2.7.1-1: NS performance and analytics Specified attribute of type ChargingDataRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nSPAChargingInformation | NSPAChargingInformation | OC | 0..1 | This field holds the network slice information, which is reported to the CHF |  |

Fourth change

## 7.1	Bindings of common CDR field, Information Element and Resource Attribute

Table 7.1-1: Bindings of common CDR field, Information Element and Resource Attribute

| Information Element |  | CDR Field |  | Resource Attribute |  |
| --- | --- | --- | --- | --- | --- |
| Session Identifier |  | Charging Session Identifier |  | /{ChargingDataRef }/ or /{OfflineChargingDataRef}/ |  |
|  |  |  |  | ChargingDataRequest |  |
| Subscriber Identifier |  | Subscriber Identifier |  | /subscriberIdentifier |  |
| Tenant Identifier |  | Tenant Identifier |  | /tenantIdentifier |  |
| Charging Id |  | Charging Id |  | /chargingId |  |
| Invocation Timestamp |  | - |  | /invocationTimeStamp |  |
| Invocation Sequence Number |  | - |  | /invocationSequenceNumber |  |
| Retransmission Indicator |  | - |  | /retransmissionIndicator |  |
| One-time Event |  | - |  | /oneTimeEvent |  |
| NF Consumer Identification |  | NF Information |  | /nfConsumerIdentification |  |
| NF Name |  | NF Name |  | /nfConsumerIdentification/nFName |  |
| NF Address |  | NF Address |  | /nfConsumerIdentification/nFIPv4Address /nfConsumerIdentification/nFIPv6Address /nfConsumerIdentification/nFFqdn |  |
| NF PLMN ID |  | NF PLMN ID |  | /nfConsumerIdentification/nFPLMNID |  |
| NF Functionality |  | NF Functionality |  | /nfConsumerIdentification/nodeFunctionality |  |
| Notify URI |  |  |  | /notifyUri |  |
| Service Specification Information |  | Service Specification Information |  | /serviceSpecificationInfo |  |
| Multiple Unit Usage |  | List of Multiple Unit Usage |  | /multipleUnitUsage |  |
| Rating Group |  | Rating Group |  | /multipleUnitUsage/ratingGroup |  |
| Requested Unit |  | - |  | /multipleUnitUsage/requestedUnit |  |
| Time |  | - |  | /multipleUnitUsage/requestedUnit/time |  |
| Total Volume |  | - |  | /multipleUnitUsage/requestedUnit/totalVolume |  |
| Uplink Volume |  | - |  | /multipleUnitUsage/requestedUnit/uplinkVolume |  |
| Downlink Volume |  | - |  | /multipleUnitUsage/requestedUnit/downlinkVolume |  |
| Service Specific Units |  | - |  | /multipleUnitUsage/requestedUnit/serviceSpecificUnits |  |
| Used Unit Container |  | Used Unit Container |  | /multipleUnitUsage/usedUnitContainer |  |
| Service Identifier |  | Service Identifier |  | /multipleUnitUsage/usedUnitContainer/serviceId |  |
| Quota management Indicator |  | Quota management Indicator Quota management Indicator Ext |  | /multipleUnitUsage/usedUnitContainer/quotaManagementIndicator |  |
| Triggers |  | Triggers |  | /multipleUnitUsage/usedUnitContainer/triggers |  |
| Trigger Timestamp |  | Trigger Timestamp |  | /multipleUnitUsage/usedUnitContainer/triggerTimestamp |  |
| Time |  | Time |  | /multipleUnitUsage/usedUnitContainer/time |  |
| Total Volume |  | Total Volume |  | /multipleUnitUsage/usedUnitContainer/totalVolume |  |
| Uplink Volume |  | Uplink Volume |  | /multipleUnitUsage/usedUnitContainer/uplinkVolume |  |
| Downlink Volume |  | Downlink Volume |  | /multipleUnitUsage/usedUnitContainer/downlinkVolume |  |
| Service Specific Unit |  | Service Specific Unit |  | /multipleUnitUsage/usedUnitContainer/serviceSpecificUnits |  |
| Event Time Stamps |  | Event Time Stamps |  | /multipleUnitUsage/usedUnitContainer/eventTimeStamps |  |
| Local Sequence Number |  | Local Sequence Number |  | /multipleUnitUsage/usedUnitContainer/localSequenceNumber |  |
| Triggers |  | Triggers |  | /triggers |  |
|  |  |  |  | ChargingDataResponse |  |
| Invocation Timestamp |  |  |  | /invocationTimeStamp |  |
| Invocation Sequence Number |  |  |  | /invocationSequenceNumber |  |
| Session Failover |  | - |  | /sessionFailover |  |
| Triggers |  | - |  | /triggers |  |
| Multiple Unit Information |  | - |  | /multipleUnitInformation |  |
| Result Code |  | - |  | /multipleUnitInformation |  |
| Rating Group |  | - |  | /multipleUnitInformation/ratingGroup |  |
| Granted Unit |  | - |  | /multipleUnitInformation/grantedUnit |  |
| Tariff Time Change |  | - |  | /multipleUnitInformation/grantedUnit/tariffTimeChange |  |
| Time |  | - |  | /multipleUnitInformation/grantedUnit/time |  |
| Total Volume |  | - |  | /multipleUnitInformation/grantedUnit/totalVolume |  |
| Uplink Volume |  | - |  | /multipleUnitInformation/grantedUnit/uplinkVolume |  |
| Downlink Volume |  | - |  | /multipleUnitInformation/grantedUnit/downlinkVolume |  |
| Service Specific Units |  | - |  | /multipleUnitInformation/grantedUnit/serviceSpecificUnits |  |
| Triggers |  | - |  | /multipleUnitInformation/triggers |  |
| Validity Time |  | - |  | /multipleUnitInformation/validityTime |  |
| Quota Holding Time |  | - |  | /multipleUnitInformation/quotaHoldingTime |  |
| Final Unit Indication |  | - |  | /multipleUnitInformation/finalUnitIndication |  |
| Time Quota Threshold |  | - |  | /multipleUnitInformation/timeQuotaThreshold |  |
| Volume Quota Threshold |  | - |  | /multipleUnitInformation/volumeQuotaThreshold |  |
| Unit Quota Threshold |  | - |  | /multipleUnitInformation/unitQuotaThreshold |  |
| Invocation Result |  | - |  | /invocationResult |  |
| Invocation Result code |  | - |  | /invocationResult/error/cause |  |
| Failed parameter |  | - |  | /invocationResult/error/invalidParams |  |
| Failure Handling |  | - |  | /invocationResult/failureHandling |  |

Fifth change

## 7.6	Bindings for NS performance and Analytics charging

Table 7.6-1: Bindings of CDR field, Information Element and Resource Attribute for NS performance and Analytics charging

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
|  |  | ChargingDataRequest |
| Multiple Unit Usage | List of Multiple Unit Usage | /multipleUnitUsage |
| Used Unit Container | Used Unit Container | /multipleUnitUsage/usedUnitContainer |
| NSPA Container Information | NSPA Container Information | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation |
| Uplink Latency | Uplink Latency | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/uplinkLatency |
| Downlink Latency | Downlink Latency | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/downlinkLatency |
| Uplink Throughput | Uplink Throughput | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/uplinkThroughput |
| Downlink Throughput | Downlink Throughput | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/downlinkThroughput |
| Maximum packet loss rate UL | Maximum packet loss rate UL | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/maximumPacketLossRateUL |
| Maximum packet loss rate DL | Maximum packet loss rate DL | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/maximumPacketLossRateDL |
| Service Experience statistics data | Service Experience statistics data | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/serviceExperienceStatisticsData |
| The number of PDU sessions | The number of PDU sessions | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/theNumberOfPDUSessions |
| The number of Registered Subscribers | The number of Registered Subscribers | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/theNumberOfRegisteredSubscribers |
| Load level | Load level | /multipleUnitUsage/usedUnitContainer/nSPAContanierInformation/loadLevel |
| NSPA Charging Information | NSPA Charging Information | /nSPAChargingInformation |
| Single NSSAI | Single NSSAI | /nSPAChargingInformation/singleNSSAI |
|  |  | ChargingDataResponse |
| - | - | - |

Sixth change

## 7.7	Bindings for NS Management charging

Table 7.7-1: Bindings of CDR field, Information Element and Resource Attribute for NS Management charging

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
|  |  | ChargingDataRequest |
| MnS Consumer Identifier | MnS Consumer Identifier | /mnSConsumerIdentifier |
| NSM Charging information | NSM Charging information | /nSMChargingInformation |
| Management operation | Management operation | /nSMChargingInformation/managementOperation |
| Identifier of NetworkSlice Instance | Identifier of NetworkSlice Instance | /nSMChargingInformation/idNetworkSliceInstance |
| List of Service profile charging information | List of Service profile charging information | /nSMChargingInformation/listOfserviceProfileChargingInformation |
| Service Profile Id | Service Profile Id | /nSMChargingInformation/serviceProfileIdentifier |
| S-NSSAIs List | S-NSSAIs List | /nSMChargingInformation/sNSSAIList |
| SST | SST | /nSMChargingInformation/sST |
| Latency | Latency | /nSMChargingInformation/latency |
| Availability | Availability | /nSMChargingInformation/availability |
| Resource Sharing Level | Resource Sharing Level | /nSMChargingInformation/resourceSharingLevel |
| Jitter | Jitter | /nSMChargingInformation/jitter |
| Reliability | Reliability | /nSMChargingInformation/reliability |
| Maximum Number of UEs | Maximum Number of UEs | /nSMChargingInformation/maxNumberofUEs |
| Coverage Area | Coverage Area | /nSMChargingInformation/coverageArea |
| UE Mobility Level | UE Mobility Level | /nSMChargingInformation/uEMobilityLevel |
| Delay Tolerance | Delay Tolerance | /nSMChargingInformation/delayToleranceIndicator |
| DL Throughput Per Slice | DL Throughput Per Slice | /nSMChargingInformation/dLThptPerSlice |
| DL Throughput Per UE | DL Throughput Per UE | /nSMChargingInformation/dLThptPerUE |
| UL Throughput Per Slice | UL Throughput Per Slice | /nSMChargingInformation/uLThptPerSlice |
| UL Throughput Per UE | UL Throughput Per UE | /nSMChargingInformation/uLThptPerUE |
| Max Number of PDU sessions | Max Number of PDU sessions | /nSMChargingInformation/maxNumberofPDUsessions |
| KPIs Monitoring list | KPIs Monitoring list | /nSMChargingInformation/kPIMonitoringList |
| Supported Access Technology | Supported Access Technology | /nSMChargingInformation/supportedAccessTechnology |
| V2X Communication Mode | V2X Communication Mode | /nSMChargingInformation/v2XCommunicationModeIndicator |
| Additional service profile charging information | Additional service profile charging information | /nSMChargingInformation/addServiceProfileInfo |
| Management operation status | Management operation status | /nSMChargingInformation/managementOperationStatus |
| Operational state | Operational state | /nSMChargingInformation/managementOperationalState |
| Administrative state | Administrative state | /nSMChargingInformation/managementAdministrativeState |
|  |  | ChargingDataResponse |
| - | - | - |

End of changes