3GPP TSG-SA5 Meeting #152	S5-237809

Chicago, US, 13 - 17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0504 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to NWDAF analytics service failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANWDAF |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Time misconfiguration, i.e., when "Time when Analytics information is needed" is lower than the "Supported Analytics Delay", may result either in late delivery analytics event or successful analytics event. However, this is not covered in the measurements currently specified in TS 28.552 for NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two new measurements added: - Number of successful analytics events during time misconfiguration - Number of late delivery analytics events during time misconfiguration Editorial changes to clarify that “Measurements related to the NWDAF service failure” is focused towards analytics and not model training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurements related to to NWDAF analytics service failure cannot be identified efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4 (new clauses added), A.112 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 5.18.4	Measurements related to the NWDAF analytics service failure

#### 5.18.4.1	Number of analytics events for time misconfiguration

a)	This measurement provides the number of analytics events in which the "Time when Analytics information is needed" is lower than the "Supported Analytics Delay". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]).

b)	CC

c)	When the "Time when Analytics is needed" is lower than the "Supported Analytics Delay", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.TimeMisconfigAnalyticsEve, and

DANS.TimeMisconfigAnalyticsEve.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.18.4.1.X	Number of successful analytics events during time misconfiguration

a)	This measurement provides the number of analytics events in which the "Time when Analytics information is needed" is lower than the "Supported Analytics Delay" but the "Time when Analytics is delivered to the consumer" is lower than the "Time when Analytics information is needed". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]).

b)	CC

c)	When the "Time when Analytics information is needed" is lower than the "Supported Analytics Delay" but the "Time when Analytics is delivered to the consumer" is lower than the "Time when Analytics information is needed", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.TimeMisconfigSuccessfulAnalyticsEve, and

DANS.TimeMisconfigSuccessfulAnalyticsEve.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.18.4.1.X	Number of late delivery analytics events during time misconfiguration

a)	This measurement provides the number of analytics events in which the "Time when Analytics information is needed" is lower than the "Supported Analytics Delay" and the "Time when Analytics information is needed" is lower than the "Time when Analytics is delivered to the consumer". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]).

b)	CC

c)	When the "Time when Analytics information is needed" is lower than the "Supported Analytics Delay" and the "Time when Analytics information is needed" is lower than the "Time when Analytics is delivered to the consumer", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.TimeMisconfigLateDeliveryAnalyticsEve, and

DANS.TimeMisconfigLateDeliveryAnalyticsEve.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.18.4.2	Number of analytics events for late delivery

a)	This measurement provides the number of analytics events in which the "Time when Analytics information is needed" is lower than the "Time when Analytics is delivered to the consumer". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]).

b)	CC

c)	When the "Time when Analytics information is needed" is lower than the "Time when Analytics is delivered to the consumer", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.LateDeliveryAnalyticsEve, and

DANS.LateDeliveryAnalyticsEve.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.18.4.3	Number of analytics events for late analytics generating

a)	This measurement provides the number of analytics events in which the "Supported Analytics Delay" is lower than the "Time when Analytics is delivered to the consumer". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]).

b)	CC

c)	When the "Supported Analytics Delay" is lower than the "Time when Analytics is delivered to the consumer", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.LateAnaGenerating, and

DANS.LateAnaGenerating.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Changes