# 3GPP TSG SA WG5 Meeting #152                 	           S5-238061

# Chicago, US, 13-17 November 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0481 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add message flow for the satellite backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As the TS 23.501 and TS 23.502, the 5GS to support satellite backhaul is specified. The message flow for satellite backhaul charging should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add message flow for the support of satellite backhaul charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message flow for the satellite backhaul charging is not covered in TS 32.255. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.2.x	Satellite backhaul charging

5.2.2.x.1	General

The support of satellite backhaul is specified in TS 23.501 [200] and TS 23.502 [201], message flows for different scenarios:

- Satellite backhaul charging.

- Edge Computing charging with satellite backhaul.

- SCC-to-SCC communications charging via satellite backhaul.

5.2.2.x.2	Message flows for different scenarios

5.2.2.x.2.1	Satellite backhaul charging

The charging message flow of PDU session establishment for supporting Satellite backhaul, is based on Figure 5.2.2.2.2-1 description with the differences identified in clause 4.3.2.2.1.1 TS 23.502 [202].

9ch-a, 	if SMF received the satellite backhaul category  in step1 in clause 4.3.2.2.1.1 TS 23.502 [202], SMF sends the Charging Data Request [Initial] including satellite backhaul category and satellite backhaul usage report to CHF for the satellite backhaul charging.

16ch-a, 	The SMF sends Charging Data Request[Update] including the satellite backhaul usage report and the corresponding current satellite backhaul category to the CHF. The update of Charging Data Request may also be triggered by satellite backhaul category changes. If the dynamic satellite backhaul is obtained by the SMF, Charging Data Request needs to include the Satellite Backhaul QoS(e.g. throughput, latency, error rate).

The charging message flow of PDU session Release for supporting Satellite backhaul, is based on Figure 5.2.2.2.4-1 description:

2ch-a, 	The SMF sends Charging Data Request [Termination] to CHF including the satellite backhaul traffic usage report and the corresponding current satellite backhaul category.

5.2.2.x.2.2	Edge Computing charging with satellite backhaul

The message flow is based on the charging principles for 5GS usage for Edge Computing specified in clause 5.1.2 of the TS 32.257 [14] with the enhancement of the satellite backhaul information for the PDU session charging information.

5.2.2.x.2.2		SCC-to-SCC communications charging via satellite backhaul

The message flow is similar as the message flows for PDU session establishment, modification and release in clause 5.2.2.2 in TS 32.255 [10] with the enhancement of the satellite backhaul information for the PDU session charging information.

End of change