# 3GPP TSG-SA5 Meeting #152	S5-238113

# 13 - 17 November 2023, Chicago, IL, USA

# Source:	Nokia, Nokia Shanghai Bell

# Title:		Input to DraftCR 28.105 Configuration of Energy Saving Functions

# Document for:	Approval

# Agenda Item:	6.4.3.1 - AIML_MGMT_WoP#1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-235988 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

The inference capability for AI/ML-based Distributed Energy Saving has been introduced in TS28.105 but without any description of its configurable features. This pCR adds the description of configurable features and requirements for AI/ML-based Distributed Energy Saving Capabilities.

# 4	Detailed proposal

Start of First change

### 6.5.4	AI/ML inference function configuration management

#### 6.5.4.1	Description

TBD

#### 6.5.4.2	Use cases

##### 6.5.4.2.1	Managing AI/ML-based distributed Network Energy Saving

An AI/ML-based Distributed Network Energy Saving (DNES) capability may use one or more ML entities to derive energy saving recommendations.

This capability needs to be managed.

To activate or deactivate cells, the DNES capability needs to compute the degree of overlap among the cells to ensure that the coverage of a deactivated cell (called a capacity booster cell) is compensated by another cell (called the reference cell). The DNES capability should enable the MnS consumer to configure the computed degree of neighborliness among any reference cell and its neighbor target cell (i.e. the candidate capacity booster cell). The degree of neighborliness may be termed as the Cell Proximity Coupling (CPC).

To optimize the selection of cells to be switched off or on, the DNES capability need to group cells according to two categories - the reference cells which are not switched off and provide primary coverage and capacity booster cells which only enhance capacity and can thus be switched off when the load is low. Given the degree of neighborliness among cells, the DNES requires and should enable the MnS consumer to configure the  cell categories, e.g. Moreover, the DNES capability should enable the MnS consumer to configure the group of cells that need to be treated together (say called the Energy Saving Cell Group), which applies if cells need to be deactivated in a particular order as the load in the group reduces

When selecting cells to deactivate, the DNES capability considers the load in a set of cells to then decide the most optimal cell(s) to be deactivated and the order in which they should be deactivated. Where the DNES does not have the capability to compute the Energy Saving Cell Group, the MnS consumer should be enabled to configure the Energy Saving Cell Group to the DNES inference capability.

Moreover, The DNES capability needs to enable the MnS consumer to configure  the cell switch-off order which is the capacity booster cell’s rank in the switch off/on process indicating the order in which the cells in a given area shall be switched off..

Given the cell switch-off order, the DNES capability routinely compares the load in the set of cells (the Cell Group load) to the low and high load thresholds to decide if activation or deactivation should be executed. Where the DNES does not have the capability to compute the cell switch-off order, the MnS consumer should be enabled to configure the cell switch-off order. Moreover, the MnS consumer should be enabled to configure the activation or deactivation load thresholds.

#### 6.5.4.3	Requirements for AI/ML inference management

Table 6.5.4.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF-01 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to request to manage the Energy saving inference capability | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- AI/ML_INF-02 | The MnS producer of AI/ML-based Distributed Mobility Optimization should enable an authorized MnS consumer to request to manage the Mobility Optimization inference capability | Managing AI/ML-enabled for distributed Mobility Optimization (clause 6.5.4.2.2) |
| REQ- AI/ML_INF-03 | The MnS producer of AI/ML-based Distributed Load balancing should enable an authorized MnS consumer to request to manage the Load balancing inference capability | Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |
| REQ- INF_DNES-01 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to configure the Cell Proximity Coupling (CPC), which is the degree of neighborliness among any candidate neighbor cells | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- INF_DNES-02 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to configure to the DNES inference capability on the group of cells that need to be treated together (called the Energy Saving Cell Group) when selecting cell to be activated or deactivated Note: Configuration is needed only if the DNES capability does not compute the Energy Saving Cell Groups. | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- INF_DNES-03 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to configure the cell categories indicating reference cells and capacity booster cells Note: reference cells are those that provide primary coverage and are not switched off while capacity booster cells are those which only enhance capacity and can thus be switched off when the load is low. | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- INF_DNES-04 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to configure the cell switch-off order which is the capacity booster cell’s rank in the switch off/on process indicating the order in which the cells in a given area shall be switched off Note: Configuration is needed only if the DNES does not compute the cell switch-off order | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- INF_DNES-05 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to configure the switch-on and switch-off load thresholds. | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |

End of Changes