3GPP TSG-SA5 Meeting #153 	S5-240074

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0507 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add satellite backhaul category for performance measurements for UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB_OAM |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | While using satellites as the backhaul, a gNB may be subject to variable backhaul delay if the backhaul connection involves varying inter-satellite links or it has different types of satellite backhaul. The change of satellite backhaul delay has impact to PCC/QoS control, hence, based on the dynamic satellite backhaul category indication, the SMF or PCF can determine to enforce QoS monitoring to measure the packet delay between UE and PSA UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add satellite backhaul category for performance measurements for UPF, including DL packet delay between PSA UPF and UE and UL packet delay between PSA UPF and UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not enforce QoS monitoring based on the dynamic satellite backhaul category indication to measure the packet delay between UE and PSA UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.4.9	One way packet delay between PSA UPF and UE

5.4.9.1	DL packet delay between PSA UPF and UE

5.4.9.1.1	Average DL packet delay between PSA UPF and UE

a)	This measurement provides the average DL packet delay between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI and per satellite backhaul category. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE: The UPF may sample the GTP packets for QoS monitoring, the specific sampling rate is up to implementation

For each received GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information included in the GTP-U header (see 23.501 [4] and 38.415 [31]):

-	T1 indicating the local time the DL GTP PDU monitoring packet was sent by the PSA UPF;

-	T2 indicating the local time that the DL GTP PDU monitoring packet was received by NG-RAN;

-	The DL Delay Result from NG-RAN to UE indicating the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface (see 38.415 [31],and the DL Delay Result is denoted by DRdl in the present document);

-	The S-NSSAI associated to the DL GTP PDU monitoring response packet and satellite backhaul category associated to the DL GTP PDU monitoring response packet.

The PSA UPF counts the number (N) of GTP PDU monitoring response packets for each S-NSSAI and each satellite backhaul category, and takes the following calculation for each S-NSSAI and each satellite backhaul category:

i=1N(T2i-T1i+DRdli)N

d)	Each measurement is a real representing the average delay in 0.1ms.

e)	GTP.DelayDlPsaUpfUeMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

GTP.DelayDlPsaUpfUeMean.SATB, where SATB identifies the satellite backhaul category.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

5.4.9.1.2	Distribution of DL packet delay between PSA UPF and UE

a)	This measurement provides the distribution of DL packet delay between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI and per satellite backhaul category. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE: The UPF may sample the GTP packets for QoS monitoring the specific sampling rate is up to implementation.

For each received DL GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information included in the GTP-U header (see 23.501 [4] and 38.415 [31]):

-	T1 indicating the local time the DL GTP PDU monitoring packet was sent by the PSA UPF;

-	T2 indicating the local time that the DL GTP PDU monitoring packet was received by NG-RAN;

-	The DL Delay Result from NG-RAN to UE indicating the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface (see 38.415 [31],and the DL Delay Result is denoted by DRdl in the present document);

-	The S-NSSAI associated to the DL GTP PDU monitoring response packet and satellite backhaul category associated to the DL GTP PDU monitoring response packet.

The PSA UPF 1) takes the following calculation for each GTP PDU monitoring response packet for each S-NSSAI and satellite backhaul category, and 2) increment the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounter per S-NSSAI and subcounter per satellite backhaul category.

T2i-T1i+ DRdli

d)	Each measurement is an integer representing the number of GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayDlPsaUpfUeDist.SNSSAI.bin, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

GTP.DelayDlPsaUpfUeDist.SATB.bin, where Bin indicates a delay range which is vendor specific, and SATB identifies the satellite backhaul category.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

5.4.9.2	UL packet delay between PSA UPF and UE

5.4.9.2.1	Average UL packet delay between PSA UPF and UE (excluding D1)

a)	This measurement provides the average UL packet delay between PSA UPF and UE, excluding the D1 UL PDCP delay occurred in the UE. This measurement is split into subcounters per S-NSSAI and per satellite backhaul category. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE:	The UPF may sample the GTP packets for QoS monitoring, the specific sampling rate is up to implementation.

For each received GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information (see TS 23.501 [4] and TS 38.415 [31]):

-	T3 received in the GTP-U header of the monitoring response packet indicating the local time that the monitoring response packet was sent by the NG-RAN;

-	T4 that the monitoring response packet was received by the PSA UPF;

-	The UL Delay Result from UE to NG-RAN indicating the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface (excluding the D1 UL PDCP delay occurred in the UE) (see TS 38.415 [31] . The UL Delay Result is denoted by DRul in the present document);

-	The S-NSSAI associated to the GTP PDU monitoring response packet and satellite backhaul category associated to the GTP PDU monitoring response packet.

The PSA UPF counts the number (N) of GTP PDU monitoring response packets for each S-NSSAI and satellite backhaul category, and takes the following calculation for each S-NSSAI and satellite backhaul category:

i=1N(T4i-T3i+DRuli)N

d)	Each measurement is a real representing the average delay in 0.1ms.

e)	GTP.DelayUlPsaUpfUeMean.SNSSAI, where SNSSAI identifies the S-NSSAI;

GTP.DelayUlPsaUpfUeMean.SATB, where SNSSAI identifies the satellite backhaul category.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

5.4.9.2.2	Distribution of UL packet delay between PSA UPF and UE (excluding D1)

a)	This measurement provides the distribution of UL packet delay between PSA UPF and UE, excluding the D1 UL PDCP delay occurred in the UE. This measurement is split into subcounters per S-NSSAI and per satellite backhaul category. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE:	The UPF may sample the GTP packets for QoS monitoring, the specific sampling rate is up to implementation.

For each received GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information (see TS 23.501 [4] and TS 38.415 [31]):

-	T3 received in the GTP-U header of the monitoring response packet indicating the local time that the monitoring response packet was sent by the NG-RAN;

-	T4 that the monitoring response packet was received by the PSA UPF;

-	The UL Delay Result from UE to NG-RAN indicating the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface (excluding the D1 UL PDCP delay occurred in the UE) (see TS 38.415 [31] . The UL Delay Result is denoted by DRul in the present document);

-	The S-NSSAI associated to the GTP PDU monitoring response packet and satellite backhaul category associated to the GTP PDU monitoring response packet.

The PSA UPF 1) takes the following calculation for each GTP PDU monitoring response packet (packet i) for each S-NSSAI and each satellite backhaul category, and 2) increment the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounter per S-NSSAI and subcounter per satellite backhaul category.

T4i-T3i+ DRuli

d)	Each measurement is an integer representing the number of GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayUlPsaUpfUeDist.SNSSAI.bin, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

GTP.DelayUlPsaUpfUeDist.SATB.bin, where Bin indicates a delay range which is vendor specific, and SATB identifies the satellite backhaul category.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

5.4.9.2.3	Average UL packet delay between PSA UPF and UE (including D1)

a)	This measurement provides the average UL packet delay between PSA UPF and UE, including the D1 UL PDCP delay occurred in the UE. This measurement is split into subcounters per S-NSSAI and per satellite backhaul category. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE:	The UPF may sample the GTP packets for QoS monitoring, the specific sampling rate is up to implementation.

For each received GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information (see TS 23.501 [4] and TS 38.415 [31]):

-	T3 received in the GTP-U header of the monitoring response packet indicating the local time that the monitoring response packet was sent by the NG-RAN;

-	T4 that the monitoring response packet was received by the PSA UPF;

-	The UL Delay Result from UE to NG-RAN indicating the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface, and the D1 UL PDCP delay occurred in the UE (see TS 38.415 [31]. The UL Delay Result is denoted by DRul in the present document;

-	The S-NSSAI associated to the GTP PDU monitoring response packet and satellite backhaul category associated to the GTP PDU monitoring response packet.

The PSA UPF counts the number (N) of GTP PDU monitoring response packets for each S-NSSAI and each satellite backhaul category, and takes the following calculation for each S-NSSAI and each satellite backhaul category:

i=1N(T4i-T3i+DRuli)N

d)	Each measurement is a real representing the average delay in 0.1ms.

e)	GTP.DelayUlPsaUpfUeIncD1Mean.SNSSAI, where SNSSAI identifies the S-NSSAI;

GTP.DelayUlPsaUpfUeIncD1Mean.SATB, where SATB identifies the satellite backhaul category

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

5.4.9.2.4	Distribution of UL packet delay between PSA UPF and UE (including D1)

a)	This measurement provides the distribution of UL packet delay between PSA UPF and UE, including the D1 UL PDCP delay occurred in the UE. This measurement is split into subcounters per S-NSSAI and per satellite backhaul category. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure.

NOTE:  The UPF may sample the GTP packets for QoS monitoring, the specific sampling rate is up to implementation.

For each received GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information (see TS 23.501 [4] and TS 38.415 [31]):

-	T3 received in the GTP-U header of the monitoring response packet indicating the local time that the monitoring response packet was sent by the NG-RAN;

-	T4 that the monitoring response packet was received by the PSA UPF;

-	The UL Delay Result from UE to NG-RAN indicating the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface, and the D1 UL PDCP delay occurred in the UE (see TS 38.415 [31]. The UL Delay Result is denoted by DRul in the present document;

-	The S-NSSAI associated to the GTP PDU monitoring response packet and satellite backhaul category associated to the GTP PDU monitoring response packet.

The PSA UPF 1) takes the following calculation for each GTP PDU monitoring response packet (packet i) for each S-NSSAI and each satellite backhaul category, and 2) increments the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounter per S-NSSAI and subcounter per satellite backhaul category.

T4i-T3i+ DRuli

d)	Each measurement is an integer representing the number of GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayUlPsaUpfUeIncD1Dist.SNSSAI.bin, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

GTP.DelayUlPsaUpfUeIncD1Dist.SATB.bin, where Bin indicates a delay range which is vendor specific, and SATB identifies the satellite backhaul category.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.