3GPP TSG-SA5 Meeting #153 	S5-240383

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0086 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add solution for MDA assisted service failure recovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of service failure recovery has been added in TS 28.104. This document adds the solution for this use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add solution for MDA assisted service failure recovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDA solution for service failure recovery is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 8.4.3.2	MDA assisted service failure recovery

##### 8.4.3.2.1	MDA type

The MDA type for service failure recovery analysis is: MDAAssistedFaultManagement.ServiceFailureRecovery.

##### 8.4.3.2.2	Enabling data

The enabling data for MDAAssistedFaultManagement.ServiceFailureRecovery MDA type are provided in table 8.4.3.2.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.3.2.2-1: Enabling data for service failure recovery analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The abnormal performance measurements or key performance indicators based on certain performance monitoring threshold. | The performance measurements as defined in TS 28.552 [4] and key performance indicators defined in TS 28.554 [5]. |
| Alarm notifications | Alarm information, e.g. the alarm notification of network functions. | Alarm information and notifications as per TS 28.532 [11] |
| Configuration data | MOIs of 5GC NFs. | TS 28.541 [15] |

##### 8.4.3.2.3	Analytics output

The specific information elements of the analytics output for service failure recovery analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.3.2.3-1.

Table 8.4.3.2.3-1: Analytics output for service failure recovery analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| affectedObjects | Indication of 5GC NFs where the service failure related issues occurred. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| failureType | Indication of type of issues that causes the failures. NOTE 1: The values can be defined as a list of example values: "Operational Violation", "Physical Violation" and "Time Domain Violation". See alarmType described in TS 28.532 [11]. | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| perceivedSeverity | This field holds the value to indicate relative level of urgency for operator attention. NOTE 2: The value can be Critical, Major, Minor, Warning, Indeterminate, Cleared, see Recommendation ITU-T X.733 [27]. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedActions | This field holds the recommended actions to service failure recovery. The recommended action may be (but not limited to): Update 5GC NF (e.g., AMF and SMF) profile. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

End of Change