# 3GPP TSG SA WG5 Meeting #153                 	           S5-240539

# Sevilla, Spain, 29 Jan-02 Feb 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0030 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend the user location for the satellite access charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 7.4.10 |  |  |  |  |  | Date: |  |  | 2024-1-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CT4 has replied to the LS that they agreed to extend the NrLocation data type to include NR NTN TAI information. The NrLocation belongs to the UserLoation. Since the NR NTN TAI information is used by the AMF to verify the selected PLMN is allowed to operate in the current UE location, the user location of charging information needs to be extended to include NR NTN TAI information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the user location for the satellite access charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of the satellite charging is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.7, 6.2.1.2, 6.2.1.3, 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.1.7	Support for satellite access charging

The AMF determines and reports the RAT type for NR satellite access, i.e. NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT) and the user location including NR NTN TAI information to confirm that the selected PLMN is allowed to operate in the current UE location during the Registration procedure.

The AMF determines and reports the RAT type for NR satellite access, i.e. NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT) and the user location including NR NTN TAI information to confirm that the selected PLMN is allowed to operate in the current UE location during the UE Triggered Service Request.

The AMF may initiate Network-initiated Deregistration if it detects that the UE's registered PLMN is not allowed to operate in the present UE location.

If N2 Context Release Request cause indicates the release is requested due to a UE using satellite access moved out of PLMN serving area, the AMF may deregister the UE, described in clause 5.2.2.2.8 Network-initiated Deregistration before continuing with the AN Release procedure in Figure 5.2.2.3.4.1.

Next change

6.2.1.2	Definition of registration charging information

Specific charging information used for 5G registration charging is provided within the Registration Charging Information.

The detailed structure of the Registration Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of Registration Charging Information

| Information Element |  | Category |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Registration message type |  | M |  | This field holds the message type received by the AMF associated to registration: registration (initial, mobility, periodic, emergency), deregistration. |  |
| User Information |  | OM |  | Group of user information. |  |
| User Identifier |  | OC |  | This field contains the identification of the user (i.e. GPSI). |  |
| User Equipment Info |  | OC |  | This field holds the identification of the terminal (i.e. PEI, MAC Address) It may be used for identifying the user and shall be present in case Subscriber Identifier is not present. |  |
| unauthenticatedFlag |  | OC |  | This field indicates the served SUPI is not authenticated. |  |
| Roamer In Out |  | OC |  | This field holds an indication if the roamer is in-bound or out-bound. This field is present only if UE is identified as a roamer. |  |
| UE 5GMM Core Network Capability |  | OC |  | This field holds the UE 5GMM Core Network Capability included in UE MM Core network capabilities, as specified in clause 5.4.4a TS 23.501 [200] |  |
| MICO Mode Indication |  | OC |  | This field indicates whether the MICO Mode is applicable or not. |  |
| SMS Supported Indication |  | OC |  | This field indicates whether SMS is supported or not. |  |
| RAT Type |  | OC |  | This field holds the Radio Access Technology (RAT) currently serving the UE. |  |
| TAI List |  | OC |  | This field holds a set of tracking areas composing the Registration Area. |  |
| User Location Info |  | OC |  | This field indicates details of where the UE is currently located (access-specific user location information). |  |
| PSCell Information |  | OC |  | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |  |
| UE Time Zone |  | OC |  | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |  |
| Mobility Restrictions |  | OC |  | This field holds the Mobility Restrictions applicable to the UE: RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction. |  |
| Requested NSSAI |  | OC |  | This field holds the requested NSSAI. |  |
| Allowed NSSAI |  | OC |  | This field holds the allowed NSSAI consisting of one or more S-NSSAIs for serving PLMN in the present Registration Area. |  |
| Rejected NSSAI |  | OC |  | This field holds the rejected NSSAI. |  |
| NSSAI mapping list |  | OC |  | This field holds the mapping of VPLMN S-NSSAIs to HPLMN S-NSSAIs. This field is applicable only in VPLMN for roaming scenarios. |  |
| AMF UE NGAP ID |  | OM |  | This fields holds the UE association over the N2 interface within the AMF. |  |
| RAN UE NGAP ID |  | OM |  | This fields holds the RAN UE NGAP ID over N2 interface. |  |
| RAN Node Id |  | OM |  | This fields holds the Global RAN Node ID. |  |
| SNPN ID |  | OC |  | This field holds PLMN ID and the NID which identifies the SNPN. The PLMN ID is the same as PLMN ID of the SUPI. |  |
| CAG ID List |  | OC |  | This field holds the Closed Access Group Identifier List. |  |
| 5G Satellite Access Indicator |  | OC |  | This field holds the use of 5G Satellite Access. |  |

Next change

6.2.1.3	Definition of N2 Connection charging information

Specific charging information used for 5G N2 Connection charging is provided within the N2 connection charging Information.

The detailed structure of the N2 connection Charging Information can be found in table 6.2.1.3.1.

Table 6.2.1.3.1: Structure of N2 Connection Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| N2 Connection message type | M | This field holds the N2 message type received by the AMF. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| User Location Info | OC | This field indicates details of where the UE is currently located (access-specific user location information). |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT). |
| AMF UE NGAP ID | OM | This fields holds the UE association over the N2 interface within the AMF. |
| RAN UE NGAP ID | OM | This fields holds the RAN UE NGAP ID over N2 interface. |
| RAN Node Id | OM | This fields holds the Global RAN Node ID. |
| Mobility Restrictions | OC | This field holds the Mobility Restrictions applicable to the UE: RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction. |
| Allowed NSSAI | OC | This field holds the allowed NSSAI consisting of one or more S-NSSAIs for serving PLMN in the present Registration Area. |
| NSSAI mapping list | OC | This field holds the mapping of VPLMN S-NSSAIs to HPLMN S-NSSAIs. This field is applicable only in VPLMN for roaming scenarios. |
| RRC Establishment Cause | OC | This fields holds the reason for RRC Connection Establishment as received from the UE. |
| 5G Satellite Access Indicator | OC | This field holds the use of 5G Satellite Access. |

Next change

6.2.1.4	Definition of Location Reporting charging information

Specific charging information used for 5G Location Reporting charging is provided within the Location reporting charging Information.

The detailed structure of the Location reporting charging Information can be found in table 6.2.1.4.1.

Table 6.2.1.4.1: Structure of Location Reporting Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Location reporting message type | M | This field holds the location reporting message type received by the AMF. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| User Location Info | OM | This field indicates details of where the UE is currently located under the access type. |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| Presence Reporting Area Information | OC | This field contains the Presence Reporting Area Information of UE as defined in TS 23.501 [200], comprising the Presence Reporting Area identifier(s) and an indication on whether the UE is inside or outside the Presence Reporting Area, if available. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT). |
| 5G Satellite Access Indicator | OC | This field holds the use of 5G Satellite Access. |

End of change