3GPP TSG-SA5 Meeting #153 	S5-240608

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0171 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Add KPI on end to end reliability for URLLC network slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Asiainfo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | URLLC_Mgt |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Reliability is a typical performance of URLLC network slice. The higher the reliability of a URLLC network slice is, the better its performance is. Reliability can be used to evaluate the performance of URLLC network slice and can optionally be used in defining EE KPI for URLLC network slice. However, existing Reliability KPIs (TS 28.554, clause 6.8) are unidirectional or single interface related. There is no end to end reliability defined for URLLC slice network. Hence, it’s necessary to add a new end to end reliability KPI for URLLC network slice. One possible use case is for comprehensive assement of energy efficiency for URLLC network slice while energy consumption is non-directional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new end to end reliability KPI for URLLC network slice. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of end to end Reliability KPI and the EE for URLLC network slice based on reliability can’t be defined accurately. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

##### 6.8.1.x	 E2E Reliability KPI for a network slice

a) ReliabilityE2ENs

b) A KPI that describes the end to end reliability for a network slice of type URLLC taking UL and DL into consideration. It can be calculated as the multiplication of reliability on Uu interface and the reliability of N3 interface. Both the reliabilities of each interfaces are percentage of the successful transmissed packets of either interface and the total packets of either interface. It is a measure of end to end reliability between UPF and UE through Uu interface and N3 interface. It is a percentage value.

c) Below is the fomular of end to end reliability KPI between UPF and UE.

Where ReliabilityE2ENs is the end to end reliability taking UL and DL, Uu and N3 interface into consideration. It’s the reliability of Uu multiplied by the reliability of N3. NumSucUu is the number of successful transmission packets on Uu interface; NumSucN3 is the number of successful transmission packets on N3; NumTotUu is the number of total packets transmitted on Uu interface; NumTotN3 is the number of total packets transmitted on N3 interface.

The four factors used in the above formualr can be obtained as:

N(T1,drbid).SNSSAI and Dloss(T1,drbid).SNSSAI are as defined in TS 38.314 clause 4.2.1.5.1. NumSuc.SNSSAI(T).SNSSAI is the number of successfully received UL PDCP sequence numbers, representing packets that are successfully delivered to higher layers, of a data radio bearer, as defined in  TS 28.552 clause 5.1.2.2.1.  NumTot.SNSSAI(T).SNSSAI is the total number of UL PDCP sequence numbers of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB until the sequence number of the last packet.  as defined in TS 28.552 clause 5.1.2.2.1. GTP.OutDataPktN3UPF.SNSSAI is as defined in TS 28.552 clause 5.4.1.2. GTP.InDataPktPacketLossN3gNB.SNSSAI is as defined in TS 28.552 clause 5.1.1.34. GTP.InDataPktN3UPF.SNSSAI is as defined in TS 28.552 clause 5.4.1.1. GTP.InDataPktPacketLossN3UPF.SNSSAI is as defined in TS 28.552 clause 5.4.1.7.

d) NetworkSlice

2nd Change

A.18	Use case for end-to-end reliability measurements of 5G network-related KPI

The end-to-end reliability is an important performance parameter for operating 5G network. In some scenarios (e.g. uRLLC), if end-to-end reliability is insufficient, the 5G network customer cannot obtain guaranteed network performance provided by the network operator. So it is necessary to assess end-to-end reliability of network utilizing the packet delivery success rate measurements defined in clauses 6.8.2,6.8.3,6.8.4 and 6.8.5.The same can be used to determine the end to end reliability of a slice.

The end to end reliability of a network slice is the measure of the reliable performance of a network slice, especially for URLLC. One typical use case is the calculation of energy efficiency of URLLC which can be used as the performance of the network slice because of the energy consumption is non-directional.

End of Change