3GPP TSG-SA5 Meeting #153 	S5-240225

# Sevilla, ES, 29 Jan 2024 – 02 Feb 2024

# Source:	Amdocs, Nokia, Nokia Shanghai Bell

# Title:	Update Solution 4.6

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.840

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# Rationale

This pCR is to update the solution #4.6 for the study topic of CHF selection by Charging Domain.

# 4	Detailed proposal

First change

#### 5.4.4.6	Solution #4.6:  Custom HTTP2 Header For Charging

A possible solution for key issue #4c is to enhance the Nchf_ConvergedCharging SBI to allow routing of Nchf_ConvergedCharging requests and responses based on HTTP2 headers that include charging domain(s). This means that the charging data request can be routed without decoding the message payload.

An HTTP custom header could be used for this purpose.

Charging Specification TS 32.291 clause 6.1.2.3 allows the use of optional HTTP custom headers specified in clause 5.2.3.3 of TS 29.500.

This solution proposal is to add an optional HTTP custom header in clause 5.2.3.3 of TS 29.500, so that both direct and indirect communication methods can be used to route a domain-specific Charging Data Request to the correct NF Producer (CHF) instance, without decoding the message payload.

Table 5.4.4.6-1: Custom HTTP2 Header for Charging

| Name | Reference | Description |
| --- | --- | --- |
| 3gpp-Sbi-Charging-Domain | Clause 5.2.3.3.x | This header is used to specify the charging domain or subsystem for a charging data request. This header shall be included when the charging service is requested for a specific charging domain or a subsystem e.g. edge charging, IMS charging etc. |

Note: The definition of the new optional custom header will need to be added in TS 29.500. The list of domains to be allowed as part of custom header shall be determined as per solution for Key Issue #4a

End of changes