3GPP TSG-SA5 Meeting #153 	S5-240226

# Sevilla, ES, 29 Jan 2024 – 02 Feb 2024

# Source:	Amdocs, Nokia, Nokia Shanghai Bell

# Title:	Use Supported Features

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.840

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# Rationale

This pCR is to propose a solution for the study topic for CHF selection by Charging Domain.

# 4	Detailed proposal

First change

### 5.4.4	Possible Solutions

#### 5.4.4.x	Solution #4.x: Use Supported Features

One possible solution for Key Issue #4b, #4c and #4d is to use the list of Supported Features and the feature negotiation mechanism described in TS 32.291 and TS 29.500.

TS 32.291 clause 6.1.8 defines optional features that may be supported by NF Producer (CHF). Some of the features indicate domain-level support e.g. MBMS Charging etc, while some are fine-grained feature support e.g. ATSSS, QoS Monitoring etc.

This solution proposal is to extend the list of features to include additional charging domains, as indicated by solution for Key Issue #4a.

The NF Consumer and NF Producer (CHF) will use the feature negotiation mechanism described in TS 29.500 clause 6.6.2 to request and indicate support for a specific Charging domain.

End of changes