3GPP TSG-SA5 Meeting #153	S5-240412

# Sevilla, Spain, 29 Jan- 2 Feb 2024

# Source:	Huawei

# Title:	pCR 28.830 Add conclusion of issue1 missing definitions

# Document for:	Approval

Agenda Item:	6.7.3.1

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	TR 28.830 "Fault supervision evolution"

3	Rationale

This tdoc addresses the conclusion of issue1 missing defintions.

4	Detailed proposal

5	Issues and potential solutions

5.1	Issue 1: Missing definitions

5.1.1	Description

TS 28.532 [5], clause 11.2, which defines the Fault Supervision MnS, does not provide the necessary definitions and descriptions required to understand the current state of art as to alarm management. This is because much of the material specified and available for the IRP Framework was not moved to SBMA.

5.1.2	Potential solutions

It is proposed to add the following definitions to an appropriate TS:

Event: Anything that occurs at a certain point in time, for example a configuration change, a threshold crossing, a transition to an error state or a transition to a failure state. Events do not have states.

Error: A state of the system different from the correct system state as defined by the service specification. An error may or may not lead to a service failure. An error has a begin and end time.

Failure: A state of inability to deliver the correct service as defined by the service specification. A service failure may be the result of an error or a poor service function design.

Fault: The (hypothesized or adjudged) cause for an error or a failure.

Alarm: An error or failure that requires attention or reaction by an operator or some machine. Alarms have state.

Root cause: The primary fault (cause), if any, leading to one or multiple errors or failures.

1st Change

5.1.3 Conclusion - Impact on normative work

The proposal in clause 5.1.2 has been considered in TS 28.111 v1.0.0, therefore no further normative work is needed.

End of change