# 3GPP TSG-SA5 Meeting #153	S5-240421

# Sevilla, Spain, 29th Jan 2024 - 2nd Feb 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Rel-18 pCR TS 28.318 Cell Availability performance measurement definition

# Document for:	Approval

Agenda Item:	6.6.4.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

# 3	Rationale

This pCR expands on the explanation of how the cell availability performance measurement can be measured.

# 4	Detailed proposal

The following changes are proposed to be added to clauses 6.5.1.4 and 6.5.1.5 in TS 28.318.

First Change

#### 6.5.1.4	Cell AvailabilityNRCellDU

a) This measurement provides the amount of time that the NRCellDU part of the cell is available to provide services to UEs within a given granularity period T(mon) (as defined in 28.622 [13]).  It is computed  as the arithmetic sum of the amount of time that the NRCellDU is unlocked and enabled to serve UEs in each granularity period Tmon.

b) DER (n = 1)

c) This measurement is obtained as: CellAvailNRCellDU =TOSTNRCellDU=Enabled &&  T(ASTNRCellDU=Unlocked)Twin*100TOSTNRCellDU=Enabled) && (ASTNRCellDU=UnlockedTmon*100, where CellAvailNRCellDU is the availability of the NRCellDU, Tmowin is the granularity period, TOSTNRCellDU=Enabled) && (ASTNRCellDU=UnlockedT(OSTNRCellDU=Enabled) is the total amount of time that the operational state attribute(OST) TwinT(ASTNRCellDU=Unlocked)and the administrative state attribute(AST) of the NRCellDU are enabled and unlocked, respectively, in a given Tmowin. For each granularity period, the counter is reset to 0 and for every second that the value of the OST attribute is equal to “enabled” and the AST attribute value is equal to “unlocked”, the counter is increased by 1, otherwise the counter is not incremented. At the end of the granularity period, the CellAvailNRCellDU will be computed as the current value of the counter over the granularity period * 100.

d) Two values: The CellAvailNRCellDU from 0 to 100 and the corresponding Tmowin.

e) CellAvailNRCellDU

f) NRCellDU

g) Valid for packet switched traffic

h) 5GS

i) One usage of this measurement is to indicate the availability (%) of the DU of the radio cell.

Second Change

TOSTNRCellCU=Enabled && T(ASTNRCellCU=Unlocked)Twin*100TwinT(OSTNRCellCU=Enabled)TwinT(ASTNRCellCU=Unlocked)Twin

Twin

End of Changes