3GPP TSG-SA5 Meeting #153	S5-240485

Sevilla, Spain, 29th Jan 2024 – 2nd Feb 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0510 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update use case on coordination among multiple NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANWDAF |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This contribution is to convert the draft CR (S5-236958) agreed in SA5#151 meeting to real CR. Technical reason: Measurements related to the coordination between multiple NWDAFs may be applicable to both analytics and training service operations. The information related to the number of ML training subscriptions and notifications from/to the FL Server NWDAF is important for the management of FL Server NWDAF. Operators can use these measurements for configuration, resource allocation or load balance purpose of FL Server NWDAFs. In this proposal, the use case on coordination between multiple NWDAFs is updated to also cover ML model training service operation. It is necessary to add these content to TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The use case related to monitoring of coordination between multiple NWDAFs is updated to cover ML model training service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurements related to coordination between multiple NWDAFs for ML model training service operation is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.110 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# A.110	Monitoring of coordination between multiple NWDAFs

There are two cases where coordination between multiple NWDAFs are required: (i) when NWDAF is able to act as an Aggregator NWDAF (the NWDAF with analytics aggregation capability); and (ii) when NWDAF is able to act as FL Server NWDAF (the NWDAF with ML model aggregation capability).

In the case where a NWDAF analytics service consumer requests Analytics ID(s) that requires multiple NWDAFs to collectively serve the request, the Aggregator NWDAF may act as the analytics service consumer to request the analytics information from other NWDAFs. After collecting analytics information from other NWDAFs, the Aggregator NWDAF may act as the analytics service producer to provide the aggregated analytics information.

In the case where a NWDAF training service consumer requests ML model training that requires multiple NWDAFs to perform ML model training to collectively serve the request, the FL Server NWDAF may act as the ML training service consumer to request the ML model training information from other NWDAFs (acting as FL clients). After several such iterations of collecting ML model training information from other NWDAFs, the FL server NWDAF provides an aggregated trained ML model to the NWDAF training service consumer[59].

The coordination between multiple NWDAFs may directly impact the performance of service provided by the Aggregator NWDAF or the FL Server NWDAF and resource management between multiple NWDAFs. Therefore, the measurements related to the coordination between multiple NWDAFs are needed to reflect the performance of NWDAF service. These measurements are important in decision-making for the management of NWDAF.

The number of SBA interaction activities needs to be monitored. It reflects how many NFs the Aggregator NWDAF or the FL Server NWDAF coordinates with and how busy the Aggregator NWDAF or the FL Server NWDAF is on aggregation operation. The SBA interaction activity may include the subscription, request, notification and response received and/or generated by the Aggregator NWDAF or the FL Server NWDAF (see TS 23.288 [59]). In order to know the performance of Aggregator NWDAF or FL Server NWDAF when providing different types of services to the consumers, the number of SBA interaction activities related to different types of analytics need to be monitored respectively. The analytics type is identified by Analytics ID (see TS 23.288 [59]).

The measurements described above are some of the basic statistic information to monitor the performance of Aggregator NWDAF or FL Server NWDAF on coordination with other NWDAFs. Operators can use this statistic information to analyse and evaluate the performance of multiple NWDAFs, and make configuration and resource allocation among these multiple NWDAFs.

End of Changes