3GPP TSG-SA5 Meeting #153 	S5-240656

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0175 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New KPIs for the UE Wake up signal utilization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-29-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 28.554, currently there is no KPI quantifying the Wake up signal feature enablement and utilization per connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new KPI showing the average number of Wake up Signal Enablement and utilization per connection. The term connection used here refers to having the UE context maintained on the NW side, i.e if the UE context is deleted or reset then a new connection is starts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Currently, there is no direct KPI to monitor the enablement of Wake up signal feature by network or utilization of Wake up signal feature by UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.X (new), 6.4.Y (new), 6.4.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 28.552 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 6.4.X	Average number of successfully configured MCG WUS configurations per connection

a)	WUSMcgEnablement

b)	This KPI shows the average number of successfully applied RRCReconfiguration configuring this feature on MCG per RRC connection (The term connection used here refers to having the UE context maintained on the NW side, i.e if the UE context is deleted or reset then a new connection is starts). It is obtained by the number of successfully applied RRCReconfiguration messages carrying support of this feature normalized by the total successful RRC establishments.

c)	To measure the average number of MCG WUS configured on MCG per connection:

WUSMcgConfigEnablement=RRC.WUS.DEPLOYMENT=MCG ΣcauseRRC.ConnEstabSucc.Cause+causeRRC.ResumeSuccByFallback.cause+RRC.ReEstabSuccWithoutUeContext

Measurement names: RRC.WUS. DEPLOYMENT =MCG, RRC.ConnEstabSucc.Cause, RRC.ResumeSuccByFallback.cause, RRC.ReEstabSuccWithoutUeContext

d)	NRCellCU

# >>>>BEGINNING OF CHANGES<<<<

### 6.4.Y	Average number of successfully configured SCG WUS configurations per added SCG

a)	WUScgEnablement

b)	This KPI shows the average number of successfully applied RRCReconfiguration configuring this feature on SCG per total number of configured NR-DC with NR SCG connections. It is obtained by the number of successfully applied RRCReconfiguration messages carrying support of this feature normalized by the total number of successfully configured NR-DC with NR SCG connections.

c)	To measure the average number of SCG configured WUS per connection:

WUSScgConfigEnablement=RRC.WUS.DEPLOYMENT=SCG RRC.RRCRECONF.Scg.Nr+ RRC.RRCRESUME.Scg.Nr

Measurement names: RRC.WUS. DEPLOYMENT, RRC.RRCRECONF.Scg.Nr+ RRC.RRCRESUME.Scg.Nr

d)	NRCellCU

# >>>>NEXT CHANGE<<<<

### 6.4.Z	Average number of WUS utilization per connection

a)	WUSUtilization

b)	This KPI shows the average number of DCI 2.6 scheduled by network carrying wake up signal order to UEs. It is obtained by the number of DCI 2.6 scheduled by the network normalized by the total number of RRCReconfiguration successfully configuring WUS.

c)	To measure the UAI utilization

UAIUtilization.DEPLOYMENT=NRCellDUs,   NRCellDU of type DEPLOYMENT  RRC.WusSchdeuled RRC.WUS. DEPLOYMENT

Measurement names: RRC.WusSchdeuled.DEPLOYMENT, RRC.WUS. DEPLOYMENT

Note the following:

- The above KPI is broken down by DEPLOYMENT.

- The sum in the numerator is performed over the per deployment NRCellDUs for a given NRCellCU.

d)	NRCellCU

# >>>>END OF CHANGES<<<<