3GPP TSG-SA5 Meeting #153 	S5-240660

Seville, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0176 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New KPIs for the NR Dual Connectivity with NR SCG feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 28.554, currently there is no KPI quantifying the NR Dual Connectivity with NR SCG feature enablement and utilization per connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new KPI showing the average number of the NR Dual Connectivity with NR SCG feature enablement and utilization per connection. The term connection used here refers to having the UE context maintained on the NW side, i.e if the UE context is deleted or reset then a new connection is starts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Currently, there is no direct KPI to monitor the enablement of the NR Dual Connectivity with NR SCG feature by network or utilization of the NR Dual Connectivity with NR SCG feature by UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.X (new), 6.4.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 28.552 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 6.4.X	Average number of the NR Dual Connectivity with NR SCG feature enablement per connection.

a)	NrDcNrScgReconfEnablement

b)	For the NR Dual Connectivity with NR SCG feature, this KPI shows the average number of NR Dual Connectivity with NR SCG being configured per RRC connection (The term connection used here refers to having the UE context maintained on the NW side, i.e if the UE context is deleted or reset then a new connection is starts). It is obtained by the number of RRCReconfiguration or RRCResume messages carrying UE side successfully applied configuration of this feature normalized by the total successful RRC establishments, reestablishments and RRC Resumes (Reestablishments and Resumes without UE context are what we consider for these later two, because these are what define a new connection).

c)	To measure the average number of successful Reconfiguration based NR SCG Additions per RRC connection:

NrDcNrScgReconfEnablement  =RRC.RRCRECONF.Scg.Nr+ RRC.RRCRESUME.Scg.NrΣcauseRRC.ConnEstabSucc.Cause+ ΣcauseRRC.ResumeSuccByFallback.cause+ RRC.ReEstabSuccWithoutUeContext

Measurement names: RRC.RRCRECONF.Scg.Nr, RRC.RRCRESUME.Scg.Nr  RRC.ConnEstabSucc.Cause, RRC.ResumeSuccByFallback.cause, RRC.ReEstabSuccWithoutUeContext

d)	NRCellCU

# >>>>NEXT CHANGE<<<<

### 6.4.Y	Average number of the NR Dual Connectivity with NR SCG feature enablement per Successful Resume Procedure.

a)	NrDcNrScgResumeEnablement

b)	For the NR Dual Connectivity with NR SCG feature, this metric shows the percentage of RRC Resume procedures that carry NR Dual Connectivity with NR SCG configuration (The term connection used here refers to having the UE context maintained on the NW side, i.e if the UE context is deleted or reset then a new connection is starts). It is obtained by the number of RRCResume messages carrying UE side successfully applied configuration of this feature normalized by the total successful RRC establishments and RRC Resumes (Resumes without UE context are what we consider for this later, because these are what define a new connection).

c)	To measure the average number of successful Resume based NR SCG Additions per resumed RRC connection:

NrDcNrScgResumeEnablement  =100* RRC.RRCRESUME.Scg.Nr ΣcauseRRC.ResumeSucc.cause

Measurement names: RRC.RRCRESUME.Scg.Nr, RRC.ResumeSucc.cause,

d)	NRCellCU

# >>>>END OF CHANGES<<<<