3GPP TSG-SA5 Meeting #153 	S5-240793

Sevilla, Spain, 29 January - 02 February 2024

Title:	LS reply to ITU-T Study Group 2 on initiating new work item ITU-T M.fmcdns: "Framework for management of cross-domain network slices in IMT-2020 network and beyond"

Response to:	sp17-sg2-oLS-00088

Release:	3GPP Rel-18

Work Item:	TEI-18

Source:	3GPP SA5

To:	ITU-T Study Group 2

Cc:	ITU-T Study Group 13

Contact Person:

#### Name:	Kai ZHANG

E-mail Address:	kai.zhangkai@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

3GPP SA5 would like to thank ITU-T Study Group 2 for the LS on initiating new work item ITU-T M.fmcdns: "Framework for management of cross-domain network slices in IMT-2020 network and beyond".

Regarding the management framework for cross-domain network slices which is shown in the "Figure 1 Management framework for cross-domain network slices in IMT-2020 network and beyond" (see copy below) provided in the attachment (SG2-TD356R3/PLEN):

Figure 1 Management framework for cross-domain network slices in IMT-2020 network and beyond.

3GPP SA5 would like to give the following feedback:

- The fundamental building block of the Service Based Management Architecture (SBMA), which is adopted by 3GPP SA5, is the Management Service (MnS). Management Service is a set of offered capabilities for management and orchestration of network and services. The entity producing an MnS is called MnS producer. The entity consuming an MnS is called MnS consumer. Management Services provided by an MnS producer can be consumed by any entity with appropriate authorisation and authentication. A Management Function (MnF) is a logical entity playing the roles of MnS consumer and/or MnS producer. The CSMF, NSMF and NSSMF shown in Figure 1 are examples of MnFs producing/consuming management services for network slicing. Further details can be found in 3GPP TS 28.533 and TS 28.530.

- Is there any plan for the interface q illustrated in the Figure 1 (Management framework for cross-domain network slices in IMT-2020 network and beyond) to adopt the SBMA concept and/or use the Management Services defined in 3GPP SA5 for network slice management?

Reference documents are:

- 3GPP TS 28.530 (Management and orchestration; Concepts, use cases and requirements) at: https://www.3gpp.org/dynareport/28530.htm

- 3GPP TS 28.533 (Management and orchestration; Architecture framework) at: https://www.3gpp.org/dynareport/28533.htm

# 2	Actions

To ITU-T Study Group 2

Action: 3GPP SA5 respectfully requests ITU-T Study Group 2 to take the above information into consideration and provide feedback.

# 3	Dates of next TSG SA WG 5 meetings

SA5#154		15 April  - 19 April 2024		China

SA5#155		27 May - 31 May 2024		Korea