3GPP TSG-SA5 Meeting #153 	S5-240802

Sevilla, Spain, 29 January - 02 February 2024                            revision of S5-240241

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | DraftCR | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update control plane congestion analysis description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Update the UC description for more background information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the UC description The changes are based on eMDAS draft CR (S5-238359). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case description for MDA supporting Control plane congestion analysis is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x.a.2 and 7.2.x.a.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 7.2.x.a	Control plane congestion analysis

##### 7.2.x.a.1	Description

This MDA capability is for analysis of control plane congestion.

##### 7.2.x.a.2	Use case

As described in TS 23.501 [x], a 5GC NF can become overloaded when it is operating over its nominal capacity resulting in diminished performance (including impacts to handling of incoming and outgoing traffic). Some mechanism, such as control plane congestion control as described in TS 23.501 [x] is designed for the purpose of avoiding and handling of 5GC NF overload. For example, as described in clause 5.19.7 of TS 23.501 [x], when AMF is under overload conditions, it may reject the received request from the UE depending on various aspects. And the UE will send a new request after some time. It is possible that the new request will be rejected again because of the load of the AMF. In virtualized environment, the signalling request may be rejected due to inadequacy of available resources at the target 5GC NF e.g. AMF or SMF. If such situation can not be resolved, it will probably cause signalling storm for the whole network and affect the services (e.g. calls and data connections) provided by the network.

It is desirable to use MDA to assist control plane congestion analysis in order to detect, prevent or resolve identified congestion issue happened at the control plane. MDAS producer may utilize the collected PM, FM, network topology datas, virtual resource information provided from ETSI NFV MANO and etc) for control plane congestion analysis and provides analytics report containing identified or predicted congestion issue for the target 5GC NF (e.g. AMF, SMF). The analytics report also provides recommended actions to optimize the target 5GC NF for avoiding or resolving congestion issue. Based on the recommendation in the report, 3GPP management system can adjust (e.g., scale-up the virtual resource) the resources to better facilitate processing of the received control plane messages.

##### 7.2.x.a.3	Requirements

Table 7.2.X.a.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CP_ANA-01 | MDA capability for control plane congestion analysis shall include identifying the 3GPP 5GC NFs with congestion issue. | Control plane congestion analysis |
| REQ-CP_ANA-02 | MDA capability for control plane congestion analysis shall include providing the prediction of congestion issue for a 3GPP 5GC NF. | Control plane congestion analysis |
| REQ-CP_ANA-03 | MDA capability for control plane congestion analysis shall include providing recommended actions to prevent congestion issue for 3GPP 5GC NFs. | Control plane congestion analysis |
| REQ-CP_ANA-04 | MDA capability for control plane congestion analysis shall include providing recommended actions to resolve identified congestion issue for 3GPP 5GC NFs. | Control plane congestion analysis |

End of Change