# 3GPP TSG-SA5 Meeting #153	S5-240932d5

# Seville, Spain, 29th January-2nd February, 2024                                     	was S5-240646

# Source:	Samsung, EUTC, EDF, DTAG, Vodafone

# Title:	Rel-18 pCR TS 28.318 Procedure Update for coordinated rapid recovery

# Document for:	Approval

Agenda Item:	6.6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	Rationale

This pCR proposes the required changes in the procedure flow of clause 7.1.2 to align with NRM definitions.

# 3	Detailed proposal

It is proposed to agree to the following text to clause 7 in TS 28.318.

The revision d1

- moves the coordinated recovery procedure to the procedures informative annex

- renumbers clause 8 (stage 3 definitions) to clause 7

- the changes to the procedure itself are not shown since the text has been moved. To see the original procedure text with the changes, please see the original pCR S5-240646.

The revision d2

- removes clause 7 (normative procedures) from the skeleton as there are no normative procedures, as well as the procedure that was moved

The revision d3

- adjusts the procedure to align it with changes in the NRM

- simplification is possible since only one IoC is created (the updated NRM does not include UPSInfo)

- names are aligned

- remove 'may' (replace with 'can') since the procedure is now in an informative annex)

The revision d4

- add Vodafone as a supporting company.

The revision d5

- step 6 in the procedure begins "If there is a change in the attributes". This is revised to "If there is a change in the attribute values."

First Change

## A.3	Energy utility and telecommunication coordinated rapid recovery of energy service procedure

### A.3.1	General

#### Coordinated energy service recovery requirements are given in clause 5.3. The use cases supported are described in Annex C.

#### The functionality supported by the signalling flow is:

-	A: Inititialization (to initialize the rest of the defined operations). As interaction between MnSs involves creation of a MOI the listed operations precede the other operations listed below.

-	B: MNO informs DSO of changes (e.g. to update the list of essential sites and their related information).

-	C: DSO updates the information (e.g. to update the expected end of an energy outage).

-	D: DSO retrieves the changes (e.g. to query the UPS available time for a site from the MNO).

### A.3.2	Signal flow

In the procedure below A occurs before B, C and D are possible. B, C and D can occur at any time after A completes, in any order.

Figure A.3.2-1: Coordinated Rapid Recovery and strategic outage plan Procedure

1.	In order to be able to provide energy service outage and recovery related information for MNO, DSO sends createMOI request for OutageAndRecovery IOC which contains information attributes for outage and rapid recovery by DSO.

2.	MnS producer in MNO creates the OutageAndRecovery MOI that contains this information for the applicable site. Which site is applicable depends on the attribute affectedArea that is sent in the createMOI request in step 1. createMOI response is sent by MNO MnS producer to DSO.

3.	In order to be able to get automatically notified of the creation of the MOIs or any changes in the attribute information in the MOIs, DSO sends createMOI request for NtfSubscriptionControl IOC to MNO. The notification that can be subscribed for is defined using notificationTypes attributes. See clause 4.4.1 of TS 28.622.

4.	The MnS Producer in MNO sends the response.

5.	MnS producer in MNO instantiate the OutageAndRecovery IOC. This is name-contained by OutageAndRecovery IOC created in step 1.

NOTE 1: 	In step 6, if done, a subscription is created such that, subject to parameters in the NtfSubscriptionControl IOC, notifications are sent from the MNO to the DSO. This allows, for example, the MNO to notify the DSO of changes in the maxServiceDuration over time. This step is shown as step 6 below.

6.	If there is a change in the attribute values, the MNO sends a notifyMOIAttributeValueChanges notification to inform DSO about the changes. The example of the change can include change of the maxServiceDuration.

NOTE 2:	In steps 7 and 8 below, there is a change in an attribute of the MOI. This is done by DSO to inform MNO on the changes, e.g. information about an expected outage for a specific site.

7.	DSO can modify/update any information like outage start time stamp by sending a modifyMOIAttributes request to MNO. DSO can create/read/update/delete (CRUD operations) the information in the MOI by using provisioning MnS (defined in TS 28.532).

8.	MNO MnS producer provides the modify response to the DSO.

NOTE 3:	In steps 9 and 10 below, the DSO requests and receives respectively the current value of one or more attribute(s).

9.	DSO can also query the maxServiceDuration as and when required by using the getMOIAttributes operation. For example the information could include identity of DSO energy supply meter, MNO base station, remaining MaxServiceDuration of a particular site, etc. DSO requests the required information from the MNO by using getMOIAttributes operation.

10.	MNO provides the required information in the response to the DSO.

Second Change

1. createMOI (DsoRapidRecovery) Request

DSO

(MnS Consumer)

MNO

(MnS Producer)

3. createMOI Response

2. MOI will be created

6. createMOI(IntfSubscriptionCtrl) Request

4. IOC containing

UPS info instantiated

7. MOI will be created

8. CreateMOI Response

Initialization

9. notifyMOIAttributeValueChanges

10. modifyMOIAttributes Request

11. modifyMOIAttributesResponse

12. getMOIAttributeRequest

13. getMOIAttributeResponse

MNO informs DSO of the changes

DSO updates the information

DSO retrieves the changes

5. Notify MOI Creation

Third Change

# 7	Stage 3 definitions

End of Change