# 3GPP TSG-SA5 Meeting #153	S5-240933

# Seville, Spain, 29th January-2nd February, 2024

# Source:	Samsung, Nokia, Nokia Shanghai Bell, Ericsson, Vodafone, EUTC, EDF, DTAG, Ericsson France

# Title:	Rel-18 pCR TS 28.318 NRM Definitions for coordinated rapid recovery

# Document for:	Approval

Agenda Item:	6.6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	Rationale

This pCR proposes the NRM definitions required to support the procedure defined in clause 7.1 of TS 28.318 v0.3.0.

# 3	Detailed proposal

It is proposed to agree to the following text to clause 6 in TS 28.318.

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	DIRECTIVE (EU) 2019/ 944 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL - of 5 June 2019 - on common rules for the internal market for electricity and amending Directive 2012/ 27/ EU (europa.eu)
https://eur-lex.europa.eu/legal-content/EN/TXT/PDF/?uri=CELEX:32019L0944&from=EN

[3]	IEC TC 57 https://www.iec.ch/ords/f?p=103:7:511571509228708::::FSP_ORG_ID,FSP_LANG_ID:1273,25

[4]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements"

[5]	IEC "Bringing intelligence to the grid", International Electrotechnical Commission, Geneva, Switzerland, 2018.  https://www.iec.ch/basecamp/bringing-intelligence-grid <accessed: 12.7.23>

[6]	IEEE SMARTGRID, "Standards", IEEE, 2023. https://smartgrid.ieee.org/about-ieee-smart-grid/standards <accessed: 12.7.23>

[7]		Sendin, A., Stafford, J., Grilli, A., "Utilities and Telecommunications in a Nutshell", EUTC, Ediciones Experiencia, 2022.

[8]	3GPP TS 22.104, "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[9]	3GPP TS 22.261, "Service requirements for the 5G system; Stage 1"

[10]	IEC TR 61000-3-6 Electromagnetic compatibility (EMC) - Part 3-6: Limits - Assessment of emission limits for the connection of distorting installations to MV, HV and EHV power systems.

[11]	IETF RFC 1628, "UPS Management Information Base", May 1994.

[12]	ETSI ES 202 336-3: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks); Part 3: AC UPS power system control and monitoring information model".

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

Next Change

# 6	Informational model definitions

## 6.2       Class diagrams

### 6.2.1      Relationships

This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document

Figure 6.2.1-1: DSO Rapid Recovery NRM fragment

### 6.2.2	Inheritance

This clause depicts the inheritance relationships.

<<InformationObjectClass>>

Top

<<InformationObjectClass>>

OutageAndRecoveryInfo

Figure 6.2.2-1: DSO Rapid Recovery NRM fragment

## 6.3       Class definitions

### 6.3.1	OutageAndRecoveryInfo

#### 6.3.1.1 Definition

This IOC represents the mutual information exchanged between DSO and MNO, for a particular instance of an energy outage at a particular location, to achieve Rapid Recovery of DSO’s energy service outage for a site/location which requires service restoration.

outageStartTime is used by the DSO to inform the MNO about the time when the outage of the energy service has occurred.

expectedOutageEndTime is used by the DSO to inform the MNO about the time when the service is expected to be restored through manual dispatch and processes, i.e. by not using rapid recovery mechanisms. In case the end of the outage cannot be estimated, the field is not filled.

dsoRapidInterventionTime is used by the DSO to inform the MNO about the start time from when the DSO expects that they will need telecommunication service to control devices in order to perform rapid intervention.

Note: Telecommunication service enable rapid intervention to start.

dsoRapidInterventionDuration is used by the DSO to inform the MNO about the duration for which the DSO requires telecommunication services from the MNO, beginning with dsoRapidInterventionTime.

mnoInterventionTime is used by the MNO to inform the DSO about the time when MNO will actually be able to provide telecommunication service to enable rapid intervention .

mnoInterventionDuration is used by the MNO to inform the DSO about the duration for which a MNO will actually be able to provide telecommunication service to enable rapid intervention, starting with mnoInterventionTime.

mnoMaxServiceDuration is used by the MNO to inform the DSO about the current estimated remaining time that the MNO is able to provide telecommunication service in the given “affectedArea”.

affectedArea is used by the DSO to inform the MNO about the area affected by the outage of the energy service. Different stakeholders are having different knowledge about the topology of the network and thus have different possibilities to identify objects in the network.

isAffectedAreaPriority indicates whether the location of DSO substation needs recovery on priority by using smart energy services or not.

actualOutageEndTime is used by the DSO to indicates the end of outage incident i.e the time at which DSO finally has restored its energy distribution services.

requiredrapidInterventionDuration is the time duration for which the DSO requires rapid intervention services from the MNO.

#### 6.3.1.2 Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| outageStartTime | M | T | T | F | T |
| expectedOutageEndTime | M | T | T | F | T |
| dsoRapidInterventionTime | M | T | T | F | T |
| dsoRapidInterventionDuration | M | T | T | F | T |
| mnoInterventionTime | M | T | F | F | T |
| mnoInterventionDuration | M | T | F | F | T |
| mnoMaxServiceDuration | M | T | F | F | T |
| affectedArea | M | T | T | F | T |
| isAffectedAreaPriority | M | T | T | F | T |
| actualOutageEndTime | M | T | T | F | T |

#### 6.3.1.3	Attribute constraints

None.

#### 6.3.1.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 6.3.2 EnergyServiceLocation  <<datatype>>

##### 6.3.2.1 Definition

This <<datatype>> defines the area/location of the service outage.

This is defined as CGI and/or energySupplyId. This defines the regions/locations where the DSO subsystems are situated.

The attribute CGI is of type NrCellId , which provides the affected area by either a geographical area or a list of cell ids of the different RAN technologies.

The energySupplyId identifies the MNO facility that is using the DSO energy services. The energySupplyId is known between both the parties using out-of-band mechanisms hence not subject to standardisation.

Thise energySupplyId, used extensively in this class definition is known to both the MNO and DSO and is the means by which information is organized, essentially the 'key identifier' used to exchange information about specific sites, which is essential for most Rapid Recovery actions (see Annex C.2) and Rapid Intervention actions (see Annex C.2).

##### 6.3.2.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CGI | M | T | T | F | T |
| energySupplyId | M | T | T | F | T |

##### 6.3.2.3	Attribute constraints

None.

##### 6.3.2.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable

### 6.4	Attribute definitions

#### 6.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document in addition to those existing in clause 4.4.1 of [X].

| affectedArea | It defines the area/location of the DSO service outage. These are the regions/locations where DSO subsystems are situated. It is an aggregation of: • CGI, which provides the affected area by either a geographical area or a list of cell ids. • energySupplyId, which identifies the MNO facility that is using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. | Type: EnergyServiceLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| --- | --- | --- |
| outageStartTime | It informs about the time when the outage of the DSO energy service has occurred at the location provided by the attribute affectedArea. Along with outage location and expected recovery time this would be beneficial for MNO to plan and prepare its UPS power backup at different site locations for its crucial communication services and also for assisting DSO for rapid intervention. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedOutageEndTime | It informs about the time when the service is expected to be restored through manual dispatch and processes. In case the end of the outage cannot be estimated, the field is not filled. This is usually a longer time period that may not be acceptable for some crucial public services like hospitals. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dsoRapidInterventionTime | It defines the time at which DSO restores its Energy feeder service. This is the time from when the DSO expects rapid intervention to start. From this time a DSO can use its automated smart energy services e.g SCADA to recover its distribution services much quicker in substations instead of manual processes. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRapidInterventionDuration | Time interval for which the DSO requires rapid intervention services from the MNO to restore its energy distribution services rapidly. The time interval starts with dsoRapidInterventionTime. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isAffectedAreaPriority | It indicates whether the location of DSO substation needs recovery on priority by using smart energy services or not. This can be used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MNO Rapid Intervention. The default value is "False". allowedValues: True, False | Type:Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| mnoInterventionTime | It defines the time at which a MNO will actually be able to provide dedicated communication services to DSO (as rapid intervention) depending on its UPS backup status , other critical service venues etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mnoInterventionDuration | It defines the time duration for which a MNO will actually be able to provide dedicated communication services to DSO (as rapid intervention) depending on its UPS backup status , other critical service venues etc. The interval starts with mnoInterventiontime. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| actualOutageEndTime | It defines the time at which DSO finally restores its energy distribution services rapidly by using rapid intervention. This time is much smaller than expectedOutageEndTime. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CGI | This attribute defines the set of cells,that are affected by the outage. The affected area is defined by a list of cell ids. | type: NrCellId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyId | energySupplyId identifies the MNO facility (e.g. base station) using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. A meter with a unique energySupplyId is installed by DSO in the MNO site. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoMaxServiceDuration | It defines the remaining time duration for which MNO site can operate on its UPS backup at a given time i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from a given time. It is defined in minutes. allowedValues: Integer with a minimum value of 1 [NOTE1] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable:False |
| NOTE 1: The value of mnoMaxServiceDuration can be taken from IETF RFC 1628 [11], upsEstimatedMinutesRemaining. ETSI ES 202 336-11 [12] defines a field "Estimated remaining battery autonomy (time) during discharge" as provided by the monitoring and control interface for battery system with integrated control and monitoring information model, however there is no corresponding stage 3 definition of this value. |  |  |

### 6.Y	Common notifications

Refer to clause 4.5.2 of [X] for Configuration notifications.

End of Change