3GPP TSG-SA5 Meeting #154 	S5-241601

Changsha, China, 19 April - 15 April 2024

# Source:	AsiaInfo

# Title:	Add use case for RAN coverage performance to be assured

# Document for:	Approval

Agenda Item:	6.19.4

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.867 v0.0.0 Study on Closed Control Loop Management

3	Rationale

This provides the use case for RAN coverage performance to be assured.

4	Detailed proposal

It proposes to make the following changes to TR 28.867.

First Change

5. Use cases

5. X Use Case X RAN coverage performance to be assured

5.X.1 Description

The goal of use case is to enable RAN coverage performance assurance in specified areas based the goals of closed control loop. The CCL may be implemented for RAN coverage performance to be assured when the RAN coverage performance may degrade.

The 3GPP management system collects the coverage related data and monitoring the KPIs of corresponding RAN NEs in the specified areas, for example, coverage performance data (i.e. performance measurement, MDT data), coverage configuration data (e.g. coverageShape, digitalTilt and digitalAzimuth) and environment data (e.g. electronic map)). Analysis hosted by MDAF may be utilized for processing the data and analyse the KPIs. MADF could analysis and predict the coverage problem (e.g. weak coverage, a coverage hole, a pilot pollution, an overshoot coverage, or a DL and UL channel coverage mismatch, etc.).

The 3GPP management system accesses if and which action to take (e.g. modify the coverage related parameters) based the results of analytic. The 3GPP management system may continuously monitor and analysis the coverage performance, perform the adjustment and configuring the coverage related parameters (e.g. selection of coverageShape, digitalTilt and digitalAzimuth) until RAN coverage performance to be assured.

5.X.2 Potential Requirements

REQ-CCL-CON-01 The 3GPP management system shall have the capability to collect RAN coverage performance information.

REQ-CCL-CON-02 The 3GPP management system shall have the capability to analyse the RAN coverage performance information in specified areas.

REQ-CCL-CON-03 The 3GPP management system shall have the capability to access the potential the action for the RAN coverage performance to be assured.

REQ-CCL-CON-04 The 3GPP management system shall have the capability to take actions for the RAN coverage performance to be assured.

5.X.3 Potential Solutions

TBD.