3GPP TSG-SA5 Meeting #154	S5-241685

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0551 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add MA PDU session release measurement for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Monitoring the access type over which an MA PDU session is requested to be released can help for analysis of the traffic distribution across various accesses and reflecting the effect of ATSSS. Therefore, it is necessary to add the measurement of MA PDU session release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MA PDU session release measurement： - Number of MA PDU session release over a single access requests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the access type over which the MA PDU session is requested to be released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.13.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

5.3.1.13	MA PDU session management

5.3.1.13.X	Number of MA PDU session release over a single access requests

a)	This measurement provides the number of MA PDU sessions requested to be released over a single access. This measurement is split into subcounters per S-NSSAI and subcounters per access type.

b)	CC.

c)	On receipt by the SMF from AMF of Nsmf_PDUSession_UpdateSMContext Request including "access for MA PDU Session Release" (see clause 5.2.8.2.6 of TS 23.502 [7]). Each requested MA PDU Session that may be released over a single access is added to the relevant subcounter per S-NSSAI and the relevant subcounter per access type("REL_MAPDU_OVER_3GPP" or "REL_MAPDU_OVER_N3GPP" in maReleaseInd as indicated in the received Nsmf_PDUSession_UpdateSMContext Request, see Table 6.1.6.3.14-1 of TS 29.502 [14]) to increment by 1 respectively.

d)	Each subcounter is an integer value.

e)	SM.MAPduSessionRel.SNSSAI and SM.MAPduSessionRel.Acctype.

Where the SNSSAI identifies the S-NSSAI; and the Acctype identifies the access type over which the MA PDU session is requested to be released.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS.

2nd change

# A.107	Use case of monitoring of MA PDU session management for ATSSS

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network. The multi-access PDU Connectivity Service is realized by a Multi-Access PDU (MA PDU) Session.

When establishing an MA PDU session, the AMF informs the SMF that the request is for a MA PDU Session. The number and success rate of MA PDU session creations, and the number of MA PDU sessions running on the SMF are some of the basic performance measurements for monitoring the performance of MA PDU session establishment and reflecting the effect of ATSSS. And the failed MA PDU session creations measured per cause are helpful to pinpoint and solve the network issues in case the performance is below the expectation.

When an ATSSS-capable UE requests to establish a single-access PDU Session, but no policy in the UE and no local restrictions mandate a single access, the 5GC network may decide to modify it to a Multi-Access PDU (MA PDU) Session. This decision may be taken when e.g. the SMF wants to offload some traffic of the requested PDU Session to non-3GPP access or when the SMF wants to apply MPTCP and/or MPQUIC to provide bandwidth aggregation for the requested PDU Session(see TS 23.502 [7]). Whether a requested PDU Session can be successful converted to a MA PDU Session by the network may help the analysis and update of ATSSS policy. Therefore the performance of UE requested PDU session establishment with network modification to MA PDU session procedures need to be monitored.

The MA PDU session release procedure is used to release the MA PDU Session, or release the MA PDU Session over a single access. The MA PDU Session Release over a single access may be triggered by the network due to e.g. when the UE is deregistered over an access or when the S-NSSAI of the MA PDU Session is no longer in the Allowed NSSAI over an access. Monitoring the access type over which the MA PDU session is requested to be released can help operators to optimize the traffic distribution across various accesses.