3GPP TSG-SA5 Meeting #154	S5-241136

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0320 | rev | - | Current version: | 16.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Common Notification Header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | All SA5 notification have a common header containing 6 parameters present in all notifications (objectClass, objectInstance, notificationId, notificationType, eventTime, systemDN). The common header is not defined in 5G documentation as a concept; however, it is referenced in multiple 3GPP and external documents, which as of necessity often still refer to 3GPP TS 32.302 (which is not valid for 5G). The definition of these parameters should but is not always the same in different notifications which is confusing. To avoid inconsistent definitions and allowing documents to reference a common header definition a new section to document the notification header shall be added. This chapter is seen as missing, it is an editorial error of 28.532. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clause defining the common notification header. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent definitions of the common parameters may lead to interoperability problems. References to old (pre-5G) documents lead to confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.0, 11.0.1, 11.0.2, 11.0.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 11	Management services – Stage 2

## 11.0	Generic management

### 11.0.1	Introduction

This clause provides specifications that are valid for multiple MnSs and capabilities.

### 11.0.2	Common notification header parameters

All notifications emitted by any MnS or function shall both support and contain the following parameters:

#### 11.0.2.1	Input parameters

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | String: name of an IOC | See comment on objectInstance |
| objectInstance | M | Distinguished name (DN) of an MOI | Together with objectClass it identifies an MOI the notification is related to. It can be e.g. an MOI that caused the notification to be emitted or an MOI about which the notification reports some information. |
| notificationId | M | Integer: This is an identifier for the individual notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the MOI specified by objectInstance. |
| notificationType | M | String: It specifies the type of notification and hence the information carried by the notification. | It should be a fixed, short, human readable string for each type of notification. |
| eventTime | M | DateTime: The date and time when the event that triggered the sending of the notification occurred. | The semantics of DateTime are specified by RFC 3339 [52]. |
| systemDN | M | Distinguished name: the DN of MnS producer. If an MnSAgent MOI is present, systemDN should be the DN of an MnSAgent. |  |

The parameters may be further specified by the individual notification types.

Individual notification types may define additional parameters.

## 11.1	Generic provisioning management service

### 11.1.0	Introduction

This clause provides the stage 2 definitions of Create, Read, Update and Delete (CRUD) operations for managing managed objects. According to clause 4.2.2 of TS 28.533 [13], these CRUD operations are the MnS component type A. The operations specified in this clause in combination with a NRM (MnS component type B) constitute a MnS, as defined in clause 4.3 of TS 28.533 [13] providing generic provisioning services for supported NRM (MnS component type B) of all MnS.

In addition, notifications to report changes related to managed objects and their attributes are specified.

End of changes