3GPP TSG-SA5 Meeting #154 	S5-241154

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0334 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R19 CR 28.623 missing MBS indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS38.413 has specified MBS Communication Service Type IE, which indicates whether doing QoE measurement shall be performed for MBS broadcast or multicast. This IE is used for both signaling and management based QoE. In QoE measurement Activation procedure, this MBS Communication Service Type shall be provided by consumer and forwarded to gNB. Furthermore, in qceIdMappingInfoList use case, the mapping shall be done by gNB if QoE measurement configuration pertains to MBS communication service. The MBS Communication Service Type can be used by the gNB to understand if QoE measurement configuration pertains to MBS communication service or not. Same requirement on UDM for signalling based QoE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the missing indication of MBS communication service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB is unable to know the MBS communication service type Unable to make use of the qoECollectionEntityIdentity at QoE reporting for QoE measurement, which may lead security issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | yang-models/_3gpp-common-qmcjob.yang |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TS 28.405 CR0031, TS 28.622 CR0328, TS28.623 CR0334 Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1069 at commit 8362e9892add9670fb39bbce026097f0ffd114a1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

*** yang-models/_3gpp-common-qmcjob.yang ***

<CODE BEGINS>

module _3gpp-common-qmcjob {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-qmcjob";

prefix "qmc3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-trace { prefix trace3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Quality of Experience Measurement Collection Job handling

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2024-04-04 { reference CR-0334; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-02-14 { reference "CR-0234"; }

revision 2022-11-04 { reference "CR-0215"; }

revision 2022-08-17 { reference "CR-0185"; }

revision 2022-04-29 { reference "CR-0171"; }

revision 2022-03-22 { reference "draftCR"; }

revision 2022-01-06 { reference "CR-0142"; }

grouping QMCJobGrp {

leaf serviceType {

description "Specifies an end user service type for QoE measurements.";

mandatory true;

type enumeration {

enum DASH;

enum MTSI;

enum VR;

}

}

list areaScope {

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area where data shall be collected.

List of eNB/list of gNB/eNB/gNB for RLF or RCEF.

List of cells or Tracking Area for QMC.

Cell, TA, LA, RA are mutually exclusive.";

reference "Clause 5.10.2 of 3GPP TS 32.422";

uses trace3gpp:AreaScopeGrp;

}

leaf qoECollectionEntityAddress {

type inet:ip-address;

mandatory true;

description "Specifies the address to which the QMC records shall be

transferred.  Ipv4 or Ipv6 address(es) may be used.";

}

list pLMNTarget {

max-elements 1;

description "Defines the PLMN for which sessions shall be selected in

the network request session in case of area based QMC when several

PLMNs are supported in the RAN (this means that shared cells and not

shared cells are allowed for the specified PLMN. Furthermore, several

PLMNs can be used for not shared RAN cases as well as for shared RAN

cases.). Only the sessions may be selected where the PLMN that the

UE reports as selected PLMN is the same as the PLMN Target.";

uses types3gpp:PLMNId;

key "mcc mnc";

}

leaf qoETarget {

type string;

description "Specifies the target object of the QMC in case of

signalling based QMC.  The qoETarget attribute shall be able

to carry IMSI or SUPI.";

mandatory true;

}

leaf qoEReference {

type string;

mandatory true;

description "Identifies the QoE measurement collection job in the

Managed Elements and in the measurement collection entity.  The QoE

reference shall be globally unique therefore it is composed as follows:

MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC

activation request from the management system to identify one PLMN

containing the management system, and QMC ID is a 3 byte Octet String.

The QMC ID is generated by the management system or the operator.";

}

leaf jobId {

type string;

yext3gpp:inVariant;

description "The jobId attribute presents the job identifier of a

QMCJob instance. The jobId can be used to associate  multiple QMCJob

instances. For example, it is possible to configure the same jobId

for multiple QMCJob instances required to produce the data

(e.g. Streaming services and MTSI reports) for a specific network

analysis.";

}

list sliceScope {

description "Contains a list of S-NSSAIs (Single Network Slice Selection

Assistance Information).  A Network Slice end to end is identified by

S-NSSAI.";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf qMCConfigFile {

type string;

description "Provides a reference to a file including the parameters for

configuration of application layer measurements, known as Container

for Application Layer Measurement Configuration.";

mandatory true;

}

list mDTAlignmentInformation {

key "idx";

min-elements 1;

max-elements 1;

description "This parameter indicates the MDT measurements with which

alignment of QoE measurement is required. This parameter is optional

and is valid for NR only.";

leaf idx { type uint32 ; }

uses trace3gpp:TraceReferenceGrp ;

}

leaf-list availableRANqoEMetrics {

type enumeration {

enum appLayerBufferLevelList;

enum playoutDelayForMedia;

enum Startup;

}

max-elements 2;

description "This parameter indicates available RAN visible QoE metrics

to the gNB. This parameter is optional and is valid for NR only.";

}

leaf mBSCommunicationServiceType {

type enumeration {

enum broadcast;

enum multicast;

}

description "This attribute indicates which type of MBS communication

service the QoE measurement configuration pertains to.";

}

}

grouping QMCJobSubTree {

description "Helps with augmenting QMCJob into multple places.";

list QMCJob {

description "The QoE Measurement Collection provides capability for

collecting QoE information from:

- UEs which are in the specified area in case of Management Based

Activation or

- an individual UE in case of Signalling Based Activation.

The QoE Measurement Collection enables collection of application layer

measurements from the UE for specified end user service type.

The supported service types are:

- Streaming services, see TS 26.247

- MTSI services, see TS 26.114

- VR services, see TS 26.118

A QMCJob instance represents the job for collecting QoE measurements

according to the job parameters. For details of the QoE measurement

collection configuration parameters see clause 5 of TS 28.405.

QMCJob instance can be name-contained by SubNetwork or ManagedElement.

A QMC Job is activated by creating a QMCJob object instance in the

producer. For details of Management Based Activation of QoE Measurement

Collection see clause 4.5 and for details of Signalling Based

Activation of QoE Measurement Collection see clause 4.6 of TS 28.405.

The attributes areaScope and pLMNTarget are only relevant when

Management Based Activation is used and the attribute qoETarget is

only relevant when Signalling Based Activation is used. All other

attributes are common for both Management Based Activation and

Signalling Based Activation.

When a MnS consumer wishes to deactivate a QMC Job, the MnS consumer

shall delete the corresponding QMCJob instance.

NOTE:  If the reporting is ongoing, when a request to delete a QMCJob

is received, the reporting does not end. The QMCJob instance is deleted,

when the last reporting for the QMC Job expires.

The jobId attribute presents the job identifier of a QMCJob instance.

jobId can be used to associate  multiple QMCJob instances. For example,

it is possible to configure the same jobId value for multiple QMCJob

instances required to produce the data (e.g. Streaming services and

MTSI reports) for a specific network analysis.

The QoE Measurement Collection provides capability for collecting QoE

information from:

- UEs which are in the specified area in case of Management Based

Activation or

- an individual UE in case of Signalling Based Activation.

The QoE Measurement Collection enables collection of application layer

from the UE for specified end user service type. The supported service

types are:

- Streaming services, see TS 26.247.

- MTSI services, see TS 26.114.

- VR services, see TS 26.118.

A QMCJob instance represents the job for collecting QoE measurements

according to the job parameters. For details of the QoE measurement

collection configuration parameters see clause 5 of TS 28.405. A QMCJob

instance can be name-contained by SubNetwork or ManagedElement.

A QMC Job is activated by creating a QMCJob object instance in the

MnS producer. For details of Management Based Activation of QoE

Measurement Collection see clause 4.5 and for details of Signalling

Based Activation of QoE Measurement Collection see

clause 4.6 of TS 28.405. The attributes areaScope and pLMNTarget are

only relevant when Management Based Activation is used and the

attribute qoETarget is only relevant when Signalling Based Activation

is used. All other attributes are common for both Management Based

Activation and Signalling Based Activation.

When a MnS consumer wishes to deactivate a QMC Job, the MnS consumer

shall delete the corresponding QMCJob instance.

NOTE:  If the reporting is ongoing, when a request to delete a QMCJob

instance is received, the reporting does not end. The QMCJob instance

is deleted, when the last reporting for the QMC Job expires.

The jobId attribute presents the job identifier of a QMCJob instance.

The jobId can be used to associate  multiple QMCJob instances.

example, it is possible to configure the same jobId value for multiple

QMCJob instances required to produce the data (e.g. Streaming services

and MTSI reports) for a specific network analysis.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QMCJobGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses QMCJobSubTree;

}

augment "/me3gpp:ManagedElement" {

uses QMCJobSubTree;

}

}

<CODE ENDS>

*** END OF CHANGE ***