3GPP TSG-SA5 Meeting #154	S5-241264

# Changsha, China, 15 – 19 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0451 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.422 Fixing the inconsistencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The scope does not include 5G. - The formatting of the condition for SNPN user consent handling is incorrect. - The sub clauses in Report Amount clause are not following the correct hierarchy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The scope has been updated to include 5G - The formatting has been corrected. - The hierarchy has been corrected for clause 5.10.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned clauses and typos |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3.2, 5.10.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 1	Scope

The present document describes the mechanisms used for the control and configuration of the Trace, Minimization of Drive Test (MDT), Radio Link Failure (RLF) and RRC Connection Establishment Failure (RCEF) reporting functionality at the management system (at the Element Manager (EM) and or Network Manager (NMs) in UMTS, EPS and NR), Network Elements (NEs) and User Equipment (UEs). For Trace functionality, it covers the triggering events for starting/stopping of subscriber/UE activity traced over 3GPP standardized signalling interfaces, the types of trace mechanisms, configuration of a trace, level of detail available in the trace data, the generation of Trace results in the NEs and UE and the transfer of these results to management system. For MDT, it also covers logged MDT and immediate MDT mechanisms in both management based and signalling based scenarios. GSM is excluded from the RAT systems which the present document can be applied to.

The mechanisms for Trace, MDT, RLF and RCEF reporting activation/deactivation are detailed in clause 4; clause 5 details the various Trace, MDT, RLF and RCEF reporting control and configuration parameters and the triggering events that can be set in a network. Trace, MDT, RLF and RCEF reporting concepts and requirements are covered in 3GPP TS 32.421 [2] while Trace and MDT data definition and management is covered in 3GPP TS 32.423 [3].

The conditions for supporting Network Sharing are stated in 3GPP TS 32.421 [2].

The condition for supporting user consent handling for MDT does not apply to Standalone Non-Public Network, i.e., SNPN as per TS 23.501[40].

Start of next Change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [4], TS 32.101 [1] , TS 23.501 [40], TS 38.300 [42] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [4].

AS	Application Server

BGCF	Breakout Gateway Control Function

CAG	Closed Access Group

CSCF	Call Session Control Function

I-CSCF	Interrogating-CSCF

IM CN SS	IP Multimedia Core Network Subsystem

IMEI-TAC	IMEI Type Allocation Code

NID	Network ID

P-CSCF	Proxy – CSCF

PNI-NPN	Public Network Integrated Non-Public Network

RCEF	RRC Connection Establishment Failure

RLF	Radio Link Failure

S-CSCF	Serving-CSCF

SNPN	Stand-alone Non-Public Network

TAU	Tracking Area Update

TRSR	Trace Recording Session Reference

TR	Trace Reference

Start of next Change

### 5.10.6	Report Amount

#### 5.10.6.1	General

The parameter is mandatory if the reporting trigger is configured for periodical UE side measurement (M1, M4, M5, M6, M7 in LTE, M1in NR and M1/M2 in UMTS) and the jobtype is configured for Immediate MDT or combined Immediate MDT and Trace. The parameter defines the number of measurement reports that shall be taken for periodical reporting while UE is in connected mode. Detailed definition of the parameter is in  TS 38.331 [43], TS 36.331 [32] and TS 25.331 [31].

#### 5.10.6.2	Report Amount in UMTS

The parameter is an enumerated type with the following values in UMTS:

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 infinity (7)

#### 5.10.6.3	Report Amount for M1 in LTE

The parameter is an enumerated type with the following values for M1 in LTE:

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 infinity (7)

#### 5.10.6.4	Report Amount for M4 in LTE

The parameter is an enumerated type with the following values for M4 in LTE:

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 infinity (7)

#### 5.10.6.5	Report Amount for M5 in LTE

The parameter is an enumerated type with the following values for M5 in LTE:

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 infinity (7)

#### 5.10.6.6	Report Amount for M6 in LTE

The parameter is an enumerated type with the following values for M6 in LTE:

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 infinity (7)

#### 5.10.6.7	Report Amount for M7 in LTE

The parameter is an enumerated type with the following values for M7 in LTE:

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 infinity (7)

#### 5.10.6.a	Report Amount for M1 in NR

The parameter is an enumerated type with the following values for M1 in NR. The detailed definition of this parameter is available in clause 6.3.2 of TS 38.331 [43].

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 INFINITY (7)

#### 5.10.6.b	Report Amount for M4 in NR

The parameter is an enumerated type with the following values for M4 in NR. The detailed definition of this parameter is available in clause 6.3.2 of TS 38.331 [43].

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 INFINITY (7)

#### 5.10.6.c	Report Amount for M5 in NR

The parameter is an enumerated type with the following values for M5 in NR. The detailed definition of this parameter is available in clause 6.3.2 of TS 38.331 [43].

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 INFINITY (7)

#### 5.10.6.d	Report Amount for M6 in NR

The parameter is an enumerated type with the following values for M6 in NR. The detailed definition of this parameter is available in clause 6.3.2 of TS 38.331 [43].

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 INFINITY (7)

#### 5.10.6.e	Report Amount for M7 in NR

The parameter is an enumerated type with the following values for M7 in NR. The detailed definition of this parameter is available in clause 6.3.2 of TS 38.331 [43].

-	1 (0),

-	2 (1),

-	4 (2),

-	8 (3),

-	16 (4),

-	 32 (5),

-	 64 (6),

-	 INFINITY (7)

End of Changes