3GPP TSG-SA5 Meeting #154	S5-241269

# Changsha, China, 15 – 19 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0530 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Include the sub_counter value when reporting Performance Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Configuration of performance measurements are done via PerfMetricJob as defined in TS 28.622. The name of the measurements as part of “performanceMetrics” in PerfMetricJob IOC are constructed including sub-counters if the measurement is defined per sub-counter. At present, the PerfMetricJob can be configured for a specific performance measurement and specific sub-counter. If the applicable sub-counter can have five different values, the Performance measurements are captured for all the 5 possible sub-counter values. But there is no standardized means to indicate that these performance measurements including sub-counters should be reported along with the value of the sub-counters. If the values are not indicated, it is not possible to correctly identify and segregate the different values of the same performance measurement for different values of a given sub-counter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of the “Filter Naming” clause has been enhanced with reporting of performance measurements including the sub-counter name and its value and then the measurement itself. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No means to indicate the value of the sub-counter leading the consumer to not use the received measurements correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 4.2.2	Filter naming

For the Performance measurements that indicate Filters, the resulting Performance measurement name will take the form: Performance measurement_Filter

If combination of Filters is used the name will take the form: Performance measurement_Filter1_Filter2

If no Filter is used the name will take the form: Performance measurement

Example:  DRB.PdcpF1DelayDl_Filter, Where Filter is a combination of PLMN and 5QI and SNSSAI.

-	Single Filter value applied: DRB.PdcpF1DelayDl_Filter1, where Filter could be any of the possible Filter value defined for the performance measurement:

-	Sub counter name: DRB.PdcpF1DelayDl.5QI

-	Multiple (two) Filter values applied: DRB.PdcpF1DelayDl_Filter1_Filter2, where Filter1 and Filter2 could be any of the possible Filter value defined for the performance measurement:

-	Sub counter name: DRB.PdcpF1DelayDl.PLMN.5QI.

-	No Filter value applied: Counter name: DRB.PdcpF1DelayDl

For Filters the separator '_' is used to append the filter name(s) to the measurement name.  For multiple Filters the order in the resulting name is not important.

For reporting of the Performance Measurements, the name of the performance measurements are constructed along with the value of the sub-counters in the following format.

DRB.PdcpF1DelayDl.sub-counter_name=”value”, where the “sub-counter_name” indicates the name of the sub-counter (e.g. PLMN, 5QI, etc.) and the “value” indicates the value of the 5QI for which measured value is reported.

Example 1:  DRB. PdcpF1DelayDl.5QI=”32” where 5QI is the sub-counter name and 32 is the value of the 5QI for which the performance measurement will be reported.

Example 2: DRB.PdcpF1DelayDl.PLMN=”PLMNID_1”, where PLMN is the sub-counter name and PLMNID_1 is the ID of the PLMN for which the performance measurement will be reported.

End of Changes