3GPP TSG-SA5 Meeting #154 	S5-241293

Changsha, China, 15 - 19 April 2024

# Source:	Huawei

# Title:	Add use case for CNF observability

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

For approval.

# 2	References

[1] 	3GPP TR 28.869: “Study on cloud aspects of management and orchestration”.

# 3	Rationale

This contribution proposes to add a use case for CNF observability.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.869 [1].

1st change

### 5.2.x	Observability of CNF using NFV MANO and Kubernetes

#### 5.2.x.1	Description

ETSI NFV Release 4 includes support for containerized virtualized network functions. A typical use case for observability of a Kubernetes-based containerized VNF is depicted in Figure 5.2.x.1-1 and described as follows:

K8s Worker node(s)

3GPP Management System

K8s resources
(Pods / Deployments)

Virtualized Network Function (VNF)

NFV MANO

K8s Master

Figure 5.2.x.1-1: Observability of CNF using NFV MANO and Kubernetes

K8s Worker nodes report information per pod and per cluster to K8s Master. This information includes fault events, performance metrics, and energy consumption data.

K8s Master reports this information per container and per cluster to NFV MANO. Within NFV MANO, the K8s Master is known as Container Infrastructure Service Management (CISM).

NFV MANO aggregates container and cluster information to the level of VNF.

NFV MANO reports resource-related VNF alarms, performance metrics, and energy consumption data to 3GPP Management system.

VNF reports NF-related alarms and performance metrics to 3GPP Management system.

#### 5.2.x.2	Potential requirements

#### 5.2.x.3	Potential solutions

#### 5.2.x.4	Evaluation of solutions

End of changes