3GPP TSG-SA5 Meeting #154 	S5-241386

Changsha, China, 15 – 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0234 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of deallocateNsi and deallocateNssi operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2024-03-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | DeallocateNsi operation takes two parameters as input, networkSliceDN and serviceProfileId. Both are marked as M mandatory. But due to the parent-child model relation between these two objects (TS 28.541), networkSliceDN can be derived from the identified sliceProfile, and does not need to be provided. Therefore the networkSliceDN should be made optional. Similarly, for DeallocateNssi operations and its parameters and their modelling relationship. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the support qualifier for attribute networkSliceDN in the deallocateNsi operation from M to O. Change the support qualifier for attribute networkSliceSubnetDN in the deallocateNssi operation from M to O. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential for interoperability problems if an application of vendor A expects to get a value as input while same application from vendor B does not expect to get a value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2, 6.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.5.3	DeallocateNsi operation

#### 6.5.3.1	Description

This operation is invoked by network slice provisioning MnS consumer to request the provider to deallocate a service profile in an NSI. The provider may terminate the requested NSI or modify the requested NSI without termination to satisfy the request.

#### 6.5.3.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| networkSliceDN | O | The DN of NetworkSlice MOI uniquely identifying the network slice instance. |  |
| serviceProfileId | M | An attribute that globally uniquely identifies the service profile in an NSI. | It specifies the global unifique identifier of the service profile in the NSI which is to be deallocated. |

#### 6.5.3.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |

Second change

### 6.5.4	DeallocateNssi operation

#### 6.5.4.1	Description

This operation is invoked by network slice subnet provisioning MnS consumer to request the provider to deallocate a slice profile in an NSSI. The provider may terminate the requested NSSI or modify the requested NSSI without termination to satisfy the request.

#### 6.5.4.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| networkSliceSubnetDN | O | The DN of NetworkSliceSubnet MOI identifying the network slice subnet instance. |  |
| sliceProfileId | M | An attribute uniquely identifies the slice profile in an NSSI. | It specifies the unifique identifier of the slice profile in the NSSI which is to be deallocated. |

#### 6.5.4.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |

### 6.5.5	Void

End of changes