# 3GPP TSG-SA5 Meeting #154	S5-241419

# 15 - 19 April 2024, Changsha, Hunan, China

# Source:	Nokia

# Title:	CCL scope management

# Document for:	Approval

# Agenda Item:	6.19.4

# 1	Decision/action requested

Discuss and agree on the text

# 2	References

[1] 	3GPP TR 28.867-010 “Closed control loop management”.

# 3	Rationale

CCLs automate the management of network resources thereby taking control away from operators, so CCLs need to be managed including the scope for which the CCL may collect data or which the CCL’s actions may impact.  This pCR introduces the use cases for management of Closed Control Loops scopes.

# 4	Detailed proposal

Start of First change

# 5. 			Use Cases

5.Y3 Use case X3: CCL scope management

5.Y3.1	Description

Each CCL should have a specific scope for which it is responsible. The whole network may be assumed to be a p-dimensional space Sp from which subregions dp Є D maybe created. Accordingly, Sp is the full scope space whose dimension may include time, geography, etc. as showed in Table 1 while dp Є D can be CCL’s scope. In that respect, scope assignment is the mapping of CCLs to regions dЄD that are part of the network’s full scope S. There may be 2 types of scopes – the measurement scope where related measurements are collected and the impact scope which is the scope to which the CCLs actions may have impact. The scopes for the different CCLs can be managed by the Mns consumer.

Table 1: Example scope-space map from which the scope of CCL may be derived

| Scope dimension | Granularity | Example values to be assigned |
| --- | --- | --- |
| Time | Seconds, minutes, days | - Every hour, - Every Saturday at 2:00 hours |
| Network domains |  | - Radio, - Core, |
| Geography | Region/City | - City x - Street y in City x |
| Network Elements | gNB | - gNB X |
|  | Cells | - Cell A on gNB X |
|  | Terminals | - users |
| Resources | slices |  |
|  | Network Function | - Virtual Network Function A - Physical Network Function B |
|  | Virtual Machine |  |
|  | Transport containers (links, flows, …) | - an identifiable link, - a specific flow |
| : |  |  |

5.Y3.2		Potential Requirements

REQ-CCL-COORD-1: The 3GPP management system should support a capability enabling the MnS consumer to manage the scopes of a CCL.

5.Y3.3		Potential Solutions

TBD

5.Y3.4			Evaluation of solutions

TBD

End of modifications