3GPP TSG-SA5 Meeting #154 	S5-241478

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0517 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.255 Update Inter-CHF for clause 5.2.2.22 scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MATRIXX Software |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE_CH_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 5.2.2.22 describes a scenario of UE PDU session charging based on Business converged charging. In this scenario, charging information are exchanged between the C-CHF and the B-CHF for the purpose of UE PDU session charging. Such charging information are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Structure of Inter-CHF Information is extented with the details of the S-NSSAI charging information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE PDU session charging based on Business converged charging cannot be achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.1.6, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [42]	Void.

[43]	3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

[44] - [50]	Void.

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53] 	Void.

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55-56] 	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: " Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] - [69]	Void.

[70]	3GPP TS 28.202: " Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[71]	3GPP TS 28.201: " Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[72]	3GPP TS 28.203: "Charging management; Network Slice Admission Control charging in the 5G System (5GS); Stage 2".

[73] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[103] - [199]	Void

[200]	3GPP TS 23.501:"System Architecture for the 5G System".

[201]	3GPP TS 23.502:"Procedures for the 5G System".

[202]	3GPP TS 23.503:"Policy and Charging Control Framework for the 5G System; Stage 2".

[203]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[204]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[205] - [299]	Void

[300] - [399]	Void.

[400] - [499]	Void.

[500] - [599]	Void.

[600]	3GPP TS 23.228:"IP Multimedia Subsystem (IMS); Stage 2".

[601]	3GPP TS 29.512:" 5G System; Session Management Policy Control Service; Stage 3".

Next change

#### 6.2.1.6	Definition of Inter-CHF information

Specific charging information used for information when the V-CHF have a connection to the H-CHF.

The detailed structure of the Inter-CHF Information can be found in table 6.2.1.6.1.

Table 6.2.1.6.1: Structure of Inter-CHF Information

| Information Element | Category | Description |
| --- | --- | --- |
| Remote CHF resource | OC | This field holds the reference the Charging Data resource in the CHF not directly connected to the NF (i.e., H-CHF) e.g., the resource URI |
| Original NF Consumer Id | OM | This field holds information on the NF triggering the request i.e., SMF |
| S-NSSAI charging information | OC | This field holds the charging information associated to the S-NSSAI the PDU session belongs to. |
| Service profile Charging Information | OC | This field holds Service profile Charging Information defined in clause 6.2.1.3 of TS 28.202 [70] |
| NSAC Container Information | OC | This field holds the NSAC Container Information defined in clause 6.2.1.3 of TS 28.203 [72] |
| NSPA Container Information | OC | This field holds the NSPA Container Information defined in clause 6.2.1.3 of TS 28.201 [71] |

Next change

### 6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMF for 5G data connectivity converged charging or offline only charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2.1 defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging or offline only charging.

Table 6.2.2.1: Supported fields in Charging Data Request message

| Information Element |  |  | Functionality of SMF | FBC |  |  | QBC |  |  | FBC |  |  | QBC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Charging Service | Converged Charging |  |  | Converged Charging |  |  | Offline Only Charging |  |  | Offline Only Charging |  |  |
|  |  |  | Supported Operation Types | I/U/T/E |  |  | I/U/T/E |  |  | I/U/T/E |  |  | I/U/T/E |  |  |
| Session Identifier |  |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  |
| Subscriber Identifier |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| NF Consumer Identification |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Invocation Timestamp |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Invocation Sequence Number |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Retransmission Indicator |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Notify URI |  |  |  | IU- |  |  | IU- |  |  | IU- |  |  | IU- |  |  |
| Supported Features |  |  |  | IU- |  |  | IU- |  |  | - |  |  | - |  |  |
| Service Specification Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Triggers |  |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  |
| Multiple Unit Usage |  |  |  | IUT- |  |  | - |  |  | IUT- |  |  | - |  |  |
| Rating Group |  |  |  | IUT- |  |  | - |  |  | IUT- |  |  | - |  |  |
| Requested Unit |  |  |  | IU-- |  |  | - |  |  | - |  |  | - |  |  |
| Used Unit Container |  |  |  | -UT- |  |  | - |  |  | -UT- |  |  | - |  |  |
| Triggers |  |  |  | -UT- |  |  | - |  |  | -UT- |  |  | - |  |  |
| PDU Container Information |  |  |  | -UT- |  |  | - |  |  | -UT- |  |  | - |  |  |
| UPF ID |  |  |  | IUT- |  |  | - |  |  | IUT- |  |  | - |  |  |
| PDU Session Charging Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Charging Id |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| SMF Charging Id |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Home Provided Charging Id |  |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  |
| SMF Home Provided Charging Id |  |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  |
| User Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| User Location Info |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| IMS Session Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| MA PDU Non 3GPP User Location Info |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| User Location Time |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| MA PDU Non 3GPP User Location Time |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| UE Time Zone |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Presence Reporting Area Information |  |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  |
| PDU Session Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| PDU Session ID |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Network Slice Instance Identifier |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| PDU Type |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| PDU Address |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| SSC Mode |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| MA PDU session information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| SUPI PLMN ID |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Serving Network Function ID |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Serving CN PLMN ID |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| RAT Type |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| MA PDU Non 3GPP RAT Type |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Data Network Name Identifier |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| DNN Selection Mode |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Authorized QoS Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Subscribed QoS Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Authorized Session-AMBR |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Subscribed Session-AMBR |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| PDU session start Time |  |  |  | I--- |  |  | I--- |  |  | I--- |  |  | I--- |  |  |
| PDU session stop Time |  |  |  | --T- |  |  | --T- |  |  | --T- |  |  | --T- |  |  |
| Diagnostics |  |  |  | --T- |  |  | --T- |  |  | --T- |  |  | --T- |  |  |
| Enhanced Diagnostics |  |  |  | --T- |  |  | --T- |  |  | --T- |  |  | --T- |  |  |
| Charging Characteristics |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Charging Characteristics Selection Mode |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| 3GPP PS Data Off Status |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| Session Stop Indicator |  |  |  | --T- |  |  | --T- |  |  | --T- |  |  | --T- |  |  |
| Redundant Transmission Type |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| PDU Session Pair ID |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| 5G LAN Type Service |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| SNPN Information |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| 5GS Bridge Information |  |  |  | IUT- |  |  | - |  |  | - |  |  | - |  |  |
| 5G Multicast Service |  |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  | IUT- |  |  |
| 5G Satellite Access Indicator |  |  |  | IUT- |  |  | IUT- |  |  | - |  |  |  | - |  |  |  |
| Satellite backhaul Information |  |  |  | IUT- |  |  | IUT- |  |  | - |  |  | - |  |  |
| Unit Count Inactivity Timer |  |  |  | IU-- |  |  | - |  |  | IU-- |  |  | - |  |  |
| RAN Secondary RAT Usage Report |  |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  | -UT- |  |  |
| Roaming QBC information |  |  |  | - |  |  | IUT- |  |  | - |  |  | IUT- |  |  |
| Multiple QFI container |  |  |  | - |  |  | IUT- |  |  | - |  |  | IUT- |  |  |
| UPF ID |  |  |  | - |  |  | IUT- |  |  | - |  |  | IUT- |  |  |
| Roaming Charging Profile |  |  |  | - |  |  | IU-- |  |  | - |  |  | IU-- |  |  |
| Inter-CHF Information |  |  |  | IUT- |  |  | IUT- |  |  | - |  |  | - |  |  |
| Remote CHF resource |  |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| Original NF Consumer Id |  |  |  | IUT- |  |  | IUT- |  |  | - |  |  | - |  |  |
| S-NSSAI charging information |  |  |  | I--- |  |  | - |  |  | - |  |  | - |  |  |

Table 6.2.2.2 defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging or offline only charging.

Table 6.2.2.2: Supported fields in Charging Data Response message

| Information Element |  | Functionality of SMF | FBC |  | QBC |  | FBC |  | QBC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Charging Service | Converged Charging |  | Converged Charging |  | Offline Only Charging |  | Offline Only Charging |  |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | I--- |  | I--- |  | I--- |  | I--- |  |
| Invocation Timestamp |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Invocation Result |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Invocation Sequence Number |  |  | IUT- |  | IUT- |  | IUT- |  | IUT- |  |
| Session Failover |  |  | IU-- |  | IUT- |  | IU-- |  | IUT- |  |
| Supported Features |  |  | IU-- |  | IU-- |  | - |  | - |  |
| Triggers |  |  | IU-- |  | IU-- |  | IU-- |  | IU-- |  |
| Multiple Unit information |  |  | IU-- |  | - |  | IU-- |  | - |  |
| Result Code |  |  | IU-- |  | - |  | IU-- |  | - |  |
| Rating Group |  |  | IU-- |  | - |  | IU-- |  | - |  |
| UPF ID |  |  | IU-- |  | - |  | IU-- |  | - |  |
| Granted Unit |  |  | IU-- |  | - |  | - |  | - |  |
| Validity Time |  |  | IU-- |  | - |  | - |  | - |  |
| Final Unit Indication |  |  | IU-- |  | - |  | - |  | - |  |
| Time Quota Threshold |  |  | IU-- |  | - |  | - |  | - |  |
| Volume Quota Threshold |  |  | IU-- |  | - |  | - |  | - |  |
| Unit Quota Threshold |  |  | IU-- |  | - |  | - |  | - |  |
| Quota Holding Time |  |  | IU-- |  | - |  | - |  | - |  |
| Triggers |  |  | IU-- |  | - |  | IU-- |  | - |  |
| PDU Session Charging Information |  |  | IU-- |  | IU-- |  | IU-- |  | IU-- |  |
| Charging Id |  |  | - |  | - |  | - |  | - |  |
| SMF Charging Id |  |  | - |  | - |  | - |  | - |  |
| Home Provided Charging Id |  |  | - |  | - |  | - |  | - |  |
| SMF Home Provided Charging Id |  |  | - |  | - |  | - |  | - |  |
| User Information |  |  | - |  | - |  | - |  | - |  |
| User Location Info |  |  | - |  | - |  | - |  | - |  |
| IMS Session Information |  |  | - |  | - |  | - |  | - |  |
| MA PDU Non 3GPP User Location info |  |  | - |  | - |  | - |  | - |  |
| User Location Time |  |  | - |  | - |  | - |  | - |  |
| MA PDU Non 3GPP User Location Time |  |  | - |  | - |  | - |  | - |  |
| UE Time Zone |  |  | - |  | - |  | - |  | - |  |
| Presence Reporting Area Information |  |  | IU-- |  | IU-- |  | IU-- |  | IU-- |  |
| PDU Session Information |  |  | - |  | - |  | - |  | - |  |
| PDU Session ID |  |  | - |  | - |  | - |  | - |  |
| Network Slice Instance Identifier |  |  | - |  | - |  | - |  | - |  |
| PDU Type |  |  | - |  | - |  | - |  | - |  |
| PDU Address |  |  | - |  | - |  | - |  | - |  |
| SSC Mode |  |  | - |  | - |  | - |  | - |  |
| MA PDU session information |  |  | - |  | - |  | - |  | - |  |
| SUPI PLMN ID |  |  | - |  | - |  | - |  | - |  |
| Serving Network Function ID |  |  | - |  | - |  | - |  | - |  |
| Serving CN PLMN ID |  |  | - |  | - |  | - |  | - |  |
| RAT Type |  |  | - |  | - |  | - |  | - |  |
| MA PDU Non 3GPP RAT Type |  |  | - |  | - |  | - |  | - |  |
| Data Network Name Identifier |  |  | - |  | - |  | - |  | - |  |
| DNN Selection Mode |  |  | - |  | - |  | - |  | - |  |
| Authorized QoS Information |  |  | - |  | - |  | - |  | - |  |
| Subscribed QoS Information |  |  | - |  | - |  | - |  | - |  |
| Authorized Session-AMBR |  |  | - |  | - |  | - |  | - |  |
| Subscribed Session-AMBR |  |  | - |  | - |  | - |  | - |  |
| PDU session start Time |  |  | - |  | - |  | - |  | - |  |
| PDU session stop Time |  |  | - |  | - |  | - |  | - |  |
| Diagnostics |  |  | - |  | - |  | - |  | - |  |
| Enhanced Diagnostics |  |  | - |  | - |  | - |  | - |  |
| Charging Characteristics |  |  | - |  | - |  | - |  | - |  |
| Charging Characteristics Selection Mode |  |  | - |  | - |  | - |  | - |  |
| 3GPP PS Data Off Status |  |  | - |  | - |  | - |  | - |  |
| Session Stop Indicator |  |  | - |  | - |  | - |  | - |  |
| Redundant Transmission Type |  |  | - |  | - |  | - |  | - |  |
| PDU Session Pair ID |  |  | - |  | - |  | - |  | - |  |
| 5G LAN Type Service |  |  | - |  | - |  | - |  | - |  |
| SNPN Information |  |  | - |  | - |  | - |  | - |  |
| 5GS TSN Bridge Information |  |  | - |  | - |  | - |  | - |  |
| 5G Multicast Service |  |  | - |  | - |  | - |  | - |  |
| Unit Count Inactivity Timer |  |  | IU-- |  | - |  | - |  | - |  |
| RAN Secondary RAT Usage Report |  |  | - |  | - |  | - |  | - |  |
| Roaming QBC information |  |  | - |  | IU-- |  | - |  | IU-- |  |
| Multiple QFI container |  |  | - |  | - |  | - |  | - |  |
| UPF ID |  |  | - |  | - |  | - |  | - |  |
| Roaming Charging Profile |  |  | - |  | IU-- |  | - |  | IU-- |  |
| Inter-CHF Information |  |  | IUT- |  | IUT- |  | - |  | - |  |
| Remote CHF resource |  |  | IUT- |  | IUT- |  | - |  | - |  |
| Original NF Consumer Id |  |  | - |  | - |  | - |  | - |  |
| S-NSSAI charging information |  |  | I--- |  | - |  | - |  | - |  |

End of changes