3GPP TSG-SA5 Meeting #154 	S5-241589

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0544 | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.552 Correction to Number of Active UEs measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The max number of Active UEs in the UL per cell is incorrect due to the missed implementation of the corresponding change of agreed S5-211489. - Meanwhile, some typos and unclarity need to be fixed in other Number of Active UEs measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct 5.1.1.23.4 Max number of Active UEs in the UL per cell. - Correct typos and unclear things in other clauses for Number of Active UEs measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect performance measurements in TS 28.552. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.23, 5.1.1.23.1, 5.1.1.23.2, 5.1.1.23.3, 5.1.1.23.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.1.1.23	Number of Active UEs

##### 5.1.1.23.1	Mean number of Active UEs in the DL per cell

a)	This measurement provides the mean number of active UEs in the DL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by aggregating the measurement "Mean number of Active UEs in the DL per DRB per cell" (see clause 4.2.1.3.2 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanActiveUeDl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI,

where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.2	Max number of Active UEs in the DL per cell

a)	This measurement provides the max number of active UEs in the DL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

b)	DER (n=1).

c)	This measurement is defined according to measurement "Max number of Active UEs in the DL per DRB per cell" (see clause 4.2.1.3.3 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MaxActiveUeDl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI,

where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.3	Mean number of Active UEs in the UL per cell

a)	This measurement provides the mean number of active UEs in the UL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained by aggregating the measurement "Mean number of Active UEs in the UL per DRB per cell" (see clause 4.2.1.3.4 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanActiveUeUl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI,

where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.4	Max number of Active UEs in the UL per cell

a)	This measurement provides the max number of active UEs in the UL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

b)	DER (n=1)

c)	This measurement is defined by the measurement "Max number of Active UEs in the UL per DRB per cell" (see clause 4.2.1.3.5 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.
[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MaxActiveUeUl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI,
where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

End of changes