3GPP TSG-SA5 Meeting #154 	S5-241599

Changsha, China, 19 April - 15 April 2024

# Source:	AsiaInfo

# Title:	Add use case for CCL conflict

# Document for:	Approval

Agenda Item:	6.19.4

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.867 v0.0.0 Study on Closed Control Loop Management

3	Rationale

This provides the use case for CCL conflicts

4	Detailed proposal

It proposes to make the following changes to TR 28.867.

First Change

5. Use cases

5.X use case X CCL conflicts

5.X.1 Description

Conflict between CCL may happen during the initiation when the goals of the new CCL contradicts with the goal of an existing CCL. These conflicts are called initiation conflict. For example, goal of improve energy efficiency and objective of resource power optimization. The MnS consumer creates the goals for new CCL. On receiving and after analysing the goals, the MnS produce may detect if there are conflicts with the goals of an existing CCL. If there are conflicts, the producer may notify the MnS consumer about goals conflicts. The MnS consumer may decide to update or create a new goal to mitigate the conflicts.

In addition, the MnS producer may detect the conflict between action plans of CCLs before execute step when several CCLs can target the same managed function. These conflicts are called execution conflict. For example, executing network performance optimization CCL and energy saving CCL in a RAN domain. The MnS producer may select the correct set of action plans, after identifying the confliction of action plans, this may imply creating new or disable existing action plans for a given CCL to mitigate the conflicts.

5.X.2 Potential Requirements

REQ-CCL-CON-01 The 3GPP management system shall have the capability to detect conflicts between Closed Control Loops, including the conflicts of goals and action plans.

REQ-CCL-CON-02 The 3GPP management system shall have the capability to mitigate conflicts between Closed Control Loops, including the conflicts of goals and action plans.

5.X.3 Potential Solutions

TBD.