3GPP TSG-SA5 Meeting #154 	S5-241678

Changsha, China, 15 - 19 April 2024

# Source:	Nokia

# Title:	Discussion paper on enhancing MnsInfo by MnS capabilities

# Document for:	Information, Discussion

Agenda Item:	6.19.10

# 1	Decision/action requested

The group is asked to discuss and endorse the proposal.

# 2	References

[1]	3GPP TR 28.842 Study on data management phase 2

[2]	3GPP TS 28.622 Generic Network Resource Model (NRM).

[3]	3GPP TS 28.532 Generic management services.

[4]	SP-231433 Study on Enhanced OAM for management exposure to external consumers.

# 3	Rationale

TR 28.842 [1] concludes and recommends to do normative work for the possible solution of discovery of management data.

Currently the set of attributes in MnsInfo [2] is very limited and neither suited to describe the MnS nor the management data provided by this MnS in detail. Therefore in any case the description of management data provided by MnsInfo needs to be enhanced (key issue #4 of [1]), e.g. as proposed by the potential solution (key issue #7 of [1]).

On one hand, currently MnsInfo is not able to describe the detailed properties of the data that a specific MnS is able to provide. This issue has been documented in [1] and will be worked on.

On the other hand, currently the value of attribute mnsType of MnsInfo almost always will indicate “ProvMnS”, since basically all interface-related managemen capabilities (e.g. jobs to transfer of PM files, management of subscriptions for alarms, management of collection of trace an PM, etc.) as well as the “real” configuration management of NF are based on the generic provisioing MnS [3]. Currently no attribute of MnsInfo is able to identify the capabilities of a MnS. However, for a discovery of specific MnS it’s required to augment MnsInfo by the capabilities that a MnS offers. Definition of such capabilities is subject of WT-1.2 of WI SP-231433 Study on Enhanced OAM for management exposure [4].

# 4	Detailed proposal

In order to keep the description of MnS and their capabilities aligned between the discovery via MnsRegistry/MnsInfo and the discovery via CAPIF the WT-1.2 of [4] needs to consider also the discovery of management data and related MnS. The results of WT-1.2 of [4] should be considered for the normative work on key issue #4 and #7 of [1].