3GPP TSG-SA5 Meeting #154	S5-241696

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0553 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Update DL Packet Loss rate to include delay threshold |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecomunication Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in TS 23.501, for GBR QoS Flows with Delay-critical GBR resource type, a packet which is delayed more than PDB(Packet Delay Budget) is counted as lost. And “Packet Uu Loss Rate with delay threshold in the DL” for delay critical scenario is introduced in TS 38.314 as a supplement to the existing packet loss rate measurement. Measurement of DL Packet Loss Rate on Uu in TS 28.552 needs to be updated correspondingly to align with TS 38.314. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update measurement of “DL Packet Loss Rate on Uu” to include Packet Loss Rate with delay threshold. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Packets loss rate measurements cannot cover delay critical scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.1.1.35	DL Packet Loss rate on Uu

a)	This measurement provides the DL Packet (i.e., RLC SDU) Loss rate including any packets not successfully transmitted or packets successfully received but delayed more than a delay threshold that can be used when the resource type of corresponding QoS Flow is Delay-critical GBR (clause 5.7.3.4 in TS 23.501 [4]) on Uu interface for an NR cell. The measurement is split into subcounters per PLMN ID per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

b)	CC.

c)	This measurement is obtained based on the following parameters defined in TS 38.314 [29]:

| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| --- | --- |
| Dexd(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which is transmitted over air interface and positively acknowledged but the DL delay of the RLC SDU is more than corresponding delay threshold during time period T. Delay threshold of this measurement can be determined by NW implementation. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed. |
| drbid | The identity of the measured DRB. |

The gNB takes the following calculation for each PLMN ID per mapped 5QI and per supported S-NSSAI:

DlossT,drbid+Dexd(T,drbid)*1000000NT,drbid+DlossT,drbid

NOTE: DexdT,drbid, number of DL packets which are delayed more than PDB, is used when the resource type of corresponding QoS Flow is Delay-critical GBR (see 23.501 [4] and 38.314 [29]). On the contrary, when the resource type of corresponding QoS Flow is not Delay-critical GBR, the value of DexdT,drbid is equal to zero.

d)	Each measurement is an integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs.

e)	DRB.PacketLossRateUu.Filter, 
Where Filter is a combination of PLMN ID and QoS level and S-NSSAI. 
The QoS level represents the mapped 5QI or QCI.

f)	NRCellDU

g)	Valid for packet switched traffic.

h)	5GS.

End of change