3GPP TSG-SA5 Meeting #155 	S5-242640

Jeju, South Korea, 27 - 31 May 2024

# Source:	China Telecom

# Title:	Add use case and corresponding requirements for measuring the amount of data collected by NWDAF through DCCF

# Document for:	Approval

Agenda Item:	6.19.18

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TS 23.288-i40: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[2]	3GPP TS 28.877-010: "Study on Enhancement of the management aspects related to Network Data Analytics Functions (NWDAF) Phase 2".

3	Rationale

According to TS 23.288 [1], Data Collection Coordination and Delivery coordinates the collection and distribution of data requested by NF consumers. It prevents data sources from having to handle multiple subscriptions for the same data and send multiple notifications containing the same information due to uncoordinated requests from data consumers. Data Collection Coordination and Delivery is supported by a DCCF via or by an NWDAF hosting DCCF.

With enhanced procedures for data collection defined in TS 23.288[1] clause 6.2.6.3, data collection may be performed via DCCF. The procedure is used by a data consumer (e.g. NWDAF) to obtain data and be notified of events via the DCCF using Ndccf_DataManagement_Subscribe service operation. Two options are supported: data delivered via the DCCF, and data delivered via a messaging framework. Which option to be used is determined by DCCF configuration.

Whether the NWDAF directly contacts the Data Source or goes via the DCCF is based on configuration of the data consumer. NWDAF directly collects data from NFs via the Nnf_EventExposure_Subscribe service operation, while data collection through DCCF is achieved by subscribing to the DCCF using the Ndccf_DataManagement_Subscribe service operation. Therefore, in the case where NWDAF collects data via DCCF, the total amount of data collected from all NFs by NWDAF through DCCF are potentially meaningful performance measurements for operators. These measurements can reflect the transmission pressure of NWDAF collecting data through DCCF, and further calculate the total amount of data collected by NWDAF from different NFs.

As a conclusion, based on the discussion above, it is proposed to add a new use case and corresponding requirement to TR 28.877 on measuring amount of data collected via DCCF for an NWDAF.

4	Detailed proposal

Start of 1st Change

# 4	Use cases, potential requirements and possible solutions

## 4.X	Use case #X: Management enhancement on measuring amount of data collected by NWDAF through DCCF

## 4.X.1	Use cases

According to TS 23.288 [1], NWDAF is able to directly collect data from NFs via the Nnf_EventExposure_Subscribe service operation, or collect data through DCCF by subscribing to the DCCF using the Ndccf_DataManagement_Subscribe service operation. Data Collection Coordination and Delivery coordinates the collection and distribution of data requested by NF consumers. It prevents data sources from having to handle multiple subscriptions for the same data and send multiple notifications containing the same information due to uncoordinated requests from data consumers. Data Collection Coordination and Delivery is supported by a DCCF or by an NWDAF hosting DCCF.

Therefore, in the case where NWDAF collects data via DCCF, the total amount of data collected from NFs by NWDAF via DCCF is a potentially meaningful performance measurement for operators. This measurement intuitively reflects how much data is collected by NWDAF via DCCF, and can be further used to estimate the data processing load and data collection efficiency of NWDAF.

### 4.X.2	Potential requirements

REQ-NWDAF-PM-DCVOLUMEDCCF-01：the 3GPP management system shall have a capability to provide a measurement of the total amount of data collected by an NWDAF via DCCF from all NFs over a period of time.