3GPP TSG SA WG5 Meeting #155	S5-242993

Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0228 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.290 Enhance the input for CHF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Verizon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy |  |  |  |  |  | Date: |  |  | 2024-05-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on TR 28.840 conclusion, solution #1.3, #1.5, #3.3, #3.4 are recommended and can be supported by existing NRF discovery mechanism specified in TS 29.510. Solution #1.3 and #1.5 support the CHF discovery and selection based on NF consumer location information, re-use the locality/extLocality or servingScope attribute in NF profile. Solution #3.3 and #3.4 support the CHF discovery and selection based on tenant information, which also re-use the existing attribute, i.e. allowed S-NSSAI in NF profile and range of SUPIs in ChfInfo. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Extend the list of inputs for CHF invoking the Nnrf_NFManagement_NFRegister or Nnrf_NFDiscovery service. 2. Add the source reference for the listed inputs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging service may not be efficient, since the NF consumer may not be able to select CHF instance with the best matched location or tenant information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-242750 |  |  |  |  |  |  |  |  |

First change

6.1	NF service framework

5G Charging Function supports to interact with NRF, as specified in clause 7.1 of TS 23.501 [201] and clauses 4.17 and 5.2.7 of TS 23.502 [202] to enable following functionalities:

-	CHF instance(s) registration, CHF service(s) instance(s) registration in a CHF instance.

-	CHF instance(s) update, CHF service(s) instance(s) update in a CHF instance.

-	CHF instance(s) deregistration.

-	CHF instance(s) and CHF service(s) instance(s) discovery by CHF service consumer.

The services specified in clause 7.2.6 TS 23.501 [201] may be used and the interaction is described in TS 29.510 [300]:

-	Nnrf_NFManagement.

-	Nnrf_NFDiscovery.

-	Nnrf_AccessToken.

The Nnrf_NFManagement_NFRegister service invoked by CHF for CHF instance(s) and CHF service(s) instance(s) registration described in the TS 29.510 [300] may include in particular:

-	The attributes in ChfInfo in NF Profile, as defined in clause 6.1.6.2.32, for example, Range(s) of SUPIs, Range(s) of GPSIs, Range(s) of PLMNs, CHF Group ID.

-	The common attributes in NF Profile, as defined in clause 6.1.6.2.2, for example, locality / extLocality, servingScope, allowedNssais, nfSetIdList that identifies the CHF set ID, nfServiceSetIdList that identifies the CHF service set ID.

These parameters may also be used by CHF service consumer(s) invoking the Nnrf_NFDiscovery service for the CHF instance(s) and CHF service(s) instance(s) discovery.

A CHF instance is either a part of:

-	a primary CHF instance and secondary CHF instance pair, or

-	a CHF set, which refers to a group of interchangeable CHF instances of the same type, supporting the same services and the same Network Slice(s), as specified in clause 3.1 TS 23.501, or

-	a CHF Group, which refers to one or more CHF instances managing a specific set of SUPIs, as specified in clause 3.1 TS 23.501.

End of change