3GPP TSG-SA5 Meeting #155 	S5-243157

Jeju, South Korea, 27 - 31 May 2024

# Source:	ZTE Corporation

# Title:	pCR TR 28.914 add use case, requirements and solution for negotiation on the possible outcomes during the fulfilment phase

# Document for:	Approval

Agenda Item:	6.19.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

(Reference - in list form - should be made to previous related SA5/3GPP/etc. documents.)

[1]	Study on intent driven management service for mobile network phase 3 v0.0.0

[2]	SP-231737 "Study on intent driven management services for mobile network phase 3"

3	Rationale

When the MnS Consumer expresses an intent for fulfilment, the producer may have numerous solutions for fulfilment. The MnS consumer may not have the capability to determine which solution is better, which means it should negotiate with the MnS Consumer to make a decision. It proposes to add a key issue for introducing the MnS capability to negotiate on the possible outcomes/solutions during the fulfilment phase.

This contribution is related to WT-3.1 of the FS_IDMS_MN_Ph3 SID [2], which is to investigate intent negotiation functionalities.

4	Detailed proposal

Start of First change

5	Use cases

5.X Use case#<X>: Negotiation on the possible outcomes during the fulfilment phase

5.X.1 Description

In TS 28.312 R18, the intent fulfilment feasibility check is already supported. When intent intent-driven MnS producer receives the intent instance creation or modification request, it automatically conducts a feasibility check to determine whether the intent instance is feasible. If the feasibility check result is feasible, the MnS producer performs the service or network management tasks to satisfy the intent instance.

However, the MnS Producer cannot decide the content of intent it receives and the MnS Consumer may express a feasible intent but the producer has multiple ways to fulfil it. One of these solutions might be better in one aspect and another solution might be better in another aspect. Although this intent is feasible, the producer may not have the knowledge and capability to determine which solution is appropriate. For example, the MnS Consumer may express intent for energy saving but only specify the target for energy consumption reduction without more constraints, e.g., reduce 20% energy consumption. There will be numerous solutions to fulfil the energy consumption target with various impacts, e.g., one solution may reduce the coverage and one may reduce the user number. The consumer may have concerns on the possible impact of the selected solution by the Producer, which means that negotiation with the MnS Consumer is necessary. Therefore, it is important to introduce the MnS capability to enable the MnS producer to provide feedback (e.g., possible solutions, possible outcomes, and possible impacts) to allow the MnS Consumer to determine the appropriate solution.

5.X.2 Potential requirements

REQ-Intent_Negotiation _01:  The intent-driven MnS should have the capability to enable the MnS producer to provide possible outcomes to the MnS Consumer.

REQ-Intent_Negotiation_02: The intent-driven MnS should have the capability to allow MnS Consumer decide appropriate outcome for fulfilment based on possible outcomes.

Note: the definition for outcome is FFS.

5.X.3 Potential solutions

5.X.3.i Potential solution #<i>: Negotiation on the possible outcomes during the fulfilment phase

This solution proposes to enhance the existing Intent IOC and IntentReport IOC defined in TS 28.312 [X] to enable the MnS producer to provide feedback including the possible outcomes to the MnS Consumer. Following are the proposed enhancements:

Enhancement on Intent IOC: Introduce attribute “recommendationType” in the Intent IOC to determine the possible feedback including the possible outcomes the MnS Consumer can obtain from the MnS Producer. The allowed values for “recommendationType” are as follows:

- RecommendedOutcomes, it represents that the MnS Consumer requests the MnS Producer to provide possible outcomes based on the received intent.

Enhancement on IntentReport IOC: Introduce IntentNegotiationReoprt <<dataType>> as an attribute of IntentReport IOC to represent the feedback including possible outcomes provided by MnS producer to allow the MnS Consumer determine the appropriate outcome. IntentNegotiationReoprt <<dataType>> includes following attributes:

- PossibleOutcomeList, supported when the “recommendationType” is “RecommendedOutcomes”. It includes a list of possible outcomes.

Note: the definition for outcome is FFS.

Note: The “PossibleOutcomeList” will include the potential impact on the related ExpectationObjects.

End of First change