| 3GPP TR 28.842 V18.1.0 (2024-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on data management phase 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Key issues	8

4.1	Key issue #1: Retrieval of stored management data	8

4.1.1	Description	8

4.1.2	Problem Statement	8

4.1.3	Potential Solution #1: Use of managementDataCollection IOC for retrieval of stored management data	8

4.2	Key issue #2: Discovery of stored management data	8

4.2.1	Description	8

4.2.2	Problem Statement	9

4.2.3	Potential solutions	9

4.2.3.1	Discover management data using try and error	9

4.2.3.2	Discover management data using one central MnsRegistry	9

4.2.3.3	Discover management data using hierarchical MnsRegistries	9

4.2.3.4	Discovery of Stored Management Data Producer supporting retrieval of stored data	10

4.3	Key issue #3: Consolidated reporting of mgmt. data	10

4.3.1	Description	10

4.3.1.1	Observations	10

4.3.2	Problem statement	11

4.3.3	Potential solutions	11

4.3.3.1	Define how ManagementDataCollection "consolidates" the reporting data	11

4.4	Key issue#4: Describe management data in more detail	11

4.4.1	Description	11

4.4.2	Observations	13

4.4.3	Problem statement	13

4.5	Key issue #5: Managing external management data	13

4.5.1	Description	13

4.5.2	Problem Statement	14

4.5.3	Potential solution	14

4.6	Key Issue #6: Define capabilities for Management Data Reporting	15

4.6.1	Description	15

4.6.2	Problem Statement	15

4.6.3	Potential Solutions	15

4.6.3.1	Add SupportedDataReporting to report supported capabilities	15

4.6.3.1.1	SupportedDataReporting <<IOC>>	15

4.6.3.2	Add reporting capabilities to MnSAgent	16

4.7	Key Issue #7: Solutions for discovery of management data	17

4.7.1	Description	17

4.7.2	Problem Statement	17

4.7.3	Potential solutions	18

4.7.3.1	Potential solution#1 Enhance the MnS Registry NRM fragment to support discovery of management data	18

4.8	Key Issue #8: Key Issue and potential solution for reporting data for managementDataCollection	20

4.8.1	Description	20

4.8.2	Potential solution	20

4.8.2.1	Potential solution#1 Area based KPI	20

5	Recommendations and conclusions	20

Annex A:	Change history	22

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies potential enhancements related to management of management data, specifically enhancing mechanisms for discovery and reporting of management data, retrieval of stored management data, and managing external management data.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[3]	3GPP TS 32.432: "Subscriber and equipment trace; Trace control and configuration management".

[4]	3GPP TS 28.532: "Generic management services".

[5]	3GPP TS 28.622: "Generic Network Resource Model (NRM)".

[6]	3GPP TS 28.533: "Architecture framework".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Key issues

## 4.1	Key issue #1: Retrieval of stored management data

### 4.1.1	Description

Clause 6.3 of [2] contains description and the requirements for management data storage; specifically, to enable reuse of management data for multiple management purposes by various MnS consumers. Clause 6.3.3 of [2] lists the following requirement:

REQ-MDMS-CON-3: The 3GPP management system shall enable an authorized data consumer to retrieve stored management data.

The stage 2 and stage 3 solutions for these requirements are underspecified, including:

-	Solutions to retrieve the stored management data by an authorized data consumer.

-	Possibility of using a unified way to report the requested data irrespective of whether it is newly generated data or historical management data to a data consumer.

### 4.1.2	Problem Statement

The details of the mechanisms of how the management data may be retrieved to support the requirement listed in Clause 6.3.3 of [2] are underspecified. Specifically, the study should include solutions that provide details using existing or new methods to support the ability for an authorized data consumer to retrieve historical management data.

### 4.1.3	Potential Solution #1: Use of managementDataCollection IOC for retrieval of stored management data

ManagementDataCollection [5] IOC represents a management data collection request job. This potential solution proposes use of managementDataCollection to also include collection of historical management data.

The changes that are required to enable use of managementDataCollection for retrieval of stored management data include the following:

-	Updating the description in Clause 4.3.47 in [5] to indicate applicability of managementDataCollection to request stored (historical) management data.

-	Updating the description of the "collectionTimeWindow" attribute to include that a time window with a start time in the past indicates historical management data.

-	Updating the description of the "reportingCtrl" attribute to enable one time reporting as well as data file-based reporting.

-	Updating the description of the "targetNodeFilter" attribute to allow selection of an exact object instance.

Note: 	Whether and how the attributes of ManagementDataCollection IOC need to be further enhanced for querying historical management data is FFS.

## 4.2	Key issue #2: Discovery of stored management data

### 4.2.1	Description

Clause 6.3 of [2] contains description and the requirements for management data storage to enable reuse of management data for multiple management purposes by various MnS consumers.

For the management data consumer to be able to access the stored management data, it needs to be able to discover stored management data; this requirement is captured in Clause 6.3.3 of [2]:

REQ-MDMS-CON-2: The 3GPP management system shall enable an authorized data consumer to discover stored management data.

The solution for this requirement is not specified. Enabling discovery of management data based on specific management data types, parameters, and context data, e.g., performance measurement type, time window, producer type, etc., would be beneficial for a wide set of use cases including training of various ML models.

### 4.2.2	Problem Statement

The solution for discovery of stored (historical) management data is not specified. The study should include the solutions that provide mechanisms for an authorized management data consumer to discover stored management data. Whether and how to enable discovery of stored management data based on potential conditions, e.g., management data types, parameters, and context data should also be studied.

### 4.2.3	Potential solutions

#### 4.2.3.1	Discover management data using try and error

Discovery of management data might be based on a simplistic try and error mechanism, where the MnS consumer tries to request the needed management data from all MnS producers known to provide management data for the requested mnsScope until the request succeeds (or all tries fail).

In case of very few MnS producers this is acceptable. However, this might be inefficient in case of many MnS producers and in case of management functions that need to select sources of data dynamically depending on the task, e.g. a management functin that implements AI/ML training needs to select the data depending on the training task.

#### 4.2.3.2	Discover management data using one central MnsRegistry

Discovery of management data might be based on one central MnsRegistry [5], where MnS producers register as part of an augmented MnsInfo the properties to the managagment data they are able to produce.

Since management data is characterised by several static and dynamic properties the MnS producers need to update all dynamic properties permanently, especially the available history in time.

In case of many MnS producers and in case of many dynamically changing properties of management data this solution might not scale well due to the permanent updates of the MnsRegistry. Although from MnS consumer point of view this solution is very convenient.

#### 4.2.3.3	Discover management data using hierarchical MnsRegistries

Management data is characterised by several static and dynamic properties. E.g. the type of management data and source objects (mnsScope) of the measurement remain stable, while the history available at a MnS producer changes dynamically over time.

The MnS producers -especially of historical data- might register these properties of the management data in two (or more) MnsRegistries: Properties that are mostly static like the association between type of data and address of related MnS producer can be stored in a central MnsRegistry, while properties that are changing over time like the available range of time can be stored in multiple MnsRegistries which are serving smaller scopes or are even local to the MnS producers. This results in a hierachy of registries.

An interested MnS consumer would first approach the central MnsRegistry to select potential MnS producers based on the static properties. In a second step the MnS consumer would approach the corresponding local MnsRegistries of the selected potential MnS producers to select the (final) MnS producer based on the dynamic properties of the management data.

As a potential solution the MnsInfo of MnsRegistry can be enhanced by the properties of management data which a given MnS producer is able to provide.

The detailed criteria for partitioning the properties into "static" and "dynamic" depends on the specific situation of the management system.

#### 4.2.3.4	Discovery of Stored Management Data Producer supporting retrieval of stored data

In this solution an authorized consumer inquires the MnS Producer supporting retrieval of the stored management data for a specific stored data.

Note 1: 	The mechanism for retrieving the stored data is addressed in key issue #1 (Clause 4.1).

There is an overhead associated with attempting to retrieve historical data from a MnS Producer that does not support retrieval of stored data; the overhead becomes significant in large scale deployments. Hence, the authorized consumer learns in advance whether MnS Producer supports the capability of reporting stored management data using the advertised NRM capabilities of MnS Producer.

Key issue #6 in Clause 4.6 highlights the need for a standard way for discovering the management data reporting capabilities of an MnS producer. The potential solution(s) for key issue #6 provide a way for management data producers to report the supported data management reporting capabilities.

In this solution, the capability to report stored management data is added as a capability to be discovered as part of data management reporting capabilities.

The potential solution #1 for key issue #6 adds a new IOC to NRM to report the supported Data Management Reporting capabilities. This solution proposes that a new attribute, "reportingOfStoredDataSupported," is added to SupportedDataReporting IOC to indicate whether MnS Producer supports reporting of stored management data.

Note 2: 	This potential solution and addition of "support for reporting of stored management data" as a capability to be advertised is applicable regardless of which potential solution is recommended for key issue #6.

## 4.3	Key issue #3: Consolidated reporting of mgmt. data

### 4.3.1	Description

Per  28.622 [5], clause 4.3.31.1, for file-based reporting the output of a PerfMetricJob is contained in the same file:

For file-based reporting, all performance metrics that are produced related to a "PerfMetricJob" instance for a reporting period shall be stored in a single reporting file.

Depending on scope, the output of a single ManagementDataCollection may include output from multiple PerfMetricJob instances, per 28.622 [5], clause 4.3.47.1:

The MnS producer may derive multiple jobs ("PerfMetricJob", "TraceJob") from a single "ManagementDataCollection" job for collecting the required management data. If the MnS producer receives the collected data from multiple sources, it consolidates the data into a set of management data for reporting.

#### 4.3.1.1	Observations

The output of each PerfMetricJob will be in its respective file. Therefore, the output from a single ManagementDataCollection instance could contain multiple files. It is unclear how these should be handled by the same ManagementDataCollection job.

For example, based on collection scope the consolidated PM output could be collected in 5G file format 28.532 [4] and/or 4G file format 32.432 [3]:

| measTypes | This is the list of measurement types for which the following, analogous list of measurement values ("measValues") pertains. The measurement types for UMTS and combined GSM/UMTS networks are specified in TS 32.405 [12], TS 32.406 [13], TS 32.407 [14], TS 32.408 [15] and for IMS in TS 32.409 [16]. Measurement types for E-UTRAN are specified in TS 32.425 [17] and for EPC in TS 32.426 [18]. Measurement types for Home Node B (HNB) Subsystem (HNS) are defined in TS 32.452 [19] and for Home enhanced Node B (HeNB) Subsystem (HeNS) in TS.32.453 [20].The GSM only measurement types are defined in TS 52.402 [7]. The measurement types for NR and 5GC are specified in TS 28.552 [21] and TS 32.404 [22] |
| --- | --- |

It is assumed data from each PerfMetricJob will be collected in its respective format.

This means the following PerfMetricJob output could be created for a single ManagementDataCollection:

-	PM data in 32.432 and PM data in 28.532	(different schema; separate files)

-	PM data in 32.432 	 (same schema; separate files)

-	PM data in 28.532 	(same schema; separate files)

ManagementDataCollection description indicates that the output will be "consolidated" per 28.622 [5], clause 4.3.47.1:

The MnS producer may derive multiple jobs ("PerfMetricJob", "TraceJob") from a single "ManagementDataCollection" job for collecting the required management data. If the MnS producer receives the collected data from multiple sources, it consolidates the data into a set of management data for reporting.

and that notifyFileReady notifications will be produced for the consolidated set of management data per 28.622 [5], clause 4.3.47.4.

### 4.3.2	Problem statement

Depending on the scope of ManagementDataCollection request the output will include a mix of files, potentially in different formats for the same management data type, e.g. PM data in different formats.

It is not clear what it means to "consolidate" the data, or how the output is actually to be reported.

### 4.3.3	Potential solutions

#### 4.3.3.1	Define how ManagementDataCollection "consolidates" the reporting data

The description should be updated to define what it means to "consolidate" the output data.

It should be clarified that the output from a ManagementDataCollection using input from multiple PerfMetricJob instances will produce a consolidated set of output data. The definition of which needs to be clarified, for example:

-	Combine the files, i.e. into a single output such as a 'zip' file; send notifyFileReady notification(s) for the resulting file(s)

-	Combine the file content, taking into account cases where the format may be mixed; send notifyFileReady notification(s) for the resulting file(s)

A benefit of this approach is metadata can reflect the contents of the set consolidated data, e.g. the filename could indicate the 'area' (as requested for the management data collection) which is not present in the constituent files.

## 4.4	Key issue#4: Describe management data in more detail

### 4.4.1	Description

Currently the management system offers a MnS consumer to control the collection of PM by the following means:

-	The MnsRegistry [5] offers the MnS consumer means to find the address of MnS producers. This discovery is based on the MnSInfo by the criteria: mnsLabel, mnsType, mnsVersion, and mnsScope.

-	Subnetwork, ManagedFunction, ManagedElement, and EP_RP are offering the attribute supportedPerfmetricGroups to inform the MnS consumer about the PM data the MnS producer is able to provide, including the possible granularity periods and reporting methods [5].

-	MnS producer offers to create instances of IOC PerfMetricJob [5]:

-	The MnS producer offers to use the administrave state to stop and resume an existing PerfMetricJob.

-	The MnS producer will indicate the creation or modification of a PerfMetricJob as failed in case the consumer requests attribute values that the producer is not able to handle.

-	MnS producer offers to create instances of IOC ManagementDataCollection [5] and create/stop/listMeasurementJob [5].

Any realistic management system of a mobile network is composed of many management functions (including the management functions implemented at the NF), representing the mobile network with different level of abstraction and working at different scope in time, e.g. as depicted in clauses 4.5 and figure A.6.1 of [6]. These management functions are able to exchange PM data between each other: Each management function is able to:

-	Receive management data from MnS producers

-	Perform internally e.g. aggregation, filtering, KPI calculation, storing.

-	Providing management data to other MnS consumers.

As a consequence, management data representing the same counter (or KPI) related to the same network function and same point of time might be available at different management functions (otherwise the interface at definitions are not needed). In turn, all these management functions might offer these management data to other MnS consumers again.

In an exemplified, yet in the realistic overall management system the following situation may be possible:

-	The MnS producers implemented at the network elements are able to provide actual counters with granularity period 15 min in real-time, i.e. as soon as a granularity period is over, the MnS producer at the network element sends a specific management data to its MnS consumer(s).

-	The corresponding element/domain management systems might be able to provide the same management data for the same objects in real time with a granularity period of 15 min.

-	Further, the element/domain management systems might be able to store a two week history of the same counter.

-	A central PM data warehouse might be able to provide a history of the same counters:

-	with a granularity period of 15 min for the last year and with a re-calculated granularity period of 1 h, 6 h, and 12 h for the last 2 years.

-	aggregated across all cells of a base station or aggregated across all cells of a set of base stations, with granularity periods 15 min 1 h, 6 h, and 12 h for the last 2 years.

-	A central MDA function might offer predictions of the same counter for one week in the future, every 15 min.

### 4.4.2	Observations

The mechanism offered by MnS Registry [5] does not offer to find MnS that provide specific management data.

To find MnS producers that offer the production of specific management data the potential MnS consumer needs to visit all MnS offered by the MnS Registry and that cover the scope in order to query all relevant objects for their supportedPerfmetricGroups [5]. This cumbersome pull mechanism only works to control the collection of management data, i.e. to control how network elements are collecting and reporting data close to real time and to transfer these data through the 3GPP management system.

The current mechanisms are not able to handle situations where data for e.g. the same counter is available at multiple MnS producers, while the different MnS producers offer different scopes of times like real-time values (i.e. as soon as measured in the network), historic values, or predicted values for some future point of time.

Neither is a MnS producer able to register its capabilities to produce a specific management data for a specific scope of time, nor is a MnS consumer able to discover a MnS producer according to the different scopes in time.

The current mechanisms of controlling MnS producers do not allow a MnS consumer to request historical management data according to specific aspects like point of time, granularity period, aggregation across elements or areas.

### 4.4.3	Problem statement

Currently the 3GPP specifications do not offer a formal, machine readable description of management data to allow producers of management data to inform consumers about the availability of such capabilities, to enable potential consumers of management data to find producers of corresponding management data, nor to enable consumer to request specific management data, especially historical management data.

The target therefore is to define a formal description of management data to cover the different aspects of management data in order to describe PM data in more detail (e.g. available granularity periods, available time in history, aggregation in time or across elements or areas) in order to allow MnS producers to register the details of the data they are able to produce, MnS consumers to find the MnS producer that offers the specific management data, and to allow the MnS consumer to request and get the management data from the MnS producer.

## 4.5	Key issue #5: Managing external management data

### 4.5.1	Description

Clause 6.4.1 of TS 28.537 [2] contains description and the requirements for managing external management data (which is the additional data not specified by 3GPP and used to enrich the management data specified by 3GPP). This functionality provides the capability to enable data consumer to discover and request the external management data to be produced and reported. This functionality is supported based on assumption that the set-up of connection of external data sources (e.g. Sensors) and 3GPP management system to allow the 3GPP management system to obtain and store the external management data. The mechanism for 3GPP management system sets up the connection with 3GPP external entities and collects the external management data is out of scope of the present document. This functionality focuses on the external management data discovery, request and deliver between MnS producer and MnS consumer.

Clause 6.4.2 of [2] is used to describe the use case for managing external management data, which is empty.

Clause 6.4.3 of [2] contains a list of requirements for managing external management data, the solutions for following requirements which need to define MnS capability to support are missing.

REQ-MDMED-CON-2: The 3GPP management system shall enable an authorized data consumer to request external management data to be reported to the requesting or another authorized data consumer.

REQ-MDMED-CON-4: The 3GPP management system shall enable an authorized data consumer to discover stored external management data.

REQ-MDMED-CON-5: The 3GPP management system shall enable an authorized data consumer to retrieve stored external management data.

### 4.5.2	Problem Statement

Problem#1: For the description for managing data in clause 6.4.1 in TS 28.537 [2], need more description and example to elaborate how the managing external management data functionality can be used.

Problem#2: The solutions for external management data discovery, request and delivery between MnS producer and MnS consumer are missing.

### 4.5.3	Potential solution

Solution for Problem#1: Improve the description in clause 6.4.1 in TS 28.537 [2], the content in clause 6.4.1 in TS 28.537 [2] can be replaced by following content (the red text is the new proposed content).

Management data which is specified by 3GPP (clause 6.1.1) can be enriched by additional data not specified by 3GPP. This so-called external management data can be produced by data sources of different nature (e.g. sensors) with different formats.

External management data can be used for example as additional input for network optimization and prediction.

The management system should be able to manage this kind of data. That means data consumers should be able to request external management data to be produced and reported. The management system should provide support for storing it. Before data consumer request the external management data, it needs to discover which type of external management data can be produced and reported by management system.

The definition of external data sources and the data formats they use is out of scope of the present document.

This functionality is supported based on the assumption that the connection between the 3GPP external entities and 3GPP management system is set up enabling the 3GPP management system to obtain and store the external management data. The mechanism for setting up the connection between the 3GPP management system and external data sources and collection of the external management data is out of scope of the present document. This functionality mainly focuses on the external management data discovery, request and deliver between MnS producer and MnS consumer. Following is an example to illustrate the framework for managing external management data.

Figure 4.5.3-1: Example to illustrate the framework for managing external management data

The target is to define generic management mechanisms for external management data discovery, request and deliver, which can scope with any kind of external data sources and data formats.

Solution for Problem#2:

TBD

## 4.6	Key Issue #6: Define capabilities for Management Data Reporting

### 4.6.1	Description

Management data can be reported using different options and methods. The choice of which options are supported can vary across different MnS Producers based on implementation choices and reporting scope.

### 4.6.2	Problem Statement

An MnS Producer reporting management data has implementation choices with respect to supported formats, protocols and schema are applicable for different management data. The details of which options are available for which management data being reported are defined in several different TS.

At present there is no standard way to report which option(s) are supported by a particular MnS Producer for reporting of management data.

Currently, the MnS Consumers relies on documentation and/or trial-and-error to determine which options are currently supported.

### 4.6.3	Potential Solutions

The proposed solution is to add a new IOC to NRM [5] to report the supported Data Management Reporting capabilities.

#### 4.6.3.1	Add SupportedDataReporting to report supported capabilities

This solution proposes a new IOC SupportedDataReporting to allow MnS Producer to report its supported capabilities. An MnS Consumer can query the attributes of this IOC to determine which methods and options are available for management data reporting.

The new IOC is proposed to be supported at SubNetwork and ManagedElement.

A proposed definition of the new IOC follows.

##### 4.6.3.1.1	SupportedDataReporting <<IOC>>

4.6.3.1.1.1	Definition

SupportedDataReporting represents the management data reporting capabilities of an MnS producer.

The reportingTypes attribute defines which reporting methods are supported by the MnSProducer.

The fileTransferProtocols attribute identifies the supported file transfer protocols. If reportingTypes includes 'FILE' a value may be provided for the supported protocols.

The streamingProtocols attribute identifies the supported streaming protocols. If reportingTypes includes 'STREAM' a value may be provided for the supported protocols.

The timestampFormat attribute indicates the supported time format.

The pmSchema attribute identifies the supported PM schema formats.

4.6.3.1.1.2	Attributes

The SupportedDataReporting IOC includes attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportingTypes | M | T | F | F | T |
| fileTransferProtocols | M | T | F | F | T |
| streamingProtocols | M | T | F | F | T |
| timestampFormat | M | T | F | F | T |
| pmSchema | M | T | F | F | T |

4.6.3.1.1.3	Attribute constraints

None

4.6.3.1.1.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC.

4.6.3.1.1.5	Attribute Definitions

| reportingType | List of supported reporting methods. AllowedValues: - FILE - STREAM | type: ENUM multiplicity: 1..* isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |
| --- | --- | --- |
| fileTransferProtocol | List of supported file transfer protocols. AllowedValues: - FTPES - SFTP - HTTPS | type: ENUM multiplicity: 0..* isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |
| streamingProtocol | List of streaming protocols. AllowedValues: - WEBSOCKET - TCP_IP | type: ENUM multiplicity: 0..* isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |
| timestampFormat | List of timestamp reporting options. AllowedValues: - UTC - UTC_plus_offset - LOCAL | type: ENUM multiplicity: 1 isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |
| pmSchema | List of supported PM schema. Value is tuple encoded as enum: <schema format, TS, technology> AllowedValues: - XML_32432_4G - XML_32432_5G - XML_28532_5G - GPB_28550_5G - ASN1_28550_5G | type: ENUM multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

#### 4.6.3.2	Add reporting capabilities to MnSAgent

The capabilities of a MnS producer to support reporting of management data by various protocols and formats are properties of the MnS producer, i.e. these are properties of the management system as such and might even depend on the solution set. For the sake of clear separation of concerns, artifacts concerning the management system as such should be clearly separated from artifacts concerning the managed network, i.e. the "management payload".

To model and control properties of the MnS producer SA5 has introduced the IOC MnsAgent [5]. The use of MnsAgent to model properties of the MnS producer results in a clear separation between:

-	As far as useful, properties of the management system as such be contained below MnsAgent, but not to be contained by SubNetwork.

-	Properties of the managed network are not to be contained below MnsAgent.

This solution proposes the same new IOC SupportedDataReporting as of the potential solution 4.6.3.1 to allow MnS producer to report its supported capabilities. However, the IOC SupportedDataReporting is proposed to be name-contained by MnsAgent.Note: This potential solution depends on a clear definition of the semantics of MnSAgent, how instances of MnSAgent relate to the properties of the management system, and how a MnS consumer is able to address individual instances of MnSAgent.

## 4.7	Key Issue #7: Solutions for discovery of management data

### 4.7.1	Description

Key issue #2 describes a solution that provide mechanisms for an authorized management data consumer to discover stored (historical) management data needs to be specified.

Key issue#4 describes a formal description of management data to cover the different aspects of management data in order to describe PM data in more detail (e.g. available granularity periods, available time in history, aggregation in time or across elements or areas) in order to allow MnS consumers to find the MnS producer that offers the specific management data

Key issue#6 describes a standard way to report which option(s) are supported by a particular MnS Producer for reporting of management data needs to be specified. This solution only focuses on reporting methods.

Attribute "supportedPerfMetricGroups" and "supportedTraceMetrics" of SubNetwork IOC in TS 28.622 [5] described a set of performance metric groups or List of trace metrics can be supported by MnS producer.

NOTE: 	The term "management data" relates to performance measurements, key performance indicators and Trace/MDT data.

### 4.7.2	Problem Statement

A unified solution for discovery of management data needs to be considered, which can enable MnS consumer to discover the MnS producer who provides the specific data management capability from MnS Registry.

Data management capability describes following aspects for a set of management data:

-	Supported management data, which can be presented by name of the management data of category of management data.

-	Supported granularityPeriods and reportingPeriods.

-	Whether supported historical data retrieve and the period for historical data retrieve (e.g. one day, one week) will be supported.

-	Supported reporting methods (e.g. file reporting or streaming reporting).

-	Supported data request MnS, it describes the associated MnS instances which can be used to request this set of management data.

-	Supported data reporting MnS, it describes the associated MnS instances which can be used to report this set of management data.

### 4.7.3	Potential solutions

#### 4.7.3.1	Potential solution#1 Enhance the MnS Registry NRM fragment to support discovery of management data

Introduce MgmtDataInfo <<IOC>> to describe the management capabilities for a set of management data, which can be used for management data discovery. The MgmtDataInfo <<IOC>> is name-contained by MnSRegistry <<IOC>> and associated to MnSInfo <<IOC>>.

Figure 4.7.3.1-1: Enhanced UML diagram to support management data discovery

The MgmtDataInfo IOC includes attributes inherited from Top IOC and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| managementData | M | T | F | F | T |
| supportedGranularityPeriods | M | T | F | F | T |
| supportedReportingPeriods | M | T | F | F | T |
| historicalDataPeriod | M | T | F | F | T |
| supportedReportingMethod | M | T | F | F | T |
| supportedDataScope | M | T | F | F | T |
| supportedDataRequestMnSs | M | T | F | F | T |
| supportedDataReportingMnSs | M | T | F | F | T |

Attribute definitions:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| managementData | This attribute defines the list of management data that can be supported. The management data is a choice between: - a list of data categories (attribute mgtDataCategory) - a list of management data identified with their name (attribute "mgtDataName"). See clause 4.3.50 in TS 28.622 | Type: ManagementData multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedGranularityPeriods | Granularity periods supported for the production of associated management data. The period is defined in seconds. | Type: integer multiplicity: * isOrdered: False isUnique: TRUE defaultValue: None isNullable: False |
| supportedReportingPeriods | Reporting periods supported for the associated management data. The period is defined in seconds. | Type: integer multiplicity: * isOrdered: False isUnique: TRUE defaultValue: None isNullable: False |
| historicalDataPeriod | This attribute describes the maximum period of the requested historical data. When the value of this attribute is NULL, which means the capability of querying historical data is not supported. | Type: String multiplicity: 1 isOrdered: False isUnique: TRUE defaultValue: None isNullable: TRUE |
| supoprtedReportingMethod | List of supported reporting methods for the associated management data. AllowedValues: - FILE - STREAM | type: ENUM multiplicity: 1..* isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |
| supportedDataScope | List of supported sub counter capabilities for the associated management data Allowed Values: - SNSSAI - 5QI - PLMN | type: ENUM multiplicity: 1..* isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |
| supportedDataRequestMnSs | List of DN of MnSInfo for the MnS instance(s) which can be used to request the associated management data | type: DN multiplicity: 1..* isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |
| supportedDataReportingMnSs | List of DN of MnSInfo for the MnS instance(s) which can be used to report the associated management data | type: DN multiplicity: 1..* isOrdered: n/a isUnique: n/a defaultValue: None isNullable: False |

## 4.8	Key Issue #8: Key Issue and potential solution for reporting data for managementDataCollection

### 4.8.1	Description

The ManagementDataCollection IOC is introduced to enable MnS consumer to request management data (including performance measurements, KPI and Trace/MDT data) in a simple way. One of the typical scenarios is MnS consumer specifies the area of interest (including geographical area, list of cells) and management data category ("COVERAGE", "CAPACITY", "MOBILITY", "ENERGY_EFFICIENCY", "ACCESSIBILITY"). This scenario mainly used for MnS consumers who are unaware of concrete managed objects (e.g. NRCellCU, NRCellDU). However, current reporting data still based on managed objects. So, it is important to investigate the reporting data which is not relying on the concrete managed objects.

### 4.8.2	Potential solution

#### 4.8.2.1	Potential solution#1 Area based KPI

Area based KPI can be one potential solution for reporting data which is not relying on the concrete managed objects. Following are examples for area based KPIs for different management data categories for NG-RAN:

-	Examples of area based KPI for Coverage:

-	weakRSRPRatio, this KPI describes the downlink weak coverage ratio for the associated areas. The numerator is the number of the cells with downlink weak RSRP, and the denominator is the total number of cells in the specified area.

-	Examples of area based KPI for Capacity:

-	highDlPrbLoadRatio, this KPI describes the high DL PRB load ratio for the associated areas. The numerator is the number of the cells with high DL PRB load, and the denominator is the total number of cells in the specified area.

-	aveDlPrbLoad, the KPI describes average downlink PRB load (i.e. DL Total PRB Usage in TS 28.552 to represent the percentage of UL PRBs used) of the cells in the specified area.

-	Examples of area based KPI for Integrity:

-	lowDLRANUEThptRatio, this KPI describes the low DL RAN UE throughput ratio for the associated areas. The numerator is the number of the cells with low DL RAN UE throughput, and the denominator is the total number of cells in the specified area.

-	aveDLRANUEThpt, the KPI describes the average DL RAN UE throughput of the cells in the specified area.

MnS consumer specifies the list of names or categories of the area based KPI in attribute "managementData" and list of interested areas in attribute "AreaOfInterest" in ManagementDataCollection IOC. Based on MnS consumer's request, MnS producer reports the area based KPIs for corresponding sub-areas.

Whether there is any KPI defined in TS 28.554 can be reused as area based KPI needs to be further investigated.

# 5	Recommendations and conclusions

The present technical report studies 8 key issues relevant to management data, specifically addressing issues related to discovery and reporting of management data, retrieval of stored management data, and managing external management data. These key issues are documented in Clause 4 and recommendations regarding normative work for each of them are listed here:

-	Key Issue #1: Retrieval of stored management data.

There is one potential solution provided in Clause 4.1.3 and it is recommended that the normative work is done as outlined in the potential solution "use of managementDataCollection IOC for retrieval of stored management data."

-	Key Issue #2: Discovery of stored management data.

There are 4 potential solutions provided in Clause 4.2.3, among which enabling potential solutions "Discover management data using try and error" (Clause 4.2.3.1) and "Discover management data using one central MnsRegistry" (Clause 4.2.3.2) does not need any specification work. It is understood that for these options no new data is required in the MnS registry.

It is recommended that the normative solution to address this key issue consider the potential solutions "Discover management data using hierarchical MnsRegistries" (Clause 4.2.3.3) and "Discovery of stored management data producer supporting retrieval of stored data" (Clause 4.2.3.4). Further evaluation of these two potential solutions is required.

-	Key Issue #3: Consolidated reporting of management data.

There is one potential solution outlined in Clause 4.3.3.1 proposing to "Define how ManagementDataCollection consolidates the reporting data," which proposes as one option to create a consolidated output data file by combining the files into a single output such as a zip file; It is recommended to base the normative work on this proposed solution, i.e., specification of a harmonized container format for the files (e.g., zip, tar) and a corresponding naming scheme.

-	Key Issue #4: Describe management data in more detail.

There is no potential solution documented for this key issue and it is recommended not to pursue this key issue for normative work.

-	Key Issue #5: Managing external management data.

There are two problems identified to be addressed for this key issue:

-	Problem#1 is lack of sufficient description in TS 28.537 [2] about management of external management data and there is a solution provided for that under Solution for Problem #1 in Clause 4.5.3. It is recommended that this solution is used as the basis of the normative work addressing problem #1.

-	Problem #2 is lack of solutions for discovery, request, and delivery of external management data. There is no potential solution documented for this problem in the present document. It is recommended that as part of normative work, existing solutions for discovery, request, and delivery of management data are evaluated for applicability to external management data; which data is available on other interfaces outside the 3GPP management system, and whether to expose it requires further discussion. If the existing solutions apply to external management data, the related description will be included in the scope of the normative work; otherwise, a follow-up study of this key issue is recommended.

-	Key Issue #6: Define capabilities for management data reporting.

There are two potential solutions provided for this key issue, "Add SupportedDataReporting to report supported capabilities" (Clause 4.6.3.1) and "Add reporting capabilities to MnSAgent" (Clause 4.6.3.2). Both of these solutions have definition of a new IOC SupportedDataReporting (Clause 4.6.3.1.1) in common, however differ on the entities which support the IOC. It is recommended that the normative work includes the definition of the new IOC, and further evaluates the two solutions to determine which entity(ies) to support this new IOC.

-	Key Issue #7: Solutions for discovery of management data

There is one potential solution provided for this key issue "Enhance the MnS Registry NRM fragment to support discovery of management data" (Clause 4.7.3.1) and it is recommended that normative work is done based on this solution.  It is recommended to avoid the duplication of data which is already available in the NRM in the normative solution.

-	Key Issue #8: Reporting data for managementDataCollection

There is one potential solution "Area based KPI" (Clause 4.8.2.1) provided for this key issue. It is recommended to enhance the existing reporting method (including streaming reporting and file reporting) to support reporting area based KPI reporting, define the new area based KPIs, and identify existing KPIs defined in TS 28.554 [8] which can be reused as area based KPI.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-11 | SA5#146 | S5-226627 |  |  |  | Initial skeleton | 0.0.0 |
| 2022-11 | SA5#146 | S5-226628 |  |  |  | Rel-18 pCR TR 28.842 Add skeleton | 0.1.0 |
| 2023-03 | SA5#147 | S5-232874 |  |  |  | pCR TR28.842 New KI on storage of management data | 0.2.0 |
| 2023-03 | SA5#147 | S5-232872 |  |  |  | pCR TR28.842 New KI on discovery of stored management data | 0.2.0 |
| 2023-04 | SA5#148e | S5-233611 S5-233612 S5-233535 |  |  |  | Add key issue on enhancing the description of management data Rel-18 pCR TR 28.842 Add key issue for reporting of consolidated management data pCR TR 28.842 Correction -Adding missing reference | 0.3.0 |
| 2023-05 | SA5#149 | S5-234558 |  |  |  | pCR TR 28.842 add issue and potential solutions for external data management | 0.4.0 |
| 2023-08 | SA5#150 | S5-235411 |  |  |  | pCR 28.842 Add issue to Define Management Data Reporting Capabilities | 0.5.0 |
| 2023-10 | SA5#151 | S5-237181 |  |  |  | pCR 28.842 Potential solution for retrieval of stored management data | 0.6.0 |
| 2023-11 | SA5#152 | S5-237470 S5-237935 S5-237944 S5-237945 S5-238266 S5-238274 |  |  |  | pCR TR 28.842 Add potential solution for discovery of management data Rel-18 pCR TR 28.842 Potential solution for discovery of stored data pCR 28.842 Potential solution for retrieval of stored management data pCR TR 28.842 Potential solution for discovery of stored management data pCR TR 28.842 Add Key Issue and potential solution for reporting data for managementDataCollection Rel-18 pCR TR 28.842 Potential solution to report supported capabilities | 0.7.0 |
| 2024-02 | SA5#153 | S5-240642 S5-240936 |  |  |  | pCR TR 28.842 Add scope pCR TR 28.842 Add recommendations and conclusions | 0.8.0 |
| 2024-03 | SA#103 | SP-240264 |  |  |  | Draft after editHelp review and sent to SA plenary for information and approval | 1.0.0 |
| 2024-03 | SA#103 |  |  |  |  | Upgrade to change control version | 18.0.0 |
| 2024-06 | SA#103 | SP-240810 | 0001 | - | F | Rel-18 CR TS 28.842 Rapporteur clean up | 18.1.0 |