3GPP TSG-SA5 Meeting #157	S5-245632

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.317 | CR | 0011 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.317 Enhance the isUnique property for stage 3 OpenAPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In cluse 6.1.11.3 TS 32.160, there is the specification for “isUnique” for OpenAPI. But not all generic NRM OpenAPI implementation is aligned with the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the OpenAPI stage 3 to align with clause 6.1.11.3 in TS32.160. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifciation may lead to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (Normative Stage 3 is in Forge) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1422 at commit eeb8191a66a0966f885f0589f532350e17b318b9 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1422 at commit eeb8191a66a0966f885f0589f532350e17b318b9

*** START OF CHANGE 1 ***

*** OpenAPI/TS28317_RanScNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: RANSC NRM

version: 18.1.0

description: >-

OAS 3.0.1 definition of the RANSC NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.317; Self-configuration of Radio Access Network Entities (RAN NEs)

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.317/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-RanScNrm:

type: object

properties:

ScMgmtProfile:

$ref: '#/components/schemas/ScMgmtProfile-Multiple'

Sc_Process:

$ref: '#/components/schemas/Sc_Process-Multiple'

#-------Definition of IOCs ----------#

ScMgmtProfile-Single:

description: >-

The ScMgmtProfile represents MnS Consumer's requirements for self-configuration management

for a set of RAN NEs or RAN NE types.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

nEInformation:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NEInfomration'

description: >-

This attribute defines the NE Type(s) or the NE instance(s) for which this

ScMgmtProfile instance is valid.

configDataFileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

Sc_Process-Single:

description: >-

This IOC represents the self-configuration process for a RAN NE, which allows the MnS

Consumer to be informed about the current situation of the Self Configuration process.

When the automated management process for an RAN NE starts, an instance of the Sc_Process

is created automatically by the MnS Producer and informed to MnS consumer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

nEIdentification:

$ref: '#/components/schemas/NEIdentification'

scProcessMonitor:

$ref: '#/components/schemas/ScProcessMonitor'

cancelScProcess:

type: string

enum:

- TRUE

- FALSE

description: >-

Setting this attribute to "TRUE" cancels the self configuration process.

scMgmtProfileRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------Definition of IOCs ----------#

#-------Definition of Data types ----------#

NEType:

type: string

description: It defines the type of RAN NE

GnbId:

type: integer

minimum: 0

maximum: 4294967295

description: It is the identity of gNB

EnbId:

type: string

minimum: 0

maximum: 268435455

description: It is the identity of ng-eNB

NEIdentification:

oneOf:

- $ref: '#/components/schemas/EnbId'

- $ref: '#/components/schemas/GnbId'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

NEInfomration:

description: >-

It defines the NE Type or the NE instance for which this

ScMgmtProfile instance is valid.

anyOf:

- $ref: '#/components/schemas/NEType'

- $ref: '#/components/schemas/NEIdentification'

ScProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type with specialisations

for usage in the RANSC management for monitoring for the self configuration process.

type: object

properties:

jobId:

type: string

status:

type: string

enum:

- NOT_STARTED

- RUNNING

- FINSHED

- FAILED

- CANCELLING

- CANCELLED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

enum:

- NE_HEALTH_CHECK

- SW_DOWNLOAD

- SW_INSTALLATION

- SW_ACTIVATION

- PREPARE_BASIC_CONFIGURATION_AND_OAMLINK

- RETRIEVE_CONFIGURATION_DATA

- SETUP_PRECONFIGURED_SIGNALLING_LINKS

- SET_FINAL_STATE_OF_NE

resultStateInfo:

oneOf:

- type: string

enum:

- UNKNOWN

- INCORRECT_CONFIGURATION

- NE_HARDWARE_ERROR_DELECTED

- DISCONNECTION_BETWEEN_NE_AND_OAM

- OTHER

- type: string

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

#-------Definition of Data types ----------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

ScMgmtProfile-Multiple:

type: array

items:

$ref: '#/components/schemas/ScMgmtProfile-Single'

Sc_Process-Multiple:

type: array

items:

$ref: '#/components/schemas/Sc_Process-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.317 for TS 28.532 --------------------------#

resources-RanScNrm:

oneOf:

- $ref: '#/components/schemas/ScMgmtProfile-Single'

- $ref: '#/components/schemas/Sc_Process-Single'

#----- Definitions in TS 28.317 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 1 ***

End of change