3GPP TSG-SA5 Meeting #157	S5-245355

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei, Deutsche Telekom

# Title:	Discussion on observations and potential enhancement for management data discovery

# Document for:	Endorsement

Agenda Item:	6.19.6

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP TS 28.537: "Management and orchestration; Management capabilities"

[2]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

3	Rationale

3.1 Background and observations

3.1.1 Management data recovery description and requirements in TS 28.537

The following requirements for management data recovery is documented in TS 28.537 [1], which applies for performance metrics and Trace/MDT data

REQ-DMSDIS-CON-1: The 3GPP management system shall enable an authorized data consumer to discover what management data can be produced by 3GPP management system.

3.1.2 Management data recovery solutions in TS 28.622

The following SupportedPerfMetricGroup <<dataType>> is introduced as an attribute of SubNetwork IOC,  ManagedElement IOC and ManagedFunction IOC to represent a group of supported performance metrics, and associated (production and monitoring) granularity periods and reporting methods that are supported for the specified performance metric group. The Performance metrics include measurements and KPIs.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceMetrics | M | T | F | F | T |
| granularityPeriods | M | T | F | F | T |
| reportingMethods | M | T | F | F | T |
| reportingPeriods | M | T | F | F | T |

The following supportedTraceMetrics is introduced as an attribute of SubNetwork IOC, ManagedElement IOC and ManagedFunction IOC to represent the supported trace metrics. Trace metrics include trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RLF and RCEF reports.

| supportedTraceMetrics | List of trace metrics. When this attribute is contained in a managed object it defines the trace metrics supported for this object and all descendant objects. Trace metrics include trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RLF and RCEF reports, see TS 32.422 [30]. Trace metrics are identified with their metric identifier. The metric identifier is constructed as defined in clause 10 of TS 32.422 [30]. For non-3GPP specified trace metrics the name is defined elsewhere. allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| --- | --- | --- |

4.3.7	SubNetwork

The SubNetwork IOC includes the attributes inherited from Domain_ IOC (defined in TS 28.620 [9]), attributes inherited from TopX IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setOfMcc | CM | T | F | F | T |
| priorityLabel | O | T | T | F | T |
| supportedPerfMetricGroups | O | T | F | F | T |
| supportedTraceMetrics | O | T | F | F | T |

4.3.3	ManagedElement

The ManagedElement IOC includes the attributes inherited from ManagedElement_ IOC (defined in TS 28.620 [9]), attributes inherited from TopX IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vendorName | M | T | F | F | T |
| userDefinedState | M | T | T | F | T |
| swVersion | M | T | F | F | T |
| priorityLabel | O | T | T | F | T |
| supportedPerfMetricGroups | O | T | F | F | T |
| supportedTraceMetrics | O | T | F | F | T |

4.3.4	ManagedFunction

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vnfParametersList | CM | T | T | F | T |
| peeParametersList | CM | T | T | F | T |
| priorityLabel | O | T | T | F | T |
| supportedPerfMetricGroups | O | T | F | F | T |
| supportedTraceMetrics | O | T | F | F | T |

3.1.3 SupportedNotifications in TS 28.622

The following SupportedNotifications IOC is defined in TS 28.622 [2] to represent the notification related capabilities of a MnS producer.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationTypes | M | T | F | F | T |
| notificationProtocols | M | T | F | F | T |

Based on above background information in clause 3.1.1, 3.1.2 and 3.1.3, following are the observations:

Observation#1: Compared to SupportedPerfMetricGroup, the associated reporting methods that are supported for the specified trace metrics is missing in the modelling for supportedTraceMetrics.

Observation#2: Current REQ-DMSDIS-CON-1 only focus on what management data can be discovered, the capability for the supported reporting methods for corresponding supported management is missing.

Observation#3: The supported management data is modelled as dataType, which supported notifications is modelled as IOC, which is not aligned. Also supportedPerfMetricGroups and supportedTraceMetrics individually are modelled in SubNetwork IOC, ManagedElement IOC and ManagedFunction IOC.

3.1.4 managementData in ManagementDataCollection in TS 28.622

The following ManagementData <<choice>> is defined as an attribute of ManagementDataCollection IOC to represent a list of data categories or a list of management data identified with their name required by MnS consumer.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 mgtDataCategory | M | T | T | T | N/A |
| CHOICE_2.1 mgtDataName | M | T | T | T | N/A |

Observation#4: In case MnS consumer request the management data by specify the management data category, the MnS consumer needs to know what categories of management data can be produced and reported by MnS producer. The requirement and corresponding solution for such capability is missing.

3.2 Solutions

To addressed the issues identified in above observations, following are the proposed solutions to update Management data recovery capability in TS 28.537 and TS 28.622.

Proposal#1: Update TS 28.537 clause 6.5	Discovery of management data to add following two requirements:

REQ-DMSDIS-CON-2: The 3GPP management system shall enable an authorized data consumer to discover what category of management data can be produced by 3GPP management system.

REQ-DMSDIS-CON-3: The 3GPP management system shall enable an authorized data consumer to obtain the reporting methods that are supported for the management data which can be produced by 3GPP management system.

Proposal#2: Add solution to support management data discovery with updating TS 28.622 with following aspects:

- Proposal#2.1 Add a new supportedManagementData IOC to represent MnS producer’s capabilities for management data production and reporting. The new supportedManagementData IOC is name contained by SubNetwork IOC,  ManagedElement IOC and ManagedFunction IOC.

- Proposal#2.2 Move the attribute supportedPerfMetricGroups and supportedTraceMetrics from SubNetwork IOC,  ManagedElement IOC and ManagedFunction IOC to this new supportedManagementData IOC.

- Proposal#2.3 The new supportedManaggementData IOC needs to describe what categories of management data can be produced and reported by MnS producer.

- Proposal#2.4 The new supportedManaggementData IOC needs to describe the associated reporting methods that are supported for the specified trace metrics

Following are the detailed UML diagram and attributes SupportedManagemenetData IOC

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedTraceMetrics | CM | T | F | F | T |
| supportedPerformanceMetrics | CM | T | F | F | T |
| supportedmgtDataCategory | CM | T | F | F | T |
| supportedGranularityPeriods | CM | T | F | F | T |
| supportedReportingMethods | M | T | F | F | T |
| supportedReportingPeriods | CM | T | F | F | T |

4	Detailed proposal

Following are the detailed proposals to update Management data recovery capability in TS 28.537 and TS 28.622:

Proposal#1: Add requirements to support management data discovery capability with updating TS 28.537 clause to add following two requirements:

REQ-DMSDIS-CON-2: The 3GPP management system shall enable an authorized data consumer to discover what category of management data can be produced by 3GPP management system.

REQ-DMSDIS-CON-3: The 3GPP management system shall enable an authorized data consumer to obtain the reporting methods that are supported for the management data which can be produced by 3GPP management system.

Proposal#2: Update TS 28.622 with following aspects:

- Proposal#2.1 Add a new supportedManagementData IOC to represent MnS producer’s capabilities for management data production and reporting. The new supportedManagementData IOC is name contained by SubNetwork IOC, ManagedElement IOC and ManagedFunction IOC.

- Proposal#2.2 Move the attribute supportedPerfMetricGroups and supportedTraceMetrics from SubNetwork IOC, ManagedElement IOC and ManagedFunction IOC to this new supportedManagementData IOC.

- Proposal#2.3 The new supportedManaggementData IOC needs to describe what categories of management data can be produced and reported by MnS producer.

- Proposal#2.4 The new supportedManagementData IOC needs to describe the associated reporting methods that are supported for the specified trace metrics