3GPP TSG-SA5 Meeting #157 	 S5-245548

Hyderabad, Korea, 14 October - 18 October 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0610 | rev | x | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New performance measurements for Reduced Capability (RedCap) Device Early Identification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-14-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently there is no performance measure defined in TS 28.552 quantifying the number of RACH attempts triggered by RedCap devices. In this CR, it is proposed to define the Number of RedCap PRACH Attempts as well as The Number of RedCap RACH Msg3 Attempts. One usage of this performance measurements is to help identiy where RedCap early identificaiton through RACH procedure happens. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new performance measures capturing the number of RedCap PRACH attempts as well as the number of RedCap RACH Msg3 attempts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without these performance measurements, appriopriate per site uplink initial access capacity planning and dimensioning for base stations that serve many RedCap devices is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.52 (new), 5.1.1.52.1 (new), 5.1.1.52.2 (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.1.1.52	Reduced Capabiltiy Device Early Identification

# >>>>NEXT CHANGE<<<<

##### 5.1.1.52.1	Number of RedCap PRACH Attempts

a)	This measurement provides the number of PRACHs received via RedCap BWP configured on a given RedCap capable DU.

b)	CC.

c)	This measurement is obtained by counting the number of PRACH preambles received on a DU that supports an UL RedCap dedicated BWP (a DU that broadcasts the IE initialUplinkBWP-RedCap-r17 in SIB1, see clause 5.2.2.4.2 of TS 38.331 [20])

d)	A single integer value.

e)	RACH.PreambleRedCap.

f)	NRCellDU.

g)	Valid for packet switching.

h)	5GS.

i)	One usage of this measurement is for performance assurance to support RACH optimization for appriopriate per site uplink initial access capacity planning and dimensioning for base stations that serve many RedCap devices (see TS 28.313 [30]).

# >>>>NEXT CHANGE<<<<

##### 5.1.1.52.2	Number of RedCap RACH Msg3 Attempts

a)	This measurement provides the number of UE RACH Msg3 PUSCH messages using logical channel ID (LCID) 35 or 36 and received by a RedCap capable DU.

b)	CC.

c)	This measurement is obtained by counting the number of PRACH preambles received on a DU that support serving RedCap capable UEs (a DU that broadcasts the IE cellBarredRedCap1Rx-r17 and/or cellBarredRedCap2Rx-r17 set to notBarred in SIB1, see clause 5.2.2.4.2 of TS 38.331 [20])

d)	A single integer value.

e)	RACH.Msg3RedCap.

f)	NRCellDU.

g)	Valid for packet switching.

h)	5GS.

i)	One usage of this measurement is for performance assurance to support RACH optimization for appriopriate per site uplink initial access capacity planning and dimensioning for base stations that serve many RedCap devices (see TS 28.313 [30]).

# >>>>NEXT CHANGE<<<<

A.X Use case for the Number of RedCap PRACH Attempts and the Number of RedCap RACH Msg3 Attempts.

The number of RedCap PRACH attempts, and the number of RedCap RACH Msg3 Attempts are useful to help identiy where cases where RedCap early identificaiton through RACH procedure happens as well as to allow operators to make informed appriopriate per site uplink initial access capacity planning and dimensioning for base stations that serve many RedCap devices.

# >>>>END OF CHANGES<<<<