3GPP TSG-SA5 Meeting #157	S5-245663

# Hyderabad, India, 14 - 18 October 2024

# Source:	Samsung, AT&T

# Title:	Managing ML models in use in a live network

# Document for:	Approval

Agenda Item:	6.19.1

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

None

# 3	Rationale

This provides the solution for the UC “Managing ML models in use in a live network”. The solution requires providing the additional information in the ML Inference Report. This information will specify the potential network impacts due to the inference output result. This information can then enable an authorized consumer to a) take an informed decision about the inference output result b) identify the ML model that is causing a particular non-optimal performance in the network at some future point of time. The consumer can then decide to either deactivate the inference or update the inference function properties to mitigate the performance degradation.

The basic assumption here is that the recommendation provided as part of inference output result (i.e provided MDA recommendations) may cause some sub-optimal network conditions. The following table shows example of some of the configuration recommendations that various MDA analytics provides and what PM data it may effect.

| InferenceName/MDT Type (28.104) | Recommended Configurations | Potential PM Impact |
| --- | --- | --- |
| CoverageAnalytics.CoverageProblemAnalysis. | Creation of new beam(s), or cell(s); Change the transmission power of the NR sector carrier; Delete some unwanted beam(s) or cell(s). | Energy Consumption may increase. |
| MDAAssistedFaultManagement.FailurePrediction. | Update 5GC NF (e.g., AMF and SMF) profile | Updates to servingScope may result in coverage hole. |
| ResourceAnalytics.virtualizedResourceUtilizationAnalysisNF | scale in a list of NFs; scale out a list of NFs. | In case of scale out Energy Consumption may increase. In case of scale in, considering the traffic projections, the throughput may decrease. |
| ResourceAnalytics.PhyiscalResourceUtilizationAnalysisNF | optimising the capacity of gNB (e.g., increasing or decreasing physical resources | In case of increasing resources, Energy Consumption may increase. |
| ResourceAnalytics.5GCControlPlaneCongestionAnalysis | scale out a list of 5GC NFs | Energy Consumption may increase. |
| MDAAssistedEnergySaving.EnergySavingAnalysis | For ES on NR cells. It may contain a set of: - Recommended NR Cell (ES-Cell) to enter energySaving state. - Recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - The time to enter and terminate the energy saving state. - The load threshold to enter and terminate the energy saving state for the ES-Cell. For ES on UPFs. It contains a set of: - Recommended UPF (ES-UPF) to conduct energy saving. - Recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - The time to conduct energy saving for the ES-UPF. | Switching energy saving state ON my reduce Throughput and increase latency. |

# 4	Detailed proposal

First Change

### 5.5.4	Managing ML models in use in a live network

#### 5.5.4.1	Description

#### 5.5.4.2	Use Cases

##### 5.5.4.2.1	Handling of underperforming ML trained models in live networks

Actions may need to be taken by a network operator once an ML trained model has been identified that is contributing towards non-optimal running of the network. These actions may involve for example, without service interruptions, reverting to running of the network without ML based optimizations or replacing current ML model with an earlier model one that was performing better.

##### 5.5.4.2.2		Performance monitoring of Network Functions with ML trained models in live networks

Several trained ML models maybe in use in an operator network with each one of them influencing network performance. The Network Functions with these ML models need to be monitored  to ensure network is running optimally with these models in use. KPIs for evaluating runtime performance of Network Functions using ML models should be provided for this purpose as part of the model and these KPIs would need to be collectable by the network operator.

#### 5.5.4.3	Potential requirements

REQ-DATA-ACT-1: The 3GPP management system should have a capability enabling an authorized consumer to identify an ML Model that caused performance measurement and/or KPI degradation.

REQ-DATA-ACT-2: The 3GPP management system should have a capability to recommend remedial actions to address performance measurement and/or KPI degradation caused by an ML model.

REQ-DATA-PERF-1: The 3GPP management system should be able to collect, from the network, performance data pertaining to Network Functions actively utilizing ML models.

REQ-DATA-PERF-2: The 3GPP management system should support performance KPIs that can be monitored for performance assurance purpose.

#### 5.5.4.4	Possible solutions

The solution requires providing the additional information in the ML Inference Report. This information will specify the potential network impacts due to the inference output result. This information can then enable an authorized consumer to a) take an informed decision about the inference output result b) identify the ML model that is causing a particular non-optimal performance in the network at some future point of time. The consumer can then decide to either deactivate the inference or update the inference function properties to mitigate the performance degradation. The information will be added as part of InferenceReport as defined in 3GPP TS 28.105 and will modelled as a new datatype called PotentialImpactsInfo. This data type will include:

- Inference output result: This will specify the attribute value pair for each of the attribute defined in the analytics output for the particular MDA type identified by the attribute aIMLInferenceName.

- AffectedScope: This will specify the scope of affect the inference output may have. This may include

  - Identifier of the network functions that may be affected by the output result of the inference function. This will be in form of a DN.

  - A Geographical location indicating that all the network function in that location may be affected by the inference output result.

  - A time duration indicating that all the related network function may be affected during this time duration by the inference output result.

- Affected Performance Data: This will identify the potential performance data that may be affected in a non-optimal way due to the recommendations/configurations provided as part of inference output result.

  - PMIdentifier: This will identify the performance data or the KPI that may be affected. This will be the name of PM and KPI as defined in 3GPP TS 28.552 and 28.554 respectively.

  - ExpectedPMValues: This will specify the potential non-optimal value of the performance data.

Figure 1: Procedure for Managing ML models in use in a live network

- The ML model gets trained, tested and deployed as per the mechanism defined in 3GPP TS 28.105.

- Consumer send the request to activate the ML inference at the node where the ML model was deployed.

- The inference gets initiated.

- Producer send the acknowledgment.

- Producer generates the Inference report with the information on the potential network impact as defined in of the embodiment here.

- Consumer is notified about the availability of the report

- Consumer send a query request to read the inference report generated by the producer.

- Producer send the response with the information

- A performance degradation is detected by the consumer utilizing the existing performance assurance mechanism defined in 3GPP TS 28.622.

- Consumer checks the historical inference report information to identify the ML model whose inference may be causing the performance degradation based on the information on the potential network impact provided as part of inference report.

- Once the ML model is identified the consumer deactivate the ML inference.

- Producer sends the acknowledgment.

- Alternatively, consumer may choose to update the ML inference

- Producer send the acknowledgment.

#### 5.5.4.5	Evaluation

End of Change