3GPP TSG-SA5 Meeting #157	S5-245721

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0458 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.623 Clarification on Real and Float data types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - TS 28.622 clause 5.2.1 specifies simple common data types; one of these simple data types is Float. Table 5.2-1 in this clause include clarifications on how Float data type is realized into YAML and YANG. This stage-3 specifities shall be captured in the TS 28.623 instead. It is important to keep stage-2 and stage-3 in separate TS (28.622 and 28.623), as done so far. - Note that OpenAPI does not have type “real”, just type “number” (see OpenAPI Specification v3.0.0). Therefore, Float data type in YAML cannot be of type real. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture stage-3 clarifications regarding usage of Float data type into TS 28.623, moving them from TS 28.622 clause 5.2.1. - Fix description for Float data type in YAML, to make it compliant with OpenAPI specification. - Add needed references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Keep stage-2/stage-3 content for Generic NRM in separate TS is not respected. - Float data type definition in YAML is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, E.1.y (new), E.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 28.622: “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[5]	Void

[6]	Void

[7]	Void

[8]	Void

[9]	Void.

[10]	Void

[11]	Void

[12]	Void

[13]	Void

[14]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[15]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[16]	IETF RFC 8528: "YANG Schema Mount".

[17]	Management and Orchestration APIs Stage 3 Repository https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel19_SA104/

[18]	RFC 8525: "YANG Library"

[19]	RFC 6022: "YANG Module for NETCONF Monitoring"

[x]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[y]	IETF RFC 7950: “The YANG 1.1 Data Modeling Language”

[z]	IETF RFC 6991: “Common YANG Data Types”

1st Change

Annex E (normative):Solution set specific provisions and examples

# E.1	RESTful HTTP-based solution set

MnS producer specific schema definitions

The NRM properties supported by a specific MnS producer are specified by MnS producer specific versions of the standardized OpenAPI definition files. These definitions reflect exactly what is supported. The OpenAPI definitions for unsupported properties are removed.

For example, the schema of the Generic NRM as published by 3GPP allows both a "SubNetwork" of type "SubNetwork-Multiple" and a "ManagedElement" of type "ManagedElement-Multiple" as roots. A concrete MnS producer has only one root class. Therefore, the schema definitions need to be modified accordingly in the file describing the NRM properties of a concrete MnS producer.

Furthermore, the standardized definition of "SubNetwork-Single" and "ManagedElement-Single" includes all possible name-contained objects. A concrete MnS producer supports in most cases only a subset of those. Unsupported name-containments need to be removed. For example, if "SubNetwork-Single" does not support the Intent NRM, then the following line "$ref: 'TS28312_IntentNrm.yaml#/components/schemas/SubNetwork-ncO-IntentNrm" needs to be deleted.

When objects, attributes or attribute fields are unsupported, its schema definitions need to be removed.

The MnS producer specific schema of the Generic NRM is always the root schema, that includes references to child schemas in other files, that in turn may reference other schemas, and so forth. It is recommended to locate the root schema at a URI that is composed by appending the path component "/schemas" to the URI specified in "mnsAddress":

<mnsAddress>/schemas

To obtain all NRM properties supported by a MnS producer it is necessary to inspect the root schema and all its descendant schemas.

References in a file may contain a relative path or an absolute path. When a reference has a relative path, the processor shall assume that the referenced file is located at the same address as the file referencing it.

Examples:

The following example shows an excerpt of a file specifying a NRM, that supports the complete Generic NRM and the complete NR NRM. The root class is "SubNetwork".

The schema definition published by 3GPP specifies the allowed root classes.

NrmRoot: oneOf: - type: object properties: SubNetwork: $ref: '#/components/schemas/SubNetwork-Multiple' - type: object properties: ManagedElement: $ref: '#/components/schemas/ManagedElement-Multiple'

This definition needs to be modified to produce the following schema definition that allows only "SubNetwork" as root class.

NrmRoot: type: object properties: SubNetwork: $ref: '#/components/schemas/SubNetwork-Multiple'

Furthermore, all objects name-contained by "SubNetwork" and "ManagedElement" that are not defined by the NR NRM need to be removed. The excerpt of the schema published by 3GPP may look as follows.

SubNetwork-Single: allOf: - $ref: '#/components/schemas/Top' - $ref: '#/components/schemas/SubNetwork-Attr' - $ref: '#/components/schemas/SubNetwork-ncO' - $ref: 'TS28104_MdaNrm.yaml#/components/schemas/SubNetwork-ncO-MdaNrm' - $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/SubNetwork-ncO-AiMlNrm' - $ref: 'TS28312_IntentNrm.yaml#/components/schemas/SubNetwork-ncO-IntentNrm' - $ref: 'TS28317_RanScNrm.yaml#/components/schemas/SubNetwork-ncO-RanScNrm' - $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/SubNetwork-ncO-CoslaNrm' - $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/SubNetwork-ncO-EdgeNrm' - $ref: 'TS28541_SliceNrm.yaml#/components/schemas/SubNetwork-ncO-SliceNrm' - $ref: 'TS28541_NrNrm.yaml#/components/schemas/SubNetwork-ncO-NrNrm' - $ref: 'TS28541_5GcNrm.yaml#/components/schemas/SubNetwork-ncO-5GcNrm' ManagedElement-Single: allOf: - $ref: '#/components/schemas/Top' - $ref: '#/components/schemas/ManagedElement-Attr' - $ref: '#/components/schemas/ManagedElement-ncO' - $ref: 'TS28104_MdaNrm.yaml#/components/schemas/ManagedElement-ncO-MdaNrm' - $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/ManagedElement-ncO-AiMlNrm' - $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/ManagedElement-ncO-CoslaNrm' - $ref: 'TS28541_NrNrm.yaml#/components/schemas/ManagedElement-ncO-NrNrm' - $ref: 'TS28541_5GcNrm.yaml#/components/schemas/ManagedElement-ncO-5GcNrm'

The schema describing the example NRM includes only name-containments from the NR NRM.

SubNetwork-Single: allOf: - $ref: '#/components/schemas/Top' - $ref: '#/components/schemas/SubNetwork-Attr' - $ref: '#/components/schemas/SubNetwork-ncO' - $ref: 'TS28541_NrNrm.yaml#/components/schemas/SubNetwork-ncO-NrNrm' ManagedElement-Single: allOf: - $ref: '#/components/schemas/Top' - $ref: '#/components/schemas/ManagedElement-Attr' - $ref: '#/components/schemas/ManagedElement-ncO' - $ref: 'TS28541_NrNrm.yaml#/components/schemas/ManagedElement-ncO-NrNrm'

The next example demonstrates how to deal with attributes whose support qualifier is optional and that are not supported by a MnS producer. The "FileDownLoadJob" is a case that has as one optional attribute, the " notificationRecipientAddress". The schema published by 3GPP includes its attribute definition.

FileDownloadJob-Single: allOf: - $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top' - type: object properties: attributes: type: object properties: fileLocation: type: string notificationRecipientAddress: $ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri' cancelJob: type: string enum: - TRUE - FALSE jobMonitor: $ref: '#/components/schemas/FileDownloadJobProcessMonitor'

The schema describing what is supported on the MnS producer does not include it.

FileDownloadJob-Single: allOf: - $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top' - type: object properties: attributes: type: object properties: fileLocation: type: string cancelJob: type: string enum: - TRUE - FALSE jobMonitor: $ref: '#/components/schemas/FileDownloadJobProcessMonitor'

The following example shows possible URIs for accessing the MnS producer specific schema of the Generic NRM, the file that always needs to be retrieved first.

https://example.com/management/ProvMnS/1800/schemas/TS28623_GenericNrm_VS.yaml sftp://example.com/management/ProvMnS/1800/schemas/TS28623_GenericNrm_VS.yaml

NtfSubscriptionControl

The format of the value of the attribute "dataNodeSelector" shall be a Jex expression that is compliant to either the Jex basic profile specified in clause 7.4 of TS 32.161 [20]) or to the Jex advanced profile specified in clause 7.5 of TS 32.161 [20]). The value of the attribute "notificationFilter" shall be a Jex expression that is compliant to the Jex conditions profile specified in clause 7.6 of TS 32.161 [20]). The accessible data nodes of the Jex expressions are equal to the nodes in the tree starting at the parent object of the "NtfSubscriptionControl" object.

Examples:

The following example demonstrates how an "NtfSubscriptionControl" object can be used for monitoring quality of service alarm notifications from all "YxzFunction" objects under all "ManagedElement" objects in a specific "SubNetwork".

PUT /3gpp-management/SubNetwork=SN1/NtfSubscriptionControl=NSC1 HTTP/1.1 Host: example.org Content-Type: application/json { "notificationRecipientAddress": "example.org/3gpp-management/alarm-notification-sink", "notificationTypes": [ "notifyNewAlarm", "notifyChangedAlarmGeneral", "notifyClearedAlarm" ], "scope": { "dataNodeSelector": "/SubNetwork[id="SN1"]/ManagedElement/XyzFunction" }, "notificationFilter": "alarmType=\"QUALITY_OF_SERVICE_ALARM\"" }

The next example shows how the operational state and administrative state attributes of all "YxzFunction" objects under all "ManagedElement" objects in a specific "SubNetwork" can be monitored.

PUT /3gpp-management/SubNetwork[id"=SN1"]/NtfSubscriptionControl=NSC1 HTTP/1.1 Host: example.org Content-Type: application/json { "notificationRecipientAddress": "http://example.org/3gpp-management/cm-notification-sink", "notificationTypes": [ " notifyMOIChanges " ], "scope": { "dataNodeSelector": "/SubNetwork[id="SN"1/ManagedElement/XyzFunction/attributes\ (operationalState ¦ administrativeState)" } }

ConditionMonitor

The value of the attribute "conditions" shall be a Jex expression that is compliant to the Jex conditions profile specified in clause 7.6 of TS 32.161 [20]). The accessible data nodes of the Jex expressions are equal to the nodes in the tree starting at the parent object of the "ConditionMonitor" object.

Examples:

The following example demonstrates how the "ConditionMonitor" can be used for monitoring alarm lists. The condition below evaluates to true, when an alarm is raised on the object instance identified by "DN1".

"condition": \ "/SubNetwork[id="SN1"]/AlarmList[id="AL1"]/attributes/alarmRecords/*/objectInstance="DN1"

The occurrence of this condition may for example switch on a "PerfMetricJob" to start collecting performance metrics on the alarmed object instance. To do so the "conditionMonitorRef" attribute of the "PerfMetricJob" must specify the DN of the "ConditionMonitor".

In the next example the condition in the example above is modified to include the status of a "Scheduler". The modified condition evaluates to true, when an alarm is raised on the object instance identified by "DN1", but only in the time periods specified in the "Scheduler".

"condition": \ "/SubNetwork[id="SN1"]/AlarmList[id="AL1"]/attributes/alarmRecords/*/objectInstance="DN1\ and /SubNetwork[id="SN1"]/Scheduler[id="S1"]/attributes/schedulerStatus=true"

### E.1.y	Common data types

Table E.1.y-1 includes clarifications on the realization of some data types.

Table E.1.y-1: Mapping of common data types

| Stage-2 Type Name | YAML type | Description |
| --- | --- | --- |
| Float | Number, with format “float” | Defined in OpenAPI data types [x] |

# E.2	YANG/Netconf-based solution set

### E.2.1	NRM properties supported

The YANG module "ietf-yang-library" (RFC 8525 [18]) is available via NETCONF at the "mnsAddress". This YANG module lists the supported YANG modules and related information. The individual supported YANG modules are accessible either via the "ietf-yang-library module" as specified by the leaves "/modules-state/module/schema", "/yang-library/module-set/import-only-module/location" and "/yang-library/module-set/module/location" or via the "ietf-netconf-monitoring module" (RFC 6022 [19]) with the "<get-schema>" operation.

### E.2.2	Common data types

Data types in YANG have the same name as in stage 2 also considering the mapping rule in TS 32.160 [14] clause 6.2.16.1. There are some data types that do not conform to this rule because the data type was defined by an external organisation (e.g. IETF) or pre-existing YANG definitions exist. Table E.2.2-1 lists the mapping for these exceptions.

Table E.2.2-1: Mapping of common data types

| Stage-2 Type Name | YANG type | Description |
| --- | --- | --- |
| FullTime | yang:time-with-zone-offset | Defined in ietf-yang-types.yang |
| Float | Decimal64 fraction-digits 1 to 7 | Defined in RFC 7950 [y] |
| Real | Decimal64 fraction-digits 8 to 18 | Defined in RFC 7950 [y] |
| DnList | types3gpp:DistinguishedName | A list or leaf-list of type types3gpp:DistinguishedName Defined in _3gpp-common-yang-types.yang [z] |
| Fqdn | inet:host-name | Defined in ietf-inet-types.yang [z] |
| Ipv4Addr | inet:ipv4-address | Defined in ietf-inet-types.yang [z] |
| Ipv6Addr | inet:ipv6-address | Defined in ietf-inet-types.yang [z] |
| Ipv6Prefix | inet:ipv6-prefix | Defined in ietf-inet-types.yang [z] |
| Uri | inet:uri | Defined in ietf-inet-types.yang [z] |

End of Changes