3GPP TSG-SA5 Meeting #157	S5-245739

Hyderabad, India, 14 - 18 October 2024

# Source:	Ericsson, Deutsche Telekom

# Title:	Discussion on simplification of ACCL model

# Document for:	Approval

Agenda Item:	6.3.14

# 1	Decision/action requested

The group is asked to discuss and endorse the detailed proposal in 4.

# 2	References

[1]	3GPP TS 28.536 Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3

[2]	3GPP TS 28.535 Management and orchestration; Management services for communication service assurance; Requirements

[3]	3GPP TS 28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

# 3	Rationale

The model for closed loop automation can be found in TS 28.536 [1] clause 4.1 and was developed in the Rel-16/Rel-17 timeframe. The model is copied from [1] and shown in Figure 3.1.

Figure 3.1: Assurance management NRM fragment [1]

From studying the ACCL model, the following observations can be made.

Observations

- An ACCL can be contained by either a SubNetwork or a ManagedElement. The entities referenced from an ACCL (i.e. the “monitored entities”) can be a NetworkSlice or NetworkSliceSubnet. The NetworkSlice and the NetworkSliceSubnet class definitions can be found in TS 28.541 clause 6 [3] and it can be noted that neither the NetworkSlice nor the NetworkSliceSubnet can be contained by a ManagedElement.

- Before an ACCL can be operational one or more AssuranceGoals are configured with the appropriate attribute value pairs (targets). This information is captured from the ServiceProfile or SliceProfile. During operation attributes (with the same name as in the AssuranceGoal) report their actual value periodically in the AssuranceReport.

NOTE: When the NetworkSlice or NetworkSliceSubnet requirements are changed the AssuranceGoal needs to be updated to reflect that change. Attributes from a ServiceProfile of a particular NetworkSlice that are monitored by an ACCL may be moved by the consumer to another NetworkSlice monitored by another ACCL. In the same way attributes from a SliceProfile of a particular NetworkSliceSubnet that are monitored by an ACCL may be moved by the consumer to another NetworkSliceSubnet monitored by another ACCL

Identified issues and discussion

- The ACCL model does not reference any entities that are associated with a ManagedElement, and there is no use case or requirements in TS 28.535 [2] that require ACCLs to be contained by a ManagedElement, therefore this relationship does not belong in the class diagram.

- An ACCL can be associated with only one “monitored entity”, this means that “monitored entities” from the same type with similar goals will require different ACCLs while in practice they may participate in the same ACCL. The model should be flexible enough to support many-to-many relationship between ACCL and “monitored entity”.

- The ACCL does not use the ServiceProfileId and SliceProfileId during operation, and visibility in the class diagram may give the impression that it does.

Proposed changes

- Remove ManagedElement from the model (CRs to TS 28.536 have been prepared to implement this change).

- Make the association between an ACCL and “monitored entity” many-to-many. Since a NetworkSlice or NetworkSliceSubnet may be associated with multiple ACCLs, the NetworkSlice NRM in TS 28.541 needs to be updated to include a list of ACCL references. (CRs to TS 28.536 and TS 28.541 have been prepared to implement this change).

- Remove references to ServiceProfile and SliceProfile <<dataTypes>> from the ACCL class diagram (CRs to TS 28.536 have been prepared to implement this change).

The updated assurance management NRM fragment shown in Figure 3.2 depicts the ACCL model after implementation of the proposed changes.

Figure 3.2: Proposed assurance management NRM fragment

PlantUML code for figure in 3.2

@startuml

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

hide circle

hide members

class SubNetwork <<InformationObjectClass>>

class AssuranceClosedControlLoop <<InformationObjectClass>>

class AssuranceReport <<InformationObjectClass>>

class AssuranceGoal <<InformationObjectClass>>

class NetworkSlice <<InformationObjectClass>>

class NetworkSliceSubnet <<InformationObjectClass>>

class AssuranceTarget <<dataType>>

class AssuranceGoalStatus <<dataType>>

class AssuranceTargetStatus <<dataType>>

SubNetwork "1" *-- "*" AssuranceClosedControlLoop: <<names>>

AssuranceClosedControlLoop "1" *-left- "*" AssuranceGoal: <<names>>

AssuranceReport "1" -left-* "1" AssuranceClosedControlLoop: <<names>>

AssuranceReport "1" --> "1..*" AssuranceGoalStatus

AssuranceGoalStatus "1" --> "*" AssuranceTargetStatus

AssuranceGoal "1" --> "*" AssuranceTarget

AssuranceClosedControlLoop "*" --> "*" NetworkSlice

AssuranceClosedControlLoop "*" --> "*" NetworkSliceSubnet

@enduml

# 4	Detailed proposal

The group acknowledges the identified issues and endorse proposed changes.