3GPP TSG-SA5 (Telecom Management)	S5-070174

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.733 | CR | 0001 |  rev | - |  Current version: | 7.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add HssFunction to the CORBA SS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia, Ericsson (gyula.bodog@nokia.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-NIM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | HssFunction is missing from the IMS NRM |  |  |  |  |  |  |  |  |
| Summary of change:  |  | HssFunction, Link_As_Hss, Link_Hss_Scscf and Link_Cscf_Hss is added to the NRM according to the IS specification |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | If HSS is not modelled there will be no possibility for managing the HSS in the 3GPP network. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Section 1, section 5 and Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.732, 32.735, 32.632, 32.633, 32.635 |  |  |  |
| Other comments:  |  | Related R7 CR to 32.732 is in S5-070173, CR to 32.735 is in S5-070175, CR to 32.632 is in S5-070176, CR to 32.633 is in S5-070177 and CR to 32.635 is in S5-070178 |  |  |  |  |  |  |  |  |

1st Modified Section

# 1	Scope

The purpose of this IMS NRM IRP: CORBA Solution Set is to define the mapping of the IRP Information Service (see TS 32.732 [3]) to the protocol specific details necessary for implementation of this IRP in a CORBA/IDL environment.

This Solution Set specification is related to 3GPP TS 32.732 V7.1.X.

Next Modified Section

## 5.2	Information Object Class (IOC) mapping

### 5.2.1	IOC AsFunction

Mapping from NRM IOC AsFunction attributes to SS equivalent MOC AsFunction

| Attributes of IOC AsFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| asFunctionId | asFunctionId | string | Read-Only, M |

### 5.2.2	IOC BgcfFunction

Mapping from NRM IOC BgcfFunction attributes to SS equivalent MOC BgcfFunction

| Attributes of IOC BgcfFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| bgcfFunctionId | bgcfFunctionId | string | Read-Only, M |

### 5.2.3	IOC CscfFunction

Mapping from NRM IOC CscfFunction attributes to SS equivalent MOC CscfFunction

| Attributes of IOC CscfFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| cscfFunctionId | cscfFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |

### 5.2.4	IOC IcscfFunction

Mapping from NRM IOC IcscfFunction attributes to SS equivalent MOC IcscfFunction

| Attributes of IOC IcscfFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| icscfFunctionId | icscfFunctionId | string | Read-Only, M |

### 5.2.5	IOC ImsMgwFunction

Mapping from NRM IOC ImsMgwFunction attributes to SS equivalent MOC ImsMgwFunction attributes

| Attributes of IOC ImsMgwFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| imsMgwFunctionId | imsMgwFunctionId | string | Read-Only, M |

### 5.2.6	IOC MgcfFunction

Mapping from NRM IOC MgcfFunction attributes to SS equivalent MOC MgcfFunction

| Attributes of IOC MgcfFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| mgcfFunctionId | mgcfFunctionId | string | Read-Only, M |

### 5.2.7	IOC MrfcFunction

Mapping from NRM IOC MrfcFunction attributes to SS equivalent MOC MrfcFunction

| Attributes of IOC MrfcFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| mrfcFunctionId | mrfcFunctionId | string | Read-Only, M |

### 5.2.8	IOC MrfpFunction

Mapping from NRM IOC MrfpFunction attributes to SS equivalent MOC MrfpFunction

| Attributes of IOC MrfpFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| mrfpFunctionId | mrfpFunctionId | string | Read-Only, M |

### 5.2.9	IOC PcscfFunction

Mapping from NRM IOC PcscfFunction attributes to SS equivalent MOC PcscfFunction

| Attributes of IOC PcscfFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| pcscfFunctionId | pcscfFunctionId | string | Read-Only, M |

### 5.2.10	IOC ScscfFunction

Mapping from NRM IOC ScscfFunction attributes to SS equivalent MOC ScscfFunction

| Attributes of IOC ScscfFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| scscfFunctionId | scscfFunctionId | string | Read-Only, M |

### 5.2.11	IOC SlfFunction

Mapping from NRM IOC SlfFunction attributes to SS equivalent MOC SlfFunction

| Attributes of IOC SlfFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| slfFunctionId | slfFunctionId | string | Read-Only, M |

### 5.2.12	IOC Link_As_Cscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.13	IOC Link_As_Scscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.14	IOC	Link_As_Slf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.15	IOC Link_Bgcf_Bgcf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.16	IOC Link_Bgcf_Cscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.17	IOC Link_Bgcf_Mgcf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.18	IOC Link_Bgcf_Scscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.19	IOC Link_Cscf_Cscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.20	IOC Link_Cscf_Icscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.21	IOC Link_Cscf_Mgcf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.22	IOC Link_Cscf_Mrfc

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.23	IOC Link_Cscf_Pcscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.24	IOC Link_Cscf_Scscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.25	IOC Link_Cscf_Slf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.26	IOC Link_Icscf_Slf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.27	IOC Link_ImsMgw_Mgcf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.28	IOC Link_Mgcf_Scscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.29	IOC Link_Mrfc_Mrfp

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.30	IOC Link_Mrfc_Scscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.31	IOC Link_Scscf_Scscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.32	IOC Link_Scscf_Slf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.xx	IOC HssFunction

Mapping from NRM IOC HssFunction attributes to SS equivalent MOC HssFunction

| Attributes of IOC HssFunction in TS 32.732 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| hssFunctionId | hssFunctionId | string | Read-Only, M |

### 5.2.yy	IOC Link_Hss_Scscf

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.zz	IOC Link_Cscf_Hss

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

### 5.2.vv	IOC Link_As_Hss

All attributes are inherited from Link. See mapping of attributes for Link IOC in 3GPP TS 32.623 [5].

Next Modified Section

Annex A (normative):
CORBA IDL, NRM Definitions

# A.1	IDL specification (file name "IMSNRMDefs.idl")

// File: IMSNRMDefs.idl

#ifndef _IMSNRMDEFS_IDL_

#define _IMSNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module IMSNRMDefs

{

/**

*  Definitions for MO class AsFunction

*/

interface AsFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "AsFunction";

// Attribute Names

//

const string asFunctionId = "asFunctionId";

};

/**

*  Definitions for MO class BgcfFunction

*/

interface BgcfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "BgcfFunction";

// Attribute Names

//

const string bgcfFunctionId = "bgcfFunctionId";

};

/**

*  Definitions for MO class CscfFunction

*/

interface CscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CscfFunction";

// Attribute Names

//

const string cscfFunctionId = "cscfFunctionId";

};

/**

*  Definitions for MO class IcscfFunction

*/

interface IcscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IcscfFunction";

// Attribute Names

//

const string icscfFunctionId = "icscfFunctionId";

};

/**

*  Definitions for MO class ImsMgwFunction

*/

interface ImsMgwFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ImsMgwFunction";

// Attribute Names

//

const string imsMgwFunctionId = "imsMgwFunctionId";

};

/**

*  Definitions for MO class MgcfFunction

*/

interface MgcfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MgcfFunction";

// Attribute Names

//

const string mgcfFunctionId = "mgcfFunctionId";

};

/**

*  Definitions for MO class MrfcFunction

*/

interface MrfcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MrfcFunction";

// Attribute Names

//

const string mrfcFunctionId = "mrfcFunctionId";

};

/**

*  Definitions for MO class MrfpFunction

*/

interface MrfpFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MrfpFunction";

// Attribute Names

//

const string mrfpFunctionId = "mrfpFunctionId";

};

/**

*  Definitions for MO class PcscfFunction

*/

interface PcscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "PcscfFunction";

// Attribute Names

//

const string pcscfFunctionId = "pcscfFunctionId";

};

/**

*  Definitions for MO class ScscfFunction

*/

interface ScscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ScscfFunction";

// Attribute Names

//

const string scscfFunctionId = "scscfFunctionId";

};

/**

*  Definitions for MO class SlfFunction

*/

interface SlfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SlfFunction";

// Attribute Names

//

const string slfFunctionId = "slfFunctionId";

};

/**

*  Definitions for MO class Link_As_Cscf

*/

interface Link_As_Cscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Cscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_As_Scscf

*/

interface Link_As_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_As_Slf

*/

interface Link_As_Slf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Bgcf

*/

interface Link_Bgcf_Bgcf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Bgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Cscf

*/

interface Link_Bgcf_Cscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Cscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Mgcf

*/

interface Link_Bgcf_Mgcf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Mgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Scscf

*/

interface Link_Bgcf_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Cscf

*/

interface Link_Cscf_Cscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Cscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Icscf

*/

interface Link_Cscf_Icscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Icscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Mgcf

*/

interface Link_Cscf_Mgcf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Mgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Mrfc

*/

interface Link_Cscf_Mrfc: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Mrfc";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Pcscf

*/

interface Link_Cscf_Pcscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Pcscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Scscf

*/

interface Link_Cscf_Scscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Slf

*/

interface Link_Cscf_Slf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Icscf_Slf

*/

interface Link_Icscf_Slf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Icscf_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_ImsMgw_Mgcf

*/

interface Link_ImsMgw_Mgcf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_ImsMgw_Mgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Mgcf_Scscf

*/

interface Link_Mgcf_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Mgcf_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Mrfc_Mrfp

*/

interface Link_Mrfc_Mrfp : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Mrfc_Mrfp";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Mrfc_Scscf

*/

interface Link_Mrfc_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Mrfc_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Scscf_Scscf

*/

interface Link_Scscf_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Scscf_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Scscf_Slf

*/

interface Link_Scscf_Slf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Scscf_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class HssFunction

*/

interface HssFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "HssFunction";

// Attribute Names

//

const string hssFunctionId = "hssFunctionId";

};

/**

*  Definitions for MO class Link_Hss_Scscf

*/

interface Link_Hss_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Hss_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Hss

*/

interface Link_Cscf_Hss : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Hss";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_As_Hss

*/

interface Link_As_Hss : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Hss";

// All Attributes inherited from Link

};

};

#endif // _IMSNRMDEFS_IDL_

Next Modified Section

End of modifications

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | R | Subject/Comment | Cat | Old | New |
| Sep 2006 | SA_33 | SP-060564 | -- | -- | Submitted to TSG SA #33 for Information | -- | -- | 1.0.0 |
| Dec 2006 | SA_34 | SP-060751 | -- | -- | Submitted to TSG SA #34 for Approval | -- | 2.0.0 | 7.0.0 |