3GPP TSG-SA5 (Telecom Management)	S5-090531

Meeting SA5#62 bis, 14-16 January 2009, Sophia Antipolis, France

- Source:	Alcatel-lucent

- Title:	32.xx1 Add requirements to Fault management Section

- Document for:	Discussion and Approval

Agenda Item:	6.1 UID_xxxxxx 3G Home NodeB OAM&P Interface Type 1 Management

# 1	Decision/action requested

The group is asked to discuss the fault management related requirements for 3G HNB Management, and introduce the agreed part into the draft TS 32.XX1.

# 2	References

[1]	3GPP TR 32.821: "Study of Self-Organising Networks (SON) related OAM for Home NodeB (Release 8)"

[2] 3GPP S5-082488: “New WT-level WID on 3G Home NodeB OAM&P (Interface Type 1 Management”)

# 3	Rationale

This contribution is to fault management requirements to the TS32.xx1 based on the contents captured in TR32.821[1].

# 4	Detailed proposal

# Text Proposal for TS 32.xx1

1st Modified Section

# 5	Business Level Requirements

## 5.1	Requirements

The figure below shows the overall general architecture of the remote management functions i.e. configuration management, performance management and event/fault management that need to be supported for Home NodeB Network. It gives high level information, broadly categorising various network elements and interfaces which will be required for efficient managent of the Home NodeB network.

End of 1st modifications

2nd Modified Section

# 5	Business Level Requirements

## 5.1	Requirements

### 5.1.4	Fault Management

Bus Req-XX1-1 The Home NodeB shall support Fault Management to enable the operator to maintain his mobile network with Home NodeBs with high availability and good quality of service.

End of 2nd modifications

3rd Modified Section

# 6	Specification level requirements

## 6.5	Requirements

### 6.5.4	Fault Management

Spec Req-XX1-3 	In case of use of RPC method, the HMS shall have the capability to activate and deactivate the alarm reporting by HNB.

Spec Req-XX1-5 	In case of use of RPC method,, the HMS shall be able to define the frequency of passive reporting.

Spec Req-XX1-6 	In case of use of RPC method, the HMS shall be informed immediately of alarms (raised, changed, cleared) classified as expedited notifications only.

End of 3rd modifications