3GPP TSG-SA5 (Telecom Management)	S5-090708

Meeting SA5#62 bis, 14-16 January 2009, Sophia Antipolis, France

- Source:	IP.Access

- Title:	32.XX1 3G Home NodeB OAM&P Requirements

- Document for:	Discussion and Approval

Agenda Item:	6.1 UID xxxxxx 3G Home NodeB OAM&P Interface Type 1 Management

# 1	Decision/action requested

The group is asked to discuss the proposed requirements for 3G HNB OAM&P (for type 1 interface), and introduce the agreed part into the draft TS 32.XX1.

# 2	References

# 3	Rationale

This is a contribution to SA5 as a result of rewording requirements for further re-consideration.

# 4	Detailed proposal

# Text Proposal for TS 32.XX1

1st Modified Section

## 6.5	Requirements

### 6.5.3	Fault Management

REQ-OAMP-FM-005:...The HNB shall maintain the following  information:

a.Alarm Management Information – which contains the alarm management and reporting parameters configurable by the HMS

b. Alarms List – Alarms currently active on the HNB

c.Alarm History  – contains the alarms previously created by the HNB.

d.Pending Delivery Queue – contains the alarms queued to be sent to the HMS on the next management connection

REQ-OAMP-FM-006:..In case of use of RPC method, the HNB may support the following ways of alarm handling:

a.Expedited handling– the HNB connects to the HMS immediately to raise the alarm and logs the alarm in the Alarm History.

b.Queued handling – the HNB queues the alarm internally pending connection to the HMS,  logs the alarm in the Alarm History, and delivers the alarm on the next connection to the HMS

c.Logged handling – the HNB does not send the alarm to the HMS and logs the alarm in the Alarm History.

d.Disabled handling– the HNB does not send the alarm to the HMS and will not log the alarm in the Alarm History

REQ-OAMP-FM-007: The HMS may configure the alarm handling for each type of HNB alarm according to the HNB alarm handling capabilities and the default handling if not specified by the HMS shall be “Logged handling”.

REQ-OAMP-FM-015: The HMS shall have the capability to completely purge on the HNB  the Alarms List and the Pending Delivery Queue and may have the capability to completely purge on the HNB the

history of Alarms.

b.

End of 1st modifications