3GPP TSG-SA5 (Telecom Management)	S5-092775

Meeting SA5#66, June 29 – July 3, 2009, Dublin, Ireland

- Source:	Ericsson

- Title:	d32Xy1 v010 H(e)NB OAM&P concepts and req - re FM Requirements

- Document for:	Discussion and Approval

Agenda Item:

# 1	Decision/action requested

Review the proposal for inclusion in [1].

# 2	References

None.

# 3	Rationale

Add specification level requirements

# 4	Detailed proposal

1st Modified Section

6.1.2	Fault management

[REQ-HNB-FUN-1] The IRPAgent shall support a capability allowing IRPManager to read, acknowledge, clear and comment alarms categorized as “Expedited handling” (see REQ-OAMP_FM-FUN-005 of [4]) or categorized as “Queued handling” (see REQ-OAMP_FM-FUN-005 of [4]).‬

[REQ-HNB-FUN-1] The IRPAgent shall support a capability allowing IRPManager to use existing AlarmIRP operations to manage alarms categorized as “Expedited handling” (see REQ-OAMP_FM-FUN-005 of [4]) or categorized as “Queued handling” (see REQ-OAMP_FM-FUN-005 of [4]).‬

End of Modified Section