3GPP TSG-SA5 (Telecom Management)	S5-092779

Meeting SA5#66, 29 June – 3 July 2009, Dublin, Ireland

- Source:	Alcatel-Lucent

- Title:	Add Requirements for HeNB Management

- Document for:	Discussion and Approval

Agenda Item:	6.4.3

# 1	Decision/action requested

The group is asked to discuss the proposed requirements, and introduce the agreed part into the draft TS 32.591.

# 2	References

[1] 3GPP TS 32.581

[2] 3GPP TS 32.591

# 3	Rationale

The proposed requirements are introduced to:

1. Add a requirement related to firmware upgrade in addition to software upgrate

2.  Add a requirement to maintain configuration data following reboots.

3.  Introduce a requirement for performing a selective troubleshoot of specific functions

4.  Stenghtening the security requirements

5.  Add a requirement to allow either the HeNB or the HeMS to start a management connection

# 4	Detailed proposal

1st Modified Section

# 5	Business level requirements

## 5.1	Requirements

### 5.1.1	Configuration Management

REQ-OAMP_CM-CON-003 The HeNB shall be able to automatically, i.e. without human operator on-line interaction or attention, upgrade its software/firmware and configuration.

End of 1st Modified Section

2nd Modified Section

# 6	Specification level requirements

## 6.1	Requirements

### 6.1.1	Configuration Management

REQ-OAMP_CM-CON-xx1 The HeNB Management Protocol shall manage downloading of HeNB software/firmware image files and provide mechanisms for version identification and notification to the HMS of the success or failure of a file download.

REQ-OAMP_CM-CON-xx2 In normal operation, the HeNB shall maintain its configuration data following a HeNB reboot.

REQ-OAMP_CM-CON-xx3 The HeNB Management Protocol shall support capabilities to inform the HeMS about the results of specific actions triggered by the HeMS.

REQ-OAMP_CM-CON-xx4 It shall be possible to initiate a management connection at the request of either the HeNB or the HeMS.

End of 2nd modifications

3rd modifications

## 6.1.4   Security Management

REQ-OAMP_SM-CON –xx1 The HeNB Management Procotol shall prevent tampering with the interactions that take place between the HeNB and the HeMS as well as management functions of a HeNB.

REQ-OAMP_SM-CON –xx2 The HeNB Management Protocol shall allow the authentication  by the HeMS of HeNBs.

REQ-OAMP_SM-CON –xx3 The HeNB Management Protocol shall allow a HeNB to authenticate the HeMS prior to responding to transactions triggered by the HeMS.

REQ-OAMP_SM-CON –xx4 The HeNB Management Protocol shall provide confidentiality for transactions taking place between the HeNB and the HeMS.

End of 3rd modifications

4th modifications

## 6.1.3  Fault Management

REQ-OAMP_FM-CON –xx1 The HeNB Management Protocol shall support the capability to allow a HeNB to make available information that the HeMS may use to diagnose and resolve connectivity or service issues.

End of 4th modifications