3GPP TS 32.507 V9.1.0 (2010-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Self Configuration of Network Elements

Integration Reference Point (IRP):

SOAP Solution Set (SS)

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management, SOAP

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Architectural features	7

4.1	General	7

5	Mapping	8

5.1	Operation and notification mapping	8

5.2	Operation parameter mapping	8

5.2.1	Operation listScManagementCapabilities	9

5.2.1.1	Input parameters	9

5.2.1.2	Output parameters	9

5.2.1.3	Fault definition	9

5.2.2	Operation listScManagementProfiles	9

5.2.2.1	Input parameters	9

5.2.2.2	Output parameters	9

5.2.3	Operation createScManagementProfile	10

5.2.3.1	Input parameters	10

5.2.3.2	Output parameters	10

5.2.4	Operation deleteScManagementProfile	10

5.2.4.1	Input parameters	10

5.2.4.2	Output parameters	11

5.2.5	Operation listScProcesses	11

5.2.5.1	Input parameters	11

5.2.5.2	Output parameters	11

5.2.6	Operation resumeScProcess	11

5.2.6.1	Input parameters	11

5.2.6.2	Output parameters	11

5.2.7	Operation resumeScProcessWithArcfData	12

5.2.7.1	Input parameters	12

5.2.7.2	Output parameters	12

5.2.8	Operation terminateScProcess	12

5.2.8.1	Input parameters	12

5.2.8.2	Output parameters	13

5.2.9	Operation changeScManagementProfile	13

5.2.9.1	Input parameters	13

5.2.9.2	Output parameters	13

Annex A (normative):	WSDL specifications	14

Annex B (informative):	Change history	23

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management, as identified below:

32.501:	Self-Configuration of Network Elements; Concepts and  Integration Reference Point (IRP) Requirements

32.502:	Self-Configuration of Network Elements Integration Reference Point (IRP); Information Service (IS)

32.503:	Self-Configuration of Network Elements Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)

32.505:	Self-Configuration of Network Elements Integration Reference Point (IRP);  eXtensible Markup Language (XML) file format definition

32.507:	Self-Configuration of Network Elements Integration Reference Point (IRP): SOAP Solution Set (SS)"

The present document is part of a TS-family which describe the requirements and information model necessary for the Telecommunication Management (TM) of 3G systems. The TM principles and TM architecture are specified in 3GPP TS 32.101 [1] and 3GPP TS 32.102 [2].

For the purpose of Self-Configuration of Network Elements IRP, see TS 32.501 [3]

# 1	Scope

The present document specifies the SOAP Solution Set for the IRP whose semantics are specified in Self-Configuration of Network Elements Integration Reference Point (IRP): Information Service (3GPP TS 32.502 [4]).

This Solution Set specification is related to 3GPP TS 32.502 V9.0.X.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.501: " Technical Specification Group Services and System Aspects; Telecommunication management; Self-Configuration of Network Elements; Concepts and  Integration Reference Point (IRP) Requirements ".

[4]	3GPP TS 32.502: " Technical Specification Group Services and System Aspects; Telecommunication management; Self-Configuration of Network Elements Integration Reference Point (IRP); Information Service (IS)".

[5]	3GPP TS 32.505: " Telecommunication management; A Self-Configuration of Network Elements Integration Reference Point (IRP);  eXtensible Markup Language (XML) file format definition"

[6]	3GPP TS 32.311: "Telecommunication management; Generic Integration Reference Point (IRP) management; Requirements"

[7]	3GPP TS 32.312: "Telecommunication management; Generic Integration Reference Point (IRP) management; Information Service (IS)".

[8]	3GPP TS 32.317: "Telecommunication management; Generic Integration Reference Point (IRP) management; SOAP solution set".

[9]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[10]	3GPP TS 32.307: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): SOAP solution set".

[11]	W3C SOAP 1.1 specification (http://www.w3.org/TR/2000/NOTE-SOAP-20000508/)

[12]	W3C XPath 1.0 specification (http://www.w3.org/TR/1999/REC-xpath-19991116)

[13]	W3C WSDL 1.1 specification (http://www.w3.org/TR/2001/NOTE-wsdl-20010315)

[14]	W3C SOAP 1.2 specification (http://www.w3.org/TR/soap12-part1/)

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.150 [9] and 3GPP TS 32.501 [3] and the following apply:

IRP document version number string (or "IRPVersion"): See 3GPP TS 32.311 [6].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

IS	Information Service

SC	Self Configuration

SS	Solution Set

WSDL	Web Service Description Language

# 4	Architectural features

## 4.1	General

The overall architectural feature of the Self-Configuration of Network Elements IRP is specified in 3GPP TS 32.502 [4].  This clause specifies features that are specific to the SOAP solution set.

The SOAP 1.1 specification [11] and WSDL 1.1 specification [13] are supported.

The SOAP 1.2 specification [14] is supported optionally.

This specification uses "document" style in WSDL file.

This specification uses "literal" encoding style in WSDL file.

The filter language used in the SS is the XPath Language (see W3C XPath 1.0 specification [12]). IRPAgents may throw a FilterComplexityLimit fault when a given filter is too complex.

Relevant definitions are imported from the Self-Configuration of Network Elements22 XML definitions of 3GPP TS 32.505 [5].

This specification uses a number of namespace prefixes throughout that are listed in Table 4.1.

Table 4.1: Prefixes and Namespaces used in this specification

| PREFIX | NAMESPACE |
| --- | --- |
| (no prefix) | http://schemas.xmlsoap.org/wsdl/ |
| soap | http://schemas.xmlsoap.org/wsdl/soap/ |
| ScIRPSystem | http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#SCIRPSystem |
| ScIRPData | http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#SCIRPData |
| xsc | http://www.3gpp.org/ftp/specs/archive/32_series/32.505#ScIRPIOCs |
| genericIRPSystem | http://www.3gpp.org/ftp/Specs/archive/32_series/32.507/schema/32317-810/GenericIRPSystem |
| ntfIRPNtfSystem | http://www.3gpp.org/ftp/Specs/archive/32_series/32.507/schema/32507-810/notification/NotificationScNtfSystem |

The WSDL structure is depicted in Figure 4.1 below, depicting port type, binding and service. The port type contains port type operations, which again contains input, output and fault messages. The binding contains binding operations, which have the same name as the port type operations. The binding connects to a port inside the service.

Figure 4.1:  Self-Configuration of Network Elements IRP SOAP Solution Set WSDL structure

## 5	Mapping

## 5.1	Operation and notification mapping

The Self-Configuration of Network Elements IRP IS (3GPP TS 32.502 [4]) defines the operations and their semantics.

Table 5.1 maps the operations defined in the Self-Configuration of Network Elements IRP IS to their equivalent port type and binding operations in this Solution Set (SS).

Table 5.1 also maps the notifications of the Self-Configuration of Network Elements IRP IS, as well as inherited operations.

Table 5.1 also qualifies if an operation is Mandatory (M) or Optional (O).

Table 5.1: Mapping from IS Operation to SS Equivalents

| IS Operation in 3GPP TS 32.502 [4] | SS: Operation for WSDL port type and WSDL binding | SS: Port of SCIRPService | Qualifier |
| --- | --- | --- | --- |
| listScManagementCapabilities | listScManagementCapabilities (note 1) | SCIRPOperation1 | M |
| listScManagementProfiles | listScManagementProfiles (note 1) | SCIRPOperation1 | M |
| createScManagementProfile | createScManagementProfile (note1) | SCIRPOperation1 | M |
| deleteScManagementProfile | deleteScManagementProfile (note1) | SCIRPOperation1 | M |
| listScProcesses | listScProcesses (note1) | SCIRPOperation1 | M |
| resumeScProcess | resumeScProcess (note1) | SCIRPOperation1 | M |
| resumeScProcessWithArcfData | resumeScProcessWithArcfData(note1) | SCIRPOperation1 | M |
| terminateScProcess | terminateScProcess (note1) | SCIRPOperation1 | M |
| changeScManagementProfile | changeScManagementProfile (note1) | SCIRPOperation1 | O |
| notifyScManagementProfileCreation | notify (note 2) | NotificationIRPNtfPort | M |
| notifyScManagementProfileDeletion | notify (note 2) | NotificationIRPNtfPort | M |
| notifyScProcessCreation | notify (note 2) | NotificationIRPNtfPort | M |
| notifyScProcessStage | notify (note 2) | NotificationIRPNtfPort | M |
| notifyScProcessDeletion | notify (note 2) | NotificationIRPNtfPort | M |
| notifyNewScManagementCapabilityAvailability | notify (note 2) | NotificationIRPNtfPort | M |
| notifyScManagementProfileChange | notify (note 2) | NotificationIRPNtfPort | O |
| getIRPVersion (note 3) | See TS 32.317 [8] | GenericIRPPort | M |
| getOperationProfile (note 3) | See TS 32.317 [8] | GenericIRPPort | O |
| getNotificationProfile (note 3) | See TS 32.317 [8] | GenericIRPPort | O |
| NOTE 1: The operation is under the port type SCIRPSystem:SCIRPPortType and under the binding SCIRPSystem:SCIRPBinding. |  |  |  |
| NOTE 2: The IS equivalent maps to an XML definition specified in 3GPP TS 32.505 [5], and this being an input parameter to the operation notify under the port type ntfIRPNtfSystem:NotificationIRPNtf and under the binding ntfIRPNtfSystem:NotificationIRPNtf of 3GPP TS 32.307 [10]. This binding is linked to a port of the SCIRPService as indicated in the table above.. |  |  |  |
| NOTE 3: The IS operation is inherited from the ManagedGenericIRP IOC specified in 3GPP TS 32.312 [7]. This inheritance is by the SelfConfigurationIRP IOC of 3GPP TS 32.502 [4] inheriting from the ManagedGenericIRP IOC. The corresponding binding is linked to a port of the SCIRPService as indicated in the table above. |  |  |  |

## 5.2	Operation parameter mapping

The SCIRP IS (3GPP TS 32.502 [4]) defines semantics of parameters carried in the operations. The tables below show the mapping of these parameters, as per operation, to their equivalents defined in this SS.

### 5.2.1	Operation listScManagementCapabilities

#### 5.2.1.1	Input parameters

Table 5.2.1.1: Mapping from IS listScManagementCapabilities input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| nEInformation | nEInformation | M |

#### 5.2.1.2	Output parameters

Table 5.2.1.2: Mapping from IS listScManagementCapabilities output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| capabilitiesList | capabilitiesList | M |
| Result | Result | M |

#### 5.2.1.3	Fault definition

Table 5.2.1.3: Mapping from IS listScManagementCapabilities exceptions to SS equivalents

| Assertion name | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| operation_failed | operation_failed | M |

### 5.2.2	Operation listScManagementProfiles

#### 5.2.2.1	Input parameters

Table 5.2.2.1: Mapping from IS listScManagementProfiles input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| nEInformation | nEInformation | M |

#### 5.2.2.2	Output parameters

Table 5.2.1.1: Mapping from IS listScManagementProfiles output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| ScManagementProfileList | ScManagementProfileList | M |
| result | result | M |

### 5.2.3	Operation createScManagementProfile

#### 5.2.3.1	Input parameters

Table 5.2.3.1: Mapping from IS createScManagementProfile input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| id | id | O |
| nEInformation | nEInformation | M |
| swVersionToBeInstalled | swVersionToBeInstalled | M |
| stepsAndSelectedStopPointList | stepsAndSelectedStopPointList | M |
| selectedFinalAdministrativeState | selectedFinalAdministrativeState | M |

#### 5.2.3.2	Output parameters

Table 5.2.3.2: Mapping from IS createScManagementProfile output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| id | id | CM |
| result | result | M |

### 5.2.4	Operation deleteScManagementProfile

#### 5.2.4.1	Input parameters

Table 5.2.4.1: Mapping from IS deleteScManagementProfile input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| id | id | M |

#### 5.2.4.2	Output parameters

Table 5.2.4.2: Mapping from IS deleteScManagementProfile output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| result | result | M |

### 5.2.5	Operation listScProcesses

#### 5.2.5.1	Input parameters

Table 5.2.5.1: Mapping from IS listScProcesses input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| nEIdentification | nEIdentification | O |

#### 5.2.5.2	Output parameters

Table 5.2.5.2: Mapping from IS listScProcesses output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| ScManagementprocessList | ScManagementprocessList | M |
| result | result | M |

### 5.2.6	Operation resumeScProcess

#### 5.2.6.1	Input parameters

Table 5.2.6.1: Mapping from IS resumeScProcess input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| id | id | M |
| startStepName | startStepName | M |

#### 5.2.6.2	Output parameters

Table 5.2.6.2: Mapping from IS resumeScProcess output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| result | result | M |

### 5.2.7	Operation resumeScProcessWithArcfData

#### 5.2.7.1	Input parameters

Table 5.2.7.1: Mapping from IS resumeScProcessWithArcfData input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| valuesOfNeededRadioParameter | valuesOfNeededRadioParameter | M |
| fileLocation | fileLocation | M |

#### 5.2.7.2	Output parameters

Table 5.2.7.2: Mapping from IS resumeScProcessWithArcfData output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| validationErrorInfo | validationErrorInfo | CM |

### 5.2.8	Operation terminateScProcess

#### 5.2.8.1	Input parameters

Table 5.2.8.1: Mapping from IS terminateScProcess input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| id | id | M |

#### 5.2.8.2	Output parameters

Table 5.2.8.2: Mapping from IS terminateScProcess output parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| result | result | M |

### 5.2.9	Operation changeScManagementProfile

#### 5.2.9.1	Input parameters

Table 5.2.9.1: Mapping from IS changeScManagementProfile input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| id | id | M |
| nEInformation | nEInformation | M |
| swVersionToBeInstalled | swVersionToBeInstalled | M |
| stepsAndSelectedStopPointList | stepsAndSelectedStopPointList | M |
| selectedFinalAdministrativeState | selectedFinalAdministrativeState | M |

#### 5.2.9.2	Output parameters

Table 5.2.9.1: Mapping from IS changeScManagementProfile input parameters to SS equivalents

| IS Operation parameter | SS WSDL type sub-element used in corresponding input message under corresponding port type operation as indicated in Table 5.1 | Qualifier |
| --- | --- | --- |
| result | result | M |
| versionNumber | versionNumber | M |
| conflictingProfileId | conflictingProfileId | C *) |

Annex A (normative):
WSDL specifications

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 32.507 Self Configuration IRP SOAP Solution Set

-->

<definitions xmlns="http://schemas.xmlsoap.org/wsdl/" xmlns:soap="http://schemas.xmlsoap.org/wsdl/soap/" xmlns:ScIRPSystem="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#ScIRPSystem" xmlns:ScIRPData="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#ScIRPData" xmlns:xsc="http://www.3gpp.org/ftp/specs/archive/32_series/32.505#scIRPIOCs" xmlns:xn="http://www.3gpp.org/ftp/specs/archive/32_series/32.625#genericNrm" xmlns:genericIRPSystem="http://www.3gpp.org/ftp/Specs/archive/32_series/32.317/schema/32317-810/GenericIRPSystem" xmlns:ntfIRPNtfSystem="http://www.3gpp.org/ftp/Specs/archive/32_series/32.307/schema/32307-810/notification/NotificationIRPNtfSystem" xmlns:ns="http://www.w3.org/2001/XMLSchema" xmlns:ns1="http://schemas.xmlsoap.org/soap/encoding/" targetNamespace="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#ScIRPSystem">

<import namespace="http://www.3gpp.org/ftp/Specs/archive/32_series/32.307/schema/32307-810/notification/NotificationIRPNtfSystem" location="http://www.3gpp.org/ftp/Specs/archive/32_series/32.307/schema/32307-810-wsdl.zip"/>

<import namespace="http://www.3gpp.org/ftp/Specs/archive/32_series/32.317/schema/32317-810/GenericIRPSystem" location="http://www.3gpp.org/ftp/Specs/archive/32_series/32.317/schema/32317-810-wsdl.zip"/>

<types>

<schema xmlns="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#ScIRPData">

<import namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.505#scIRPIOCs"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.625#genericNrm"/>

<!-- The following types are defined for the SC IRP operations -->

<simpleType name="NEInformation">

<restriction base="string"/>

</simpleType>

<simpleType name="Id">

<restriction base="xn:dn"/>

</simpleType>

<simpleType name="radioParameterName">

<restriction base="string"/>

</simpleType>

<simpleType name="radioParameterValue">

<restriction/>

</simpleType>

<simpleType name="FileLocation">

<restriction base="string"/>

</simpleType>

<complexType name="capability">

<sequence>

<element name="Id" type="ScIRPData:Id"/>

<element name="NEInformation" type="ScIRPData:NEInformation"/>

<element name="StepsAndOfferedStopPointList" type="xsc:StepsAndOfferedStopPointList"/>

<element name="OfferedFinalAdministrativeStateInformation" type="xsc:OfferedFinalAdministrativeStateInformation"/>

</sequence>

</complexType>

<complexType name="capabilitiesList">

<sequence>

<element name="capability" type="ScIRPData:capability" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="ScProfile">

<sequence>

<element name="Id" type="ScIRPData:Id"/>

<element name="VersionNumber" type="xsc:VersionNumber"/>

<element name="NEInformation" type="ScIRPData:NEInformation"/>

<element name="StepsAndSelectedStopPointList" type="xsc:StepsAndSelectedStopPointList"/>

<element name="FinalAdministrativeStateValue" type="xsc:FinalAdministrativeStateValue"/>

<element name="swVersionToBeInstalled" type=" xsc:VersionNumber" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ScProfileList">

<sequence>

<element name="ScProfile" type=" ScIRPData:ScProfile" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="ScProcess">

<sequence>

<element name="Id" type="ScIRPData:Id"/>

<element name="NEInformation" type="ScIRPData:NEInformation"/>

<element name="ProfileId" type="xsc:ProfileId"/>

<element name="StepInfoList" type="xsc:StepInfoList"/>

</sequence>

</complexType>

<complexType name="ScProcessList">

<sequence>

<element name="ScProcess" type="ScIRPData:ScProcess" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="startStepName">

<sequence>

<element name="startStepName" type="string"/>

</sequence>

</complexType>

<complexType name="listOfNeededRadioParameters">

<sequence>

<element name="radioParameterName" type="ScIRPData:radioParameterName"/>

<element name="radioParameterValue" type="ScIRPData:radioParameterValue"/>

</sequence>

</complexType>

<complexType name="valuesOfNeededRadioParameter">

<sequence>

<element name="listOfNeededRadioParameters" type="ScIRPData:listOfNeededRadioParameters"/>

</sequence>

</complexType>

<!--listScManagementCapabilities Request-->

<element name="listScManagementCapabilitiesRequest">

<complexType>

<sequence>

<element name="nEInformation" type="ScIRPData:NEInformation"/>

</sequence>

</complexType>

</element>

<!--listScManagementCapabilities Response -->

<element name="listScManagementCapabilitiesResponse">

<complexType>

<sequence>

<element name="capabilitiesList" type="ScIRPData:capabilitiesList"/>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="success"/>

<enumeration value="failure"/>

<enumeration value="stepNameNotMatch"/>

<enumeration value="nEInformationIntersection"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--listScManagementCapabilities Fault -->

<element name="listScManagementCapabilitiesFault">

<simpleType>

<restriction base="string">

<enumeration value="operationfailed"/>

</restriction>

</simpleType>

</element>

<!--listScManagementProfiles  Request -->

<element name="listScManagementProfilesRequest">

<complexType>

<sequence>

<element name="nEInformation" type="ScIRPData:NEInformation"/>

</sequence>

</complexType>

</element>

<!--listScManagementProfiles  Response -->

<element name="listScManagementProfilesResponse">

<complexType>

<sequence>

<element name="ScProfileList" type="ScIRPData:ScProfileList"/>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="success"/>

<enumeration value="failure"/>

<enumeration value="stepNameNotMatch"/>

<enumeration value="nEInformationIntersection"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--createScManagementProfile Request -->

<element name="createScManagementProfileRequest">

<complexType>

<sequence>

<element name="id" type="ScIRPData:Id" minOccurs="0"/>

<element name="nEInformation" type="ScIRPData:NEInformation"/>

<element name="swVersionToBeInstalled" type="xsc:VersionNumber"/>

<element name="stepsAndSelectedStopPointList" type="xsc:StepsAndSelectedStopPointList"/>

<element name="selectedFinalAdministrativeState" type="xsc:FinalAdministrativeStateValue"/>

</sequence>

</complexType>

</element>

<!--createScManagementProfile Response -->

<element name="createScManagementProfileResponse">

<complexType>

<sequence>

<element name="id" type="ScIRPData:Id" minOccurs="0"/>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="Success"/>

<enumeration value="Failure"/>

<enumeration value="nEInformationIntersection"/>

<enumeration value="notAllowedBecauseOfOngoingSwmActivity"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--deleteScManagementProfile Request -->

<element name="deleteScManagementProfileRequest">

<complexType>

<sequence>

<element name="id" type="ScIRPData:Id"/>

</sequence>

</complexType>

</element>

<!--deleteScManagementProfile Response -->

<element name="deleteScManagementProfileResponse">

<complexType>

<sequence>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="success"/>

<enumeration value="failure"/>

<enumeration value="stepNameNotMatch"/>

<enumeration value="nEInformationIntersection"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--listScProcesses Request -->

<element name="listScProcessesRequest">

<complexType>

<sequence>

<element name="nEIdentification" type="ScIRPData:NEInformation" minOccurs="0"/>

</sequence>

</complexType>

</element>

<!--listScProcesses Response -->

<element name="listScProcessesResponse">

<complexType>

<sequence>

<element name="ScProcessList" type="ScIRPData:ScProcessList"/>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="success"/>

<enumeration value="failure"/>

<enumeration value="stepNameNotMatch"/>

<enumeration value="nEInformationIntersection"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--resumeScProcess Request -->

<element name="resumeScProcessRequest">

<complexType>

<sequence>

<element name="id" type="ScIRPData:Id"/>

<element name="startStepName" type="ScIRPData:startStepName"/>

</sequence>

</complexType>

</element>

<!--resumeScProcess Response -->

<element name="resumeScProcessResponse">

<complexType>

<sequence>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="success"/>

<enumeration value="failure"/>

<enumeration value="stepNameNotMatch"/>

<enumeration value="nEInformationIntersection"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--resumeScProcessWithArcfData Request -->

<element name="resumeScProcessWithArcfDataRequest">

<complexType>

<sequence>

<element name="valuesOfNeededRadioParameter" type="ScIRPData:valuesOfNeededRadioParameter"/>

<element name="fileLocation" type="ScIRPData:FileLocation"/>

</sequence>

</complexType>

</element>

<!--resumeScProcessWithArcfData Response -->

<element name="resumeScProcessWithArcfDataResponse">

<complexType>

<sequence>

<element name="validationErrorInfo">

<simpleType>

<restriction base="string">

<enumeration value="ParameterNotSupported"/>

<enumeration value="InvalidParameter"/>

<enumeration value="ValueNotSupported"/>

<enumeration value="MissingParameterValue"/>

<enumeration value="ConflictingParamterValue"/>

<enumeration value="SemanticsError"/>

<enumeration value="OtherError "/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--terminateScProcess Request -->

<element name="terminateScProcessRequest">

<complexType>

<sequence>

<element name="id" type="ScIRPData:Id"/>

</sequence>

</complexType>

</element>

<!--terminateScProcess Response -->

<element name="terminateScProcessResponse">

<complexType>

<sequence>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="success"/>

<enumeration value="failure"/>

<enumeration value="stepNameNotMatch"/>

<enumeration value="nEInformationIntersection"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

<!--ChangeScManagementProfile Request -->

<element name="ChangeScManagementProfileRequest">

<complexType>

<sequence>

<element name="id" type="ScIRPData:Id"/>

<element name="nEInformation" type="ScIRPData:NEInformation"/>

<element name="swVersionToBeInstalled" type="xsc:VersionNumber"/>

<element name="stepsAndSelectedStopPointList" type="xsc:StepsAndSelectedStopPointList"/>

<element name="selectedFinalAdministrativeState" type="xsc:FinalAdministrativeStateValue"/>

</sequence>

</complexType>

</element>

<!--ChangeScManagementProfile Response -->

<element name="ChangeScManagementProfileResponse">

<complexType>

<sequence>

<element name="Result">

<simpleType>

<restriction base="string">

<enumeration value="Success"/>

<enumeration value="Failure"/>

<enumeration value="nEInformationIntersection"/>

<enumeration value="notAllowedBecauseOfOngoingSwmActivity"/>

</restriction>

</simpleType>

</element>

<element name="versionNumber" type="xsc:VersionNumber"/>

<element name="conflictingProfileId" minOccurs="0">

<simpleType>

<restriction base="string">

<enumeration value="swMprofileId"/>

<enumeration value="swMprofileAId"/>

<enumeration value="swMprofileBId"/>

<enumeration value="empty"/>

</restriction>

</simpleType>

</element>

</sequence>

</complexType>

</element>

</schema>

</types>

<message name="listScManagementCapabilitiesRequest">

<part name="parameter" element="ScIRPData:listScManagementCapabilitiesRequest"/>

</message>

<message name="listScManagementCapabilitiesResponse">

<part name="parameter" element="ScIRPData:listScManagementCapabilitiesResponse"/>

</message>

<message name="listScManagementCapabilitiesFault">

<part name="parameter" element="ScIRPData:listScManagementCapabilitiesFault"/>

</message>

<message name="listScManagementProfilesRequest">

<part name="parameter" element="ScIRPData:listScManagementProfilesRequest"/>

</message>

<message name="listScManagementProfilesResponse">

<part name="parameter" element="ScIRPData:listScManagementProfilesResponse"/>

</message>

<message name="createScManagementProfileRequest">

<part name="parameter" element="ScIRPData:createScManagementProfileRequest"/>

</message>

<message name="createScManagementProfileResponse">

<part name="parameter" element="ScIRPData:createScManagementProfileResponse"/>

</message>

<message name="deleteScManagementProfileRequest">

<part name="parameter" element="ScIRPData:deleteScManagementProfileRequest"/>

</message>

<message name="deleteScManagementProfileResponse">

<part name="parameter" element="ScIRPData:deleteScManagementProfileResponse"/>

</message>

<message name="listScProcessesRequest">

<part name="parameter" element="ScIRPData:listScProcessesRequest"/>

</message>

<message name="listScProcessesResponse">

<part name="parameter" element="ScIRPData:listScProcessesResponse"/>

</message>

<message name="resumeScProcessRequest">

<part name="parameter" element="ScIRPData:resumeScProcessRequest"/>

</message>

<message name="resumeScProcessResponse">

<part name="parameter" element="ScIRPData:resumeScProcessResponse"/>

</message>

<message name="terminateScProcessRequest">

<part name="parameter" element="ScIRPData:terminateScProcessRequest"/>

</message>

<message name="terminateScProcessResponse">

<part name="parameter" element="ScIRPData:terminateScProcessResponse"/>

</message>

<message name="resumeScProcessWithArcfDataRequest">

<part name="parameter" element="ScIRPData:resumeScProcessWithArcfDataRequest"/>

</message>

<message name="resumeScProcessWithArcfDataResponse">

<part name="parameter" element="ScIRPData:resumeScProcessWithArcfDataResponse"/>

</message>

<message name="ChangeScManagementProfileRequest">

<part name="parameter" element="ScIRPData:ChangeScManagementProfileRequest"/>

</message>

<message name="ChangeScManagementProfileResponse">

<part name="parameter" element="ScIRPData:ChangeScManagementProfileResponse"/>

</message>

<portType name="ScIRPOperation1">

<operation name="listScManagementCapabilities">

<input message="ScIRPSystem:listScManagementCapabilitiesRequest"/>

<output message="ScIRPSystem:listScManagementCapabilitiesResponse"/>

<fault name="listScManagementCapabilitiesFault" message="ScIRPSystem:listScManagementCapabilitiesFault"/>

</operation>

<operation name="listScManagementProfiles">

<input message="ScIRPSystem:listScManagementProfilesRequest"/>

<output message="ScIRPSystem:listScManagementProfilesResponse"/>

</operation>

<operation name="createScManagementProfile">

<input message="ScIRPSystem:createScManagementProfileRequest"/>

<output message="ScIRPSystem:createScManagementProfileResponse"/>

</operation>

<operation name="deleteScManagementProfile">

<input message="ScIRPSystem:deleteScManagementProfileRequest"/>

<output message="ScIRPSystem:deleteScManagementProfileResponse"/>

</operation>

<operation name="listScProcesses">

<input message="ScIRPSystem:listScProcessesRequest"/>

<output message="ScIRPSystem:listScProcessesResponse"/>

</operation>

<operation name="resumeScProcess">

<input message="ScIRPSystem:resumeScProcessRequest"/>

<output message="ScIRPSystem:resumeScProcessResponse"/>

</operation>

<operation name="resumeScProcessWithArcfData">

<input message="ScIRPSystem:resumeScProcessWithArcfDataRequest"/>

<output message="ScIRPSystem:resumeScProcessWithArcfDataResponse"/>

</operation>

<operation name="terminateScProcess">

<input message="ScIRPSystem:terminateScProcessRequest"/>

<output message="ScIRPSystem:terminateScProcessResponse"/>

</operation>

</portType>

<portType name="ScIRPOperation2">

<operation name="ChangeScManagementProfile">

<input message="ScIRPSystem:ChangeScManagementProfileRequest"/>

<output message="ScIRPSystem:ChangeScManagementProfileResponse"/>

</operation>

</portType>

<binding name="ScIRPOperation1" type="ScIRPSystem:ScIRPOperation1">

<soap:binding style="document" transport="http://schemas.xmlsoap.org/soap/http"/>

<operation name="listScManagementCapabilities">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#listScManagementCapabilities" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="listScManagementCapabilitiesFault"/>

</operation>

<operation name="listScManagementProfiles">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#listScManagementProfiles" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

<operation name="createScManagementProfile">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#createScManagementProfile" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

<operation name="deleteScManagementProfile">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#deleteScManagementProfile" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

<operation name="listScProcesses">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#listScProcesses" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

<operation name="resumeScProcess">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#resumeScProcess" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

<operation name="resumeScProcessWithArcfData">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#resumeScProcessWithArcfData" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

<operation name="terminateScProcess">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#terminateScProcess" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

</binding>

<binding name="ScIRPOperation2" type="ScIRPSystem:ScIRPOperation2">

<soap:binding style="document" transport="http://schemas.xmlsoap.org/soap/http"/>

<operation name="ChangeScManagementProfile">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#ChangeScManagementProfile" style="document"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

</operation>

</binding>

<service name="ScIRPService">

<port name="ScIRPOperation1" binding="ScIRPSystem:ScIRPOperation1">

<soap:address location="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#ScIRP"/>

</port>

<port name="GenericIRPPort" binding="genericIRPSystem:GenericIRPBinding">

<soap:address location="http://www.3gpp.org/ftp/Specs/archive/32_series/32.317#GenericIRP"/>

</port>

<port name="NotificationIRPNtfPort" binding="ntfIRPNtfSystem:NotificationIRPNtf">

<soap:address location="http://www.3gpp.org/ftp/Specs/archive/32_series/32.307#NotificationIRPNtf"/>

</port>

<port name="ScIRPOperation2" binding="ScIRPSystem:ScIRPOperation2">

<soap:address location="http://www.3gpp.org/ftp/Specs/archive/32_series/32.507#ScIRP"/>

</port>

</service>

</definitions>

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | R | Subject/Comment | Cat | Old | New |
| Dec 2009 | SA#46 | SP-090856 | -- | -- | Presentation to SA for information | -- | --- | 1.0.0 |
| Mar 2010 | SA#47 | SP-100051 | -- | -- | Presentation to SA for approval | -- | 1.0.0 | 2.0.0 |
| Mar 2010 | -- | -- | -- | -- | Publication of SA approved version | -- | 2.0.0 | 9.0.0 |
| Jun 2010 | SA#48 | SP-100261 | 001 | - | Alignment of SOAP SS with TS 32.507 for ARCF | B | 9.0.0 | 9.1.0 |